SA WG2 Meeting #114	S2-161844

11 - 15 April 2016, Sophia Antipolis	(revision of S2-161605)

Source:	Samsung

Title:	Key Issue 1 solution: Provisioning of sponsored data connectivity information

Document for:	Approval

Agenda Item:	6.12

Work Item / Release:	SDCI / 14

Abstract of the contribution:

This contribution provides an alternative solution for key issue 1 dealing with provisioning of sponsored data connectivity information.

# Discussion

This contribution provides an alternative solution for key issue 1 which is related to provisioning of sponsored data connectivity information. It makes use of Service Capability Exposure Function (SCEF) which behaves as a mediator between 3GPP network and the sponsored data provider. Sponsored data provider is also referred as Application Function (AF) in this contribution.

Sponsored data connectivity information which is referred as sponsored context in this contribution, is provisioned within a new functional entity named Sponsored Flow Description Database (SFD-DB) which optionally interacts with existing SPR. Interface Spt’ between the SFD -DB and the SCEF is used for the sponsored data management.

Note that sponsored data context can also be part of SPR which eliminates need of having SFD -DB. Although, having SFD-DB will provide separate sponsored context management and hence will be a cleaner management solution rather than merging the sponsored context with existing subscriber profile definition in the SPR.

# Proposal

***** Start of Change # 1 *****

## 6.x	Solution x: Use of separate database and SCEF  - Key Issue 1

### 6.x.1	Solution Description

#### 6.x.1.1	General description of the solution

This solution addresses key issue 1 – provisioning of the packet flow description.

A new functional entity referred as Sponsored Flow Description Database (SFD-DB) is an entity that stores sponsored data connectivity related information received from the Application Function (AF).

Figure 1 – Enhancements to existing reference model

NOTE: This contribution does not preclude SFD-DB being part of SPR/UDR. In that case, Solution#2 defined in TR 23.721 for key issue 1 is applicable.

Sponsored data connectivity information includes information such as sponsored data provider identity, charging key and a list of packet flow descriptions.

#### 6.x.1.2	Procedures

The following figure represents the flow diagram detailing the procedures involved in provisioning of packet flow description outside the IP-CAN session context.

Figure 2 – Provisioning of sponsored data connectivity information

Step 1: When the sponsored data provider has new packet flow information, it sends a Provision Service Data Flow (SDF) Request to the SCEF. This includes information such as sponsored data provider’s identity, list of packet flow description(s).

Step 2: Upon receipt of the Provision SDF Request, the SCEF verifies authenticity of the AF and also performs parameters mapping if necessary.

Step 3. The SCEF then forwards the Provision SDF Request to the SPR.

Step 4. The SFD-DB

optionally communicates with the SPR to add a notification flag for the presence of the sponsored data availability in the SFD-DB.

Step 5: Once the SFD-DB updates the subscriber record to include sponsored data information, the response is returned to the PCRF.

Step 6: The PCRF forwards the response to the SCEF.

Step 7: The AF is then notified on the completion of the request.

This procedure is repeated when it is essential to modify or delete existing sponsored data connectivity information stored in the SFD-DB.

Once provisioning of the service data flow is completed, the SFD-DB is accessed to get that information.

### 6.x.2 Impacts on existing nodes and functionality

The following is the list of potential impact which includes defining a new functional entity SFD-DB and updates needed for the existing network functions:

- The SCEF shall be enhanced to receive sponsored data related parameters from the sponsoring data providers.

- New database SFD-DB may be included to store data pertaining to sponsored data subscription.

  - Interface between the SFD-DB and SCEF for managing sponsored data.

- There may be minor impact to existing SPR if it is updated to include indication of sponsored data subscription for a subscriber, a group of subscriber or all subscribers.

***** End of Change # 1 *****