SA WG2 Meeting #116	S2-163898

11 - 15 July 2016, Vienna, Austria	(revision of S2-163674)

Source:	Deutsche Telekom AG

Title:	Update of solution 4 for eCall

Document for:	Approval

Agenda Item:	6.11

Work Item / Release:	FS_SeDoc / Release 14

Abstract of the contribution: This document proposes an update of solution 4 to support eCall

# Discussion

This document proposes an update of solution 4 to include eCall .

# Proposal

It is proposed to change following clause in TR 23.719

Start of change

## 6.4	Solution 4: Emergency Calls Routed via IMS Service Domain

### 6.4.1	Overview

This solution addresses key issue #2.

The ICS MSC-Server and the SeDoC MSC Server shall be able to detect the Emergency Call setup based on the CS Setup Indication. The ICS MSC Server already is able to transfer CS calls into IMS sessions, but needs to be able to support some P-CSCF capabilities in case of emergency call.

-	Detection of non-UE detectable emergency calls

-	Map the eCall specific CS signalling information to IMS eCall information

-	Allow IMS Emergency calls also when the UE is not registered with the IMS or does not have sufficient credentials

-	In the case that UE is not already IMS registered, it shall perform a registration for the support of emergency services (emergency registration).

-	Forwarding of the emergency call request to E CSCF in the same network

-	Since the MSC-Server is the mobility management entity of the CS UE, it can provide location information and can include it already in the request to the E-CSCF.

The following figure shows the basic architecture to support emergency sessions in the VPLMN for the inbound roaming UE.

Figure 6.4.1-1: Basic architecture for Emergency Call support

As a summary the ICS MSC-Server main impact is to support detection of the CS Emergency Setup, the procedures of the P-CSCF in case of emergency according to TS 23.167 [15] and a new interface towards the E-CSCF.

Editor's note:	It is FFS whether the ISC MSC-Server performs the IMS Emergency Registration.

End of change