SA WG2 Meeting #119	S2-171008

16-20 January 2017, Dubrovnik, Croatia	(was S2-17xxxx)

Source:	Nokia

Title:	TS 23.501: P-CR to update overall architecture

Document for:	Discussion/Approval

Agenda Item:	6.5.1

Work Item / Release:	5G_ph1/Rel-15

Abstract of the contribution: Proposes to update overall architecture.

# Rationale for changes

- It is proposed to update the arch. principle in 4.1 to clarify that the NF should be orthogonal entities to avoid duplication of functionality and ensure modularity. Orthogonality is essential for extensibility system.

- In the TR, following agreements were made:

“To support "stateless" NFs (where the "compute" resource is decoupled from the "storage" resource that stores state as opaque data), 3GPP will specify (possibly by referencing) interfaces from NFs to a data storage function. NFs may use data storage function to store opaque data.

Exposure of information as a service is supported as follows:

-	Network functions may expose structured information (e.g. UE related information) to other network functions as a capability. Exposure of information can trigger actions in the consuming NF.

NOTE 6:	What information is to be exposed will be determined during the normative phase.

-	A network function may use information exposed by other network functions.

-	A network function may expose information that it received from another NF.

-	The Network Exposure Function receives information from other network functions (based on exposed capabilities of other network functions). It may store the received information as structured data using a standardized interface to a data storage network function (interface to be defined by 3GPP). The stored information can be "re-exposed" by the NEF to other network functions and used for other purposes such as analytics.”

According to the agreements, it is understood that the NFs may use DSF for storing opaque data and NEF may store structured data in DSF. Furthermore, DSF is a 3GPP defined network function.

It is up to implementations/deployments to decide whether a single instance of DSF is deployed for both the purpose or different instances of DSFs are deployed for both the purpose.

Just as for other interfaces, it is up to stage 3 (CT4) decision to determine whether it is the same protocol or different protocol to be used for the two purpose. In either case, a DSF network function can support one or two protocols (e.g. e.g. a single NF can support GTP and PMIP or GTP and DIAMETER in EPS).

For both the use cases, DSF should be in the PLMN local to the respective NF. For storing session context, DSF should be local to respective NF that uses DSF for session context. For storing structured data for information exposure, DSF should be local to NEF. Thus for roaming scenarios, it must be present in the VPLMN and HPLMN.

- Fixed roaming SB architecture figures to move AF outside the enclosed box.

- Added a reference point for AMF to EIR. Just as in TS 23.401, reference point number and description has been added for EIR but not added in the figure.

# 2	Proposal

It is proposed to modify TS 23.501 as follows…

* * * * First Change * * * *(update existing text)

# 4	Architecture model and concepts

## 4.1	General concepts

The 5G system architecture is defined to support overall system service of 5G. The system design enables the deployments to use techniques e.g. Network Function Virtualization and Software Defined Networking. The 5G system architecture shall leverage service-based interactions between Control Plane (CP) network functions where identified. Some key principles and concept are the following:

-	Separate User Plane (UP) functions from the Control Plane (CP) function, allowing independent scalability, evolution and a flexible deployment e.g. central location or distributed (remote) location.

-	Modularize the functions design into orthogonal entities to enable flexible and efficient network slicing and overall extensibility of the system.

-	Wherever applicable, procedures (i.e. the set of interactions between network functions) are defined as services, so that their re-use is possible.

-	Each Network function can interact with the other NF directly if required. The architecture does not preclude the use of an intermediate function to help route control plane messages (e.g. like a DRA).

-	Minimizes dependencies between Access Network (AN) and Core Network (CN), the architecture is defined with a converged access-agnostic core network with a common AN - CN interface which integrates different 3GPP and non-3GPP access types.

-	Support a unified authentication framework.

-	Support "stateless" NFs, where the "compute" resource is decoupled from the "storage" resource.

-	Support capability exposure.

-	Concurrent access to local and centralized services is supported. To support low latency services and access to local data networks, UP functions can be deployed close to the radio network.

-	Support for roaming with both Home routed traffic as well as Local breakout traffic in the visited PLMN.

* * * * Next Change * * * *(update existing text)

## 4.2	Architecture reference model

### 4.2.1	General

This specification describes the architecture for the 5G system. The interaction between network functions is represented in two ways.

-	A service-based representation, where network functions (e.g. AMF) within the control plane enables other authorized network functions to access their services. This representation also includes point-to-point reference points where necessary.

-	A reference point representation, focusing on the interactions between pairs of network functions described by point-to-point reference point (e.g. N11) between any two network functions (e.g. AMF and SMF) is depicted when some interaction exists between these two network functions.

Service-based interfaces are listed in clause 4.2.4. Reference points are listed in clause 4.2.5.

Network functions within the 5G Core Control Plane unless explicitly stated otherwise, shall exhibit service-based interfaces for services that can be used by other authorized network functions.

NOTE:	This will be evaluated on a case by case basis when specifying the procedure.

Editor's note:	The above two statements will be revisited when the work on system procedure is stable.

### 4.2.2	Network Functions and entities

The 5G System architecture consists of the following network functions (NF). The functional description of these network functions is specified in clause 6.

-	Authentication Server Function (AUSF)

-	Core Access and Mobility Management Function (AMF)

-	Data network (DN), e.g. operator services, Internet access or 3rd party services

-	Data Storage network function (DSF)

Editor's note:	Different data storage functions can be used for storing structured data from NEF and storing opaque data from NF. How this is captured is FFS.

-	Network Exposure Function (NEF)

-	NF Repository Function (NRF)

-	Policy Control function (PCF)

-	Session Management Function (SMF)

-	Unified Data Management (UDM)

-	User plane Function (UPF)

-	User Equipment (UE)

-	(Radio) Access Network ((R)AN)

### 4.2.3	Non-roaming reference architecture

Figure 4.2.3-1 depicts the non-roaming reference architecture with service-based interfaces within the Control Plane.

Figure 4.2.3-1: 5G System Service-based architecture

NOTE 1:	In actual deployments, different data storage functions (DSF) instances can be deployed for storing structured data from NEF and storing opaque data from NF. Each NF can have its own data storage function (DSF) or multiple NFs can share a single DSF for storing opaque data.

Figure 4.2.3-2 depicts the 5G System architecture in the non-roaming case, using the reference point representation showing how various network functions interact with each other.

Figure 4.2.3-2: Non-Roaming 5G System Architecture in reference point representation

NOTE 2:	N9, N14 are not shown in all other figures however they may also be applicable for other scenarios.

NOTE 3:	For the sake of clarity of the point-to-point diagrams, the DSF, NEF and NRF have not been depicted. However, all depicted Network Functions can interact with the DSF, NEF and NRF as necessary.

Figure 4.2.3-3 depicts the non-roaming architecture for UEs concurrently accessing two (e.g. local and central) data networks using multiple PDU Sessions, using the reference point representation. This figure shows the architecture for multiple PDU sessions where two SMFs are selected for the two different PDU sessions. However, each SMF may also have the capability to control both a local and a central UPF within a PDU session.

Figure 4.2.3-3: Applying non-roaming 5G System architecture for multiple PDU session in reference point representation

Figure 4.2.3-4 depicts the non-roaming architecture in case concurrent access to two (e.g. local and central) data networks is provided within a single PDU session, using the reference point representation.

Figure 4.2.3-4: Applying non-roaming 5G System architecture for concurrent access to two (e.g. local and central) data networks (single PDU session option) in reference point representation

### 4.2.4	Roaming reference architectures

Figure 4.2.4-1 depicts the 5G System roaming architecture with local breakout with service-based interfaces within the Control Plane.

Figure 4.2.4-1 Roaming 5G System architecture- local breakout scenario with AF in VPLMN in service-based interface representation

Figure 4.2.4-2 depicts the 5G System roaming architecture with local breakout with AF in HPLMN in service-based interfaces within the Control Plane.

Figure 4.2.4-2 Roaming 5G System architecture - local breakout scenario with AF in HPLMN in service-based interface representation

Figure 4.2.4-2 depicts the 5G System roaming architecture in case of home routed scenario when service-based interfaces within the Control Plane.

Figure 4.2.4-3 Roaming 5G System architecture - home routed scenario in service-based interface representation

Figure 4.2.4-4 depicts 5G System roaming architecture in case of local break out scenario with AF in VPLMN using the reference point representation.

Figure 4.2.4-4: Roaming 5G System architecture - local breakout scenario with AF in VPLMN in reference point representation

Figure 4.2.4-5 depicts 5G System roaming architecture in case of local break out scenario with AF in the HPLMN using the reference point representation.

Figure 4.2.4-5: Roaming 5G System architecture -local breakout scenario with AF in the HPLMN in reference point representation

Following figure 4.2.4-4 depicts the 5G System roaming architecture in case of home routed scenario using the reference point representation.

Figure 4.2.4-5: Roaming 5G System architecture-Home routed scenario in reference point representation

### 4.2.5	Service-based interfaces

The 5G System Architecture contains the following service-based interfaces:

Namf:	Service-based interface exhibited by AMF.

Nsmf:	Service-based interface exhibited by SMF.

Nnef:	Service-based interface exhibited by NEF.

Npcf:	Service-based interface exhibited by PCF.

Nudm:	Service-based interface exhibited by UDM.

Naf:	Service-based interface exhibited by AF.

Nnrf:	Service-based interface exhibited by NRF.

Nausf:	Service-based interface exhibited by AUSF.

Ndsf:	Service-based interface exhibited by DSF.

### 4.2.6	Reference points

The 5G System Architecture contains the following reference points:

N1:	Reference point between the UE and the Access and Mobility Management function.

N2:	Reference point between the (R)AN and the Access and Mobility Management function.

N3:	Reference point between the (R)AN and the User plane function (UPF).

N4:	Reference point between the Session Management function (SMF) and the User plane function (UPF).

N5:	Reference point between the Policy Function (PCF) and an Application Function (AF).

N6:	Reference point between the UP function (UPF) and a Data Network (DN).

NOTE:	The traffic forwarding details of N6 between a UPF acting as an uplink classifier and a local data network will not be specified in this release.

N7:	Reference point between the Session Management function (SMF) and the Policy Control function (PCF).

N7r:	Reference point between the vPCF and the hPCF.

N8:	Reference point between Unified Data Management and AMF.

N9:	Reference point between two Core User plane functions (UPFs).

N10:	Reference point between UDM and SMF.

N11:	Reference point between Access and Mobility Management function (AMF) and Session Management function (SMF).

N12:	Reference point between Access and Mobility Management function (AMF) and Authentication Server function (AUSF).

N13:	Reference point between UDM and Authentication Server function (AUSF).

N14:	Reference point between two Access and Mobility Management functions (AMFs).

N15:	Reference point between the PCF and the AMF in case of non-roaming scenario, V-PCF and AMF in case of roaming scenario.

N16:	Reference point between two SMFs, (in roaming case between V-SMF and the H-SMF).

N17:	Reference point between AMF and EIR.

* * * * Next Change * * * *(update existing text)

# 6	Network Functions

Editor's note:	This should include Network functions, functionalities and NF selection functionality, etc.

## 6.1	General

## 6.2	Network Function Functional description

Editor's note:	This should include various network functions in the architecture, features and functionalities supported.

## 6.2.1	AMF

The Access and Mobility Management function (AMF) includes the following functionality. Some or all of the AMF functions may be supported in a single instance of a AMF:

-	Termination of RAN CP interface (N2).

-	Termination of NAS (N1), NAS ciphering and integrity protection.

-	Registration management.

-	Connection management.

-	Reachability management.

-	Mobility Management.

-	Lawful intercept (for AMF events and interface to LI System).

-	Transparent proxy for routing SM messages.

-	Access Authentication.

-	Access Authorization.

-	Security Anchor Function (SEA). It interacts with the AUSF and the UE, receives the intermediate key that was established as a result of the UE authentication process. In case of USIM based authentication, the AMF retrieves the security material from the AUSF.

-	Security Context Management (SCM). The SCM receives a key from the SEA that it uses to derive access-network specific keys.

NOTE:	Regardless of the number of Network functions, there is only one NAS interface instance per access network between the UE and the CN, terminated at one of the Network functions that implements at least NAS security and mobility management.

NOTE:	Not all of the functions are required to be supported in an instance of a network slice.

Editor's note:	When there is an update to security architecture defined by SA WG3, security functionality mapping to the overall architecture will be updated.

## 6.2.2	SMF

The Session Management function (SMF) includes the following functionality. Some or all of the SMF functions may be supported in a single instance of a SMF:

-	Session Management e.g. Session establishment, modify and release, including tunnel maintain between UPF and AN node.

-	UE IP address allocation & management (incl optional Authorization).

-	Selection and control of UP function.

-	Configures traffic steering at UPF to route traffic to proper destination.

-	Termination of interfaces towards Policy control functions.

-	Control part of policy enforcement and QoS.

-	Lawful intercept (for SM events and interface to LI System).

-	Termination of SM parts of NAS messages.

-	Downlink Data Notification.

-	Initiator of AN specific SM information, sent via AMF over N2 to AN.

-	Determine SSC mode of a session (for IP type PDU session)

-	Roaming functionality:

-	Handle local enforcement to apply QoS SLAs (VPLMN).

-	Charging data collection and charging interface (VPLMN).

-	Lawful intercept (in VPLMN for SM events and interface to LI System).

-	Support for interaction with external DN for transport of signaling for PDU session authorization/authentication by external DN.

NOTE:	Not all of the functions are required to be supported in an instance of a network slice.

Editor's note:	when there is an update to security architecture defined by SA WG3, security functionality mapping to the overall architecture will be updated.

## 6.2.3	UPF

The User plane function (UPF) includes the following functionality. Some or all of the UPF functions may be supported in a single instance of a UPF:

-	Anchor point for Intra-/Inter-RAT mobility (when applicable).

-	External PDU session point of interconnect to Data Network.

-	Packet routing & forwarding.

-	Packet inspection and User plane part of Policy rule enforcement.

-	Lawful intercept (UP collection).

-	Traffic usage reporting.

-	Uplink classifier to support routing traffic flows to a data network.

-	Branching point to support multi-homed PDU session.

-	QoS handling for user plane, e.g. packet filtering, gating, UL/DL rate enforcement

-	Uplink Traffic verification (SDF to QoS flow mapping).

-	Transport level packet marking in the uplink and downlink.

-	Downlink packet buffering and downlink data notification triggering.

NOTE:	Not all of the UPF functions are required to be supported in an instance of user plane function of a network slice.

## 6.2.4	PCF

The Policy function (PCF) includes the following functionality:

-	Supports unified policy framework to govern network behaviour.

-	Provides policy rules to control plane function(s) to enforce them.

NOTE:	The Policy control function (PCF) is assumed to retrieve subscription information from a subscription repository (UDM).

## 6.2.5	NEF

The Network Exposure Function (NEF) supports the following functionality:

-	It provides a means to securely expose the services and capabilities provided by 3GPP network functions e.g 3rd party, internal exposure/re-exposure.

-	The Network Exposure Function receives information from other network functions (based on exposed capabilities of other network functions). It may store the received information as structured data using a standardized interface to a data storage network function (interface to be defined by 3GPP). The stored information can be "re-exposed" by the NEF to other network functions and used for other purposes such as analytics.

Editor's note:	Bullet above needs to be reflected in the figure.

## 6.2.6	NRF

The NF Repository Function (NRF) supports the following functionality:

-	Supports service discovery function. Receive NF Discovery Request from NF instance, and provides the information of the discovered NF instances (be discovered) to the NF instance.

NOTE:	Whether NRF is an enhancement of DNS server is to be determined during Stage 3.

Editor's note:	Whether NRF is used only for NF instance discovery or both NF discovery and selection is FFS.

Editor's note:	Whether Network repository function (NRF) is an enhancement of DNS server will be determined by CT WGs.

## 6.2.7	UDM

The Unified Data Management (UDM) supports the following functionality:

-	Supports Authentication Credential Repository and Processing Function (ARPF). This function stores the long-term security credentials used in authentication for AKA.

-	Stores Subscription information.

NOTE:	UDR (User data repository) could be present within the UDM.

## 6.2.8	AUSF

The AUSF supports the following functionality:

-	Supports Authentication Server Function (AUSF) as specified by SA WG3.

Editor's note:	when there is an update to security architecture defined by SA WG3, security functionality mapping to the overall architecture will be updated.

## 6.2.9	DSF Functional description

The DSF supports the following functionality. Some or all of the DSF functions may be supported in a single instance of a DSF:

-	Storage of session context as opaque data.

-	Storage of information as structured data.

NOTE 1:	Not all of the functions are required to be supported in a single instance of a DSF.

NOTE 2:	In actual deployments, different data storage functions (DSF) instances can be deployed for storing structured data from NEF and storing opaque data from NF. Each NF can have its own data storage function (DSF) or multiple NFs can share a single DSF for storing opaque data.

* * * * End Change * * *