SA WG2 Meeting #120	S2-172414

27 – 31 March 2017, Busan, Korea

Source:	ITRI

Title:	Solution for authorization of eRemote-UE

Document for:	Approval

Agenda Item:	6.10

Work Item / Release:	FS_REAR

Abstract of the contribution: This contribution proposes a solution for Key Issue #1 on authorization of eRemote-UE.

1.	Introduction

This contribution propose a solution for authorization of an eRemote-UE to access the network through a eRemote-UE during attach procedure.

2.	Discussion

In current TR 36.746 [2] the architecture for control plane radio protocol stack (PC5) is as follow:

Uu PDCP and RRC are terminated between the evolved ProSe Remote UE and the eNB. In legacy LTE, before the attach procedure, UE need to go into RRC_CONNECTED. This involve some signalling messages between UE and eNB, e.g. RRC Connection Request and RRC Connection Setup. If this or similar principle is to be followed, eRemote-UE needs to get permitted by the eRelay-UE and eRelay-UE needs to set up the layer 2 relaying for eRemote-UE and eNB, i.e., a layer 2 link between eRemote-UE and eRelay-UE, which will be mapped to the radio bearers in the Uu link between eRelay-UE and eNB

.

It is desirable for the eRelay-UE to determine the acceptance of relaying for control plane and data plane separately. For example, the user plane data relaying may not be setup until Attach Complete is sent from eRemote-UE to eRemote MME.

Proposal 1: Separate the requests for user plane relaying and control plane relaying.

Proposal 2: The acceptance for control plane relaying are determined by eRelay-UE based on local decision, pre-configured information, information from the network or any combination of above.

Reference

[2] 3GPP TR 36.746 “Study on further enhancements to LTE Device to Device (D2D), User Equipment (UE) to network relays for Internet of Things (IoT) and wearables”

3.	Proposal

It is proposed to add the following text to clause 6.1.3 in TR 23.733.

* * * * BEGINNING OF CHANGES * * * *

6.1. X	Solution 4: eRemote-UE attach procedure via eRelay-UE

6.1.X.1	Description

6.1.X.1.1	General

This solution address partial of key issue 1 on how the network authorize the eRemote-UE through an eRelay-UE in the eRemote-UE’s attach procedure.

6.1.X.1.2	eRemote-UE Attach procedure via eRelay-UE

In this authorization approach, request for control plane relaying and user plane relaying are separated. The acceptance for control plane relaying are determined by eRelay-UE based on local decision, pre-configured information, information from the network or any combination of above.

Figure 6.1.X.1.2-1: eRemote-UE attach procedure

0.	Configuration information is provided for eRemote-UE and eRelay-UE.

1.	eRelay-UE attaches to the network. If a UE is authorized as an eRelay-UE by MME, the authorization information is sent to eNB by MME.

2.	eRemote-UE performs Relay discovery procedure.

3.	eRemote-UE sends Relay Request message to eRelay-UE. eRemote-UE identifier is included in the message. eRelay-UE determine whether to setup the control plane layer-2 relaying between the eRemote-UE and the eNB based on local decision, pre-configured information, information from the network or any combination of above.

4. 	A layer-2 link between eRemote-UE and eRelay-UE is set up and is relayed to/from the Uu link between the eRelay-UE and eNB.

Editor's note:	It is up to RAN to define this is step.

5.	The eRelay-UE sends Relay Response message the eRemote-UE. The eRelay-UE identifier is included in the message.

6.	RRC connection is setup between eRemote-UE and eRelay-UE. The RRC messages are layer2-relayed via the eRelay-UE.

7.	The eRemote-UE initiates Attach procedure. The eRelay-UE identifier is included in the Attach Request message. The Attach Request message is included in the RRC message.

NOTE 1:	Step 6 may overlaps with step 5, i.e. the Attach Request is embedded in the RRC Connection Setup Complete message in step 5.

8.	The eNB includes the Attach Request in the Initial UE message and sends it to the eRemote-UE MME.

9.	The remaining steps of Attach procedure following the TS 23.401 are performed.

Editor's note:	Difference between the TS 23.401 is FFS.

6.1.x.2	Impact analysis

6.1.2.3	Evaluation

* * * * END OF CHANGES * * * *