SA WG2 Meeting #121	S2-173305

May 15 – 19, 2017, Hangzhou, China	(revision of S2-17xxxx)

Source:	Huawei, HiSilicon

Title:	TS 23.501: Storing PFDs in SDSF

Document for:	Approval

Agenda Item:	6.5.1

Work Item / Release:	5G_ph1/Rel-15

Abstract of the contribution: This contribution proposes to converge PFDF in SDSF and store the PFDs in SDSF.

# Discussion

The SCEF is the key entity within the 3GPP architecture for service capability exposure that provides a means to securely expose the services and capabilities provided by 3GPP network interfaces. In the EPC, the support of PFD management via SCEF is described in TS 23.682:

- PFD (Packet Flow Description) is a set of information enabling the detection of application traffic.

- The PFDs may be managed by the 3rd party SCS/AS via the SCEF, which ensures the secure access to the operator's network even from the 3rd party SCS/AS in untrusted domain. The 3rd party SCS/AS may request to create, update or remove PFDs in the PFDF via the SCEF.

According to the functionalities of Packet Flow Description Function (PFDF) described in TS 23.401:

A Packet Flow Description Function (PFDF) is an element which stores Packet Flow Descriptions (PFDs) associated with an application identifier and transfers them to the PCEF/TDF via Gw/Gwn interface to enable the PCEF/TDF to perform application detection when the PFDs are managed by a 3rd party SP.

It supports:

- Termination of the Nu reference point where SCEF connects to the PFDF

- Management of Packet Flow Description (PFDs) (i.e. provision, modification and removal) according to the instructions received from the SCS/AS via the SCEF.

- Provision of Packet Flow Description (PFDs) to the PCEF/TDF.

The specific procedure for PFD management via SCEF is described in TS23.682.

Similarly, in 5G system, the PFDs managed by the 3rd party SCS/AS via NEF may be stored in the following options:

Solution 1: The PFDs are stored in SDSF and managed by the 3rd party AS via the NEF.

In solution 1, the NEF locates in 5G core network as a capability enabler, which takes charge of network capability exposure. The SDSF is used to store structured data intended for network external and network internal exposure by the NEF. The PFDs stored in the SDSF can be exposed to the network function by the NEF as structured data. Management of Packet Flow Descriptions in the SDSF refers to the capability to create, update or remove PFDs in the SDSF via NEF and the distribution from the SDSF to the SMF (i.e. PCEF in EPC).

Solution 2: The PFDs are supported by the UDM.

In solution 2, the PFDs can be stored in the UDM by the 3rd party AS via the NEF and retrieved directly from UDM by SMF. According to TS 23.501, the Unified Data Management (UDM) includes the application front end (FE) and the User Data Repository (UDR). The data stored in UDM is UE level rather than application level based on functionality definition of UDM. In addition, the SMF accesses to centralized UDM may result in heavy burden of signalling between SMF and UDM because of the volume of PFDs. Compared with solution 1, the NEFs are in distributed deployment which can throttle the distribution of updates to PFDs.

Observation: The PFDF is a storage function to store PFDs. The PFDs stored in the PFDF are managed by the 3rd party SCS/AS via the SCEF (i.e. NEF in NG). The functionalities of PFDF to store and manage the PFDs can be supported by SDSF.

Proposal: The PFDs are stored in SDSF and managed by the 3rd party AS via the NEF. The SMF retrieves the PFDs from SDSF.

# Proposal

It is proposed to update TS 23.501 as follows:

* * * Begin of 1st Changes * * *

# 2	References

…

[19]	3GPP TS 23.682: "3rd Generation Partnership Project; Technical Specification Group Services and Systems Aspects; Architecture enhancements to facilitate communications with packet data networks and applications".

* * * End of 1st Changes * * *

* * * Begin of 2nd Changes * * *

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

……

PFD	Packet Flow Description

……

* * * End of 2nd Changes * * *

* * * Begin of 3rd Changes * * *

# A.1	High level architectural requirements

The policy framework shall provide the relevant parts of the PCC framework as specified in TS 23.203 [4], including:

a.	Policy Control Function (PCF) shall support interfaces to the Policy and Charging Enforcement Function (PCEF), Network Exposure Function (NEF), the Application Function (AF), and the Online Charging System (OCS).

b.	The PCF shall be able to evaluate operator policies that are triggered by events received from the PCEF, NEF, the AF, and the OCS.

c.	The PCF shall provide Rules for application and service data flow detection, gating, QoS and flow based charging to the PCEF.

d.	The Policy Framework shall be able to manage the Packet Flow Descriptions (PFDs) in the PCEF by the 3rd party AS via the NEF.

e.	The Policy Framework shall support to negotiate the background data transfer policy with the 3rd party AS via the NEF.

f.	The PCF shall implement a Front End to access subscription information relevant for policy decisions in a User Data Repository (UDR) including dynamic profile updates pushed by the UDR.

g.	Traffic Steering Control for steering traffic for the services on the DN side of the N6 reference point,

h.	The PCF shall be able to take input from Network Data Analytics (NWDA) into consideration for policies on assignment of network resources and for traffic steering policies.

* * * End of 3rd Changes * * *

* * * Begin of 4th Changes * * *

# A.3	Functional description

## A.3.1	Overall description

The Policy framework architecture provides the functions for:

application and service data flow detection,

QoS and gating control,

Credit management,

Flow based charging,

Background data transfer policy negotiation,

Management of the PFDs in the PCEF via NEF by the 3rd party AS,

Traffic Steering Control for steering traffic for the services on the DN side of the N6 reference point,

Provide a Front End to subscription information relevant for policy decisions in a User Data Repository,

Provide Network selection and Mobility Management related policies (e.g. RFSP index) to the Access and Mobility Management,

Editor's note:	Further work is required to include the different aspects of the Policy framework architecture.

The PCF evaluates operator policies that are triggered by events received from the Application Function, the Session Management, Mobility Management and the Online Charging System as well as Changes in User Profile.

For policy control, the AF interacts with the PCF and the PCF interacts with the PCEF as instructed by the AF as specified in TS 23.203 [4].

Editor's note:	Further work would be required to cover details of these aspects.

NOTE 1:	Credit management and reporting are defined in SA WG5 specification.

NOTE 2:	The policy control framework for phase 1 does not preclude potential extensions in phase 2 for policy control in multiple administrative areas.

* * * End of 4th Changes * * *

* * * Begin of 5th Changes * * *

### 6.2.11	SDSF

# The SDSF is an optional function that supports the following functionality:

-	Storage and retrieval of information as structured data by the NEF.

The information stored in SDSF includes:

- Packet Flow Description (PFD)

# NOTE 1:	Deployments can choose to collocate SDSF with other NFs (e.g. UDR, UDSF).

NOTE 2: The information included in a PFD is defined in TS 23.682 [19].

* * * End of 5th Changes * * *