SA WG2 Meeting #121	S2-173413

29 May– 2 Jun 2017, Hangzhou, China	(was S2-17xxxx)

Source:	Nokia, Alcatel-Lucent Shanghai Bell, ATT

Title:	TS 23.502: PCF services identification

Document for:	Approval

Agenda Item:	6.5.11

Work Item / Release:	5G_ph1/Rel-15

Abstract of the contribution: Proposes PCF services and service operations

# 1	Background

During the last SA2, following was agreed:

- NF services are derived from the information flows that describe end to end functionality, where applicable (see 3GPP TS 23.502 [3], Annex B drafting rules).

- Complete end to end message flows can be described by a sequence of NF service invocations.

NOTE:	While the use of NF services is not restricted to the identified consumers, no specific effort will be made in this release of the specification, to develop the description of NF services beyond what is necessary to support their use in information flows.

It was also agreed that:

“Each of the NF services offered by a Network Function shall be self-contained, reusable and use management schemes independently of other NF services offered by the same Network Function (e.g. for scaling, healing, etc).”

“NF service: a functionality exposed by a NF through a service based interface and consumed by other authorized NFs.

NF service operation: An elementary unit a NF service is composed of. “

It is proposed that we summarize the NF services in a table with service description, input/output parameters, other relevant information and have a column to determine whether the given call flow is referring to NF service operation or NF service.

# 2	Criteria

We consider the following set of criteria for defining the services:

- each NF service operates on its own set of context(s)

- services operations that operate on a common context data shall be defined within a single service. Any direct access of a context owned by a NF service are only made by the service operations of that NF service.

- One non-service based interface shall be tied to at most one NF service in an NF.

# 3	Considerations for identifying PCF services and service operations

The services exhibited by PCF should mainly include features that are essential for other Network functions would need as described in e2e call flows in section 4.

To start with, the list of functions that UDM performs need to be considered. From 3GPP TS 23.501 section 6.2.4 the following are the list of PCF functions:

The Policy function (PCF) includes the following functionality: - Supports unified policy framework to govern network behaviour. - Provides policy rules to Control Plane function(s) to enforce them. - Implements a Front End to access subscription information relevant for policy decisions in a User Data Repository (UDR).

It is proposed to update TS 23.502 to include the following services for PCF

If this is agreeable, then it is proposed to update the call flows in section 5 accordingly (i.e. remove the ones that are not relevant as per e2e call flows in section 4).

* * * * Start Change * * *.*

### 5.2.5	PCF Services

Editor's note:	The identified NF service(s) in this clause are to be evaluated. 5.2.z.1 "Get UE Policy" service.

Editor's note:	The requester is assumed to be AMF. It need be evaluated whether it can be applied to other NF.

Table 5.2.3-1: NF Services provided by PCF

| NF service | Description | NF service operations | Consumer | Ref. |
| --- | --- | --- | --- | --- |
| Subscriber Policy Retrieval | Retrieve Subscriber Policy; Receive notification of an updated Subscriber Policy event; Subscribe/Unsubscribe from future notification. | Npcf_Policy_Request | AMF, SMF | 4.2.2.2, 4.2.3.2, 4.2.3.3, 4.3.2.2, 4.3.3.2, 4.3.4.2 |
|  |  | Npcf_Updated_Policy_Notification | AMF, SMF |  |
|  |  | Npcf_Policy_Notification_subscribe/unsubscribe | AMF, SMF |  |
| Policy Authorization | Applying Application Function related policy for the subscriber | Npcf_AF_Policy_Install_Request | AF | TBD in section 4. |
| UE Event Notification | Subscription to UE related event notification. | Npcf_UE_event_detection_Request | AF, NEF | TBD in section 4. |
|  |  | Npcf_UE_event_detection_Notification | AF, NEF |  |

Editor's note:	The identified NF service(s) in this clause are to be evaluated.

#### 5.2.5.1	The procedure of “Subscriber Policy Retrieval” Service

| NF service operation | Description | Input parameters | Output parameters |
| --- | --- | --- | --- |
| Policy_Request | Request Subscriber Policy | UE Permanent ID, Requested Policy Container | requested type policy |
| Updated_Policy_ Notification | Notification of an new (or updated) Subscriber Policy | None | Updated policy |
| Policy_Notification_subscribe/unsubscribe | Subscribe or unsubscribe to policy update notification | UE Permanent ID, Requested Policy Container, subscribe/unsubscribe | result of the operation (ack/nack) |

#### 5.2.5.2	The procedure of " UE Event Notification" Service

| NF service operation | Description | Input parameters | Output parameters |
| --- | --- | --- | --- |
| UE_event_detection_Request | Request event detection | SUPI or External ID, Event filter, Reporting options (e.g. periodic reporting, event based reporting) | result of the operation (ack/nack) |
| UE_event_detection_Notification | Notification of an detected event | None | The detected event |

#### 5.2.5.3	The procedure of "Policy Authorization" Service

| NF service operation | Description | Input parameters | Output parameters |
| --- | --- | --- | --- |
| AF_Policy_Install_Request | Install, update, or remove AF’s policy for the subscriber | The UE Identity (e.g. UE IP address, MSISDN), the requested policy container | result of the operation (ack/nack) |

* * * * End Change * * *.*