SA WG2 Meeting #121	S2-173414

29 May– 2 Jun 2017, Hangzhou, China	(was S2-17xxxx)

Source:	Nokia, Alcatel-Lucent Shanghai Bell, ATT

Title:	TS 23.502: NEF services identification

Document for:	Approval

Agenda Item:	6.5.11

Work Item / Release:	5G_ph1/Rel-15

Abstract of the contribution: Proposes NEF services and service operations

# 1	Background

During the last SA2, following was agreed:

- NF services are derived from the information flows that describe end to end functionality, where applicable (see 3GPP TS 23.502 [3], Annex B drafting rules).

- Complete end to end message flows can be described by a sequence of NF service invocations.

NOTE:	While the use of NF services is not restricted to the identified consumers, no specific effort will be made in this release of the specification, to develop the description of NF services beyond what is necessary to support their use in information flows.

It was also agreed that:

“Each of the NF services offered by a Network Function shall be self-contained, reusable and use management schemes independently of other NF services offered by the same Network Function (e.g. for scaling, healing, etc).”

“NF service: a functionality exposed by a NF through a service based interface and consumed by other authorized NFs.

NF service operation: An elementary unit a NF service is composed of. “

It is proposed that we summarize the NF services in a table with service description, input/output parameters, other relevant information and have a column to determine whether the given call flow is referring to NF service operation or NF service.

# 2	Criteria

We consider the following set of criteria for defining the services:

- each NF service operates on its own set of context(s)

- services operations that operate on a common context data shall be defined within a single service. Any direct access of a context owned by a NF service are only made by the service operations of that NF service.

- One non-service based interface shall be tied to at most one NF service in an NF.

# 3	Considerations for identifying NEF services and service operations

The services exhibited by NEF should mainly include features that are essential for other Network functions would need as described in e2e call flows in section 4.

To start with, the list of functions that NEF performs need to be considered. From 3GPP TS 23.501 section 6.2.5 the following are the list of NEF functions:

The Network Exposure Function (NEF) supports the following functionality: - It provides a means to securely expose the services and capabilities provided by 3GPP network functions for e.g 3rd party, internal exposure/re-exposure, Application Functions, Edge Computing as described in section 5.13. - The Network Exposure Function receives information from other network functions (based on exposed capabilities of other network functions). It may store the received information as structured data using a standardized interface to a data storage network function (interface to be defined by 3GPP). The stored information can be "re-exposed" by the NEF to other network functions and Application Functions, and used for other purposes such as analytics.

It is proposed to update TS 23.502 to include the following services for NEF

If this is agreeable, then it is proposed to update the call flows in section 5 accordingly (i.e. remove the ones that are not relevant as per e2e call flows in section 4).

* * * * Start Change * * *.*

### 5.2.6	NEF Services

### Table 5.2.6-1: NF Services provided by NEF

| NF service | Description | NF service operations | Consumer | Ref. |
| --- | --- | --- | --- | --- |
| Device Trigger | Placing/Removing a trigger in the device | Nnef_Device_Trigger_Event_Setting | AF | TBD in section 4. |
| Device monitoring | Enable the AF to subscribe/unsubscribe UE’s event related to UE location, reachability, roaming status, or/and loss of connectivity | Nnef_Device_event_detection_Request | AF | TBD in section 4. |
|  |  | Nnef_Device_event_detection_Notification | AF |  |
| Device Provisioning | Allow the AF to provide QoS and Charging profile to a device or PDU session. | Nnef_AF_Policy_Install_Request | AF | TBD in section 4. |
|  |  | Nnef_AF_Policy_Result_Notification | AF |  |

Editor's note:	The identified NF service(s) in this clause are to be evaluated.

#### 5.2.6.1	The procedure of "Device Trigger" Service

| NF service operation | Description | Input parameters | Output parameters |
| --- | --- | --- | --- |
| Device_Trigger_ Event_Setting | Placing/Removing a trigger in the device | Trigger add/remove: External Identifier or MSISDN, Requester Identifier, validity period, Application Port ID and Trigger Payload | result of the operation (ack/nack) |

* * * * End Change * * *.*