SA WG2 Meeting #122bis	S2-176715

August 21 – 25, 2017, Sophia Antipolis, France	(e-mail revision 2 of S2-176680)

Source:	China Mobile, ZTE, Verizon

Title:	23.502: Service Request procedure using Mobile Termination service

Document for:	Discussion and Approval

Agenda Item:	6.5.2

Work Item / Release:	5G_ph1 / Rel-15

Abstract of the contribution: This contribution updated the network triggered service request procedures by MT services and the AMF_MT service is further clarified.

# Introduction

In TS 23.502 section 5.2.2.4, Namf_ MT_EnableUEReachability service has been introduced but the Network triggered Service Request procedure has not been updated accordingly.

Approach:

When the network has downlink data or signaling to send, the other NF(s) first invokes AMF MT service, to ensure that the UE enter the connection state, and then sends the downlink signaling to the UE through AMF.

Cons: addition of two signaling and some latency This can be alleviated with synchronized way of service invoking.

Pros:

- NFs will know the UE status (IDLE/Connected), therefore, can decide whether to further send downlink signaling (or data in SMSF case). This can avoid AMF to buffer data (in case SMSF send SMS to AMF). This also simplifies the failure handling case, i.e., no need to roll back for AMF.

- As discussed in S2-175253, enable a separate Mobile Termination (MT) service will help decouple the functionality into more granular services and meet for the scalability requirements, in view that the resources consuming for paging is a big part of the current network element (MME).

# Proposal

It is proposed to update 23.502 cause 4 the Network triggered Service Request using the Namf_ MT_EnableUEReachability service.

The way to update 4.2.3.4 “Network Triggered Service Request”

There are three alternatives to leverage MT services:

Option 1) Consumer (e.g., SMF, SMSF) invokes AMF Communication service and Communication service invokes MT service if required. That is: Consumer Communication service MT service (do paging if required)

Option 2) Consumer invokes MT service and MT service further invokes Communication service: Consumer MT service (do paging if required) Communication service

Option 3) Consumer invokes MT service first then invokes Communication service: Step a) Consumer MT service (do paging if required), Step b) Consumer Communication service

The Option 1 and 2 contains the internal service invoke. The option 1) is actually can be considered as current mechanism and the MT service is defined but not exhibits service interface to NF services external of AMF. Therefore,

Proposal 1: it is suggested to adopt Option 3.

To do that, the current Step 3a (the N11 message) is replaced by invoking MT service to make UE reachable.

The N1N2 message in current Step 3a is sent when UE becomes connected (i.e., becomes Step 9).

* * * First Change * * *

The text below are all new. The changes are shown as compared with 4.2.3.4 for reference

#### -

* * * Second Change * * *

### 5.2.2	AMF Services

#### 5.2.2.1	General

The following table shows the AMF Services and AMF Service Operations.

Table 5.2.2.1-1: List of AMF Services

| Service Name | Service Operations | Known Consumer(s) | Operation Semantic | Reference |
| --- | --- | --- | --- | --- |
| Namf_Communication | UEContextTransfer | Peer AMF | Request/ Response | Step 4 and 5 of clause 4.2.2.2.2. |
|  | UERegistrationCompleteNotify | Peer AMF | Subscribe / Notify | Step 10 of clause 4.2.2.2.2. |
|  | N1MessageNotify | SMF, SMSF, PCF, NEF | Subscribe / Notify | Step 3 clause 4.3.2.2.1. Step 5a of clause 4.2.2.2.3. |
|  | N1MessageSubscribe | SMF, SMSF, PCF, NEF | Subscribe / Notify | Step 5a of clause 4.13.3.6. |
|  | N1MessageUnSubscribe | SMF, SMSF, PCF, NEF | Subscribe / Notify | Step 7 of clause 4.13.3.3. Step 6c of clause 4.13.3.6. |
|  | N1N2MessageTransfer | SMF, SMSF, PCF, NEF | Request/ Response | Step 10 of clause 4.3.2.2.1. Step 14 of lause 4.3.2.2.2. Step 5a of clause 4.13.3.6. |
|  | N2InfoSubscribe | SMF | Subscribe / Notify | Step 10 of clause 4.3.2.2.1. Step 14 of clause 4.3.2.2.2. |
|  | N2InfoUnSubscribe | SMF | Subscribe / Notify |  |
|  | N2InfoNotify | SMF | Subscribe / Notify | Step 14 of clause 4.3.2.2.1. Step 18 of clause 4.3.2.2.2. Step 9 of clause 4.2.3.2. |
| Namf_EventExposure | Subscribe | NEF, SMF, PCF, UDM | Subscribe / Notify | Step 10 of clause 4.3.2.2.1. Step 14 of clause 4.3.2.2.2. |
|  | UnSubscribe | NEF, SMF, PCF, UDM | Subscribe / Notify |  |
|  | Notify | NEF, SMF, PCF, UDM | Subscribe / Notify | Step 4a of clause 4.2.3.2. Step 3a of clause 4.2.3.3. |
| Namf_MT | EnableUEReachability | NEF, SMF, PCF, UDM | Request/ Response | EnableUEReachability |

Editor's note:	Whether additional NF services can be identified or be decomposed from existing NF services are FFS.

#### 5.2.2.4	Namf_MT service

##### 5.2.2.4.1	General

Service description: It provides a NF the service that make sure UE is reachable to send MT signaling or data. The following are the key functionalities of this NF service

-   paging UE if UE is in IDLE state and respond other NF after the UE enters CM-CONNECTED state.

-  response to the requester NF if UE is in CONNECTED state.

##### 5.2.2.4.2	Namf_MT_ EnableUEReachability service operation

Service operation name: Namf_MT_EnableUEReachability.

Description: The consumer NF uses this service operation to request enabling UE reachability.

Known NF Consumers: SMSF

Inputs, Required: NF ID, UE ID.

Inputs, Optional: None.

Outputs, Required: None.

Outputs, Optional: None.

See step 3 of clause 4.2.3.4 for details on the usage of this service operation, and 4.2.2.3.3 for UDM initiated de-registration.

The consumer NF does not need to know UE state. The AMF accepts the request and respond the consumer NF immediately if UE is in CM-CONNECTED state. If the UE is in CM-IDLE state, the AMF may page UE and respond to the consumer NF after the UE enters CM-CONNECTED state.

*************** end of change *********************