SA WG2 Meeting #123	S2-177412

23 - 27 October 2017 Ljubljana, Slovenia

Source:	Huawei, HiSilicon

Title:	TS 23.502: Updates to mobility restriction

Document for:	Approval

Agenda Item:	6.5.2

Work Item / Release:	5GS_Ph1/Rel-15

Abstract of the contribution: This contribution proposes to clarify that RAT restriction is per PLMN in TS 23.501.

Discussion

In current TS 23.501, the description of RAT restriction as following is already implied that RAT restriction is per PLMN, but not clearly enough.

- RAT restriction:

Defines the 3GPP Radio Access Technology(ies), a UE is not allowed to access. In a restricted RAT a UE is based on subscription not permitted to initiate any communication for this PLMN.

This pCR proposes to clarify that RAT restriction is per PLMN.

Proposal

It is proposed to add the following changes into TS 23.501.

* * * * Start of changes * * * *

5.3.4.1	Mobility Restrictions

5.3.4.1.1	General

Mobility Restrictions restrict mobility handling or service access of a UE in the 5G System. The Mobility Restriction functionality is provided by the UE, the radio access network and the core network.

Mobility Restrictions only apply to 3GPP access, they do not apply to non-3GPP access.

Mobility Restrictions for CM-IDLE state and, for CM-CONNECTED state when in RRC Inactive state are executed by the UE based on information received from the core network. Mobility Restrictions for CM-CONNECTED state when in RRC-Connected state are executed by the radio access network and the core network.

In CM-CONNECTED state, the core network provides Mobility Restrictions to the radio access network within Handover Restriction List.

Mobility restrictions consists of RAT restriction, Forbidden area, Service area restrictions and Core Network type restriction as follows:

-	RAT restriction:

Defines the 3GPP Radio Access Technology(ies), a UE is not allowed to access in a PLMN. In a restricted RAT a UE is based on subscription not permitted to initiate any communication for this PLMN. For CM-CONNECTED state, when radio access network determines target RAT and target PLMN during handover procedure, it should take per PLMN RAT restriction into consideration.

-	Forbidden area:

In a Forbidden area under a given RAT, the UE is based on subscription not permitted to initiate any communication with the network for this PLMN. The UE behavior in terms of cell selection, RAT selection and PLMN selection depends on the network response that informs the UE of Forbidden area.

NOTE 1:	The UE reactions to specific network responses are described in TS 24.xxx [XX].

-	Service area restrictions:

Defines areas in which the UE may or may not initiate communication with the network as follows:

-	Allowed area:

In an Allowed area under a given RAT, the UE is permitted to initiate communication with the network as allowed by the subscription.

-	Non-allowed area:

In a Non-allowed area under a given RAT a UE is service area restricted based on subscription. The UE and the network are not allowed to initiate Service Request or SM signalling to obtain user services (both in CM-IDLE and in CM-CONNECTED states). The RRC procedures while the UE is in CM-CONNECTED with RRC Inactive state are unchanged compared to when the UE is in an allowed area. The RM procedures are unchanged compared to when the UE is in an allowed area. The UE in a Non-allowed area shall respond to core network paging with Service Request and RAN paging.

-	Core Network type restriction:

Defines whether UE is allowed to connect to 5GC for this PLMN.

NOTE 2:	The Core Network type restriction can be used e.g. in network deployments where the E-UTRAN connects to both EPC and 5GC as described in clause 5.17.

For a given UE, the core network determines the Mobility restrictions based on UE subscription information, UE location and local policy. The Mobility restriction may change due to e.g. UE’s subscription, location change and local policy. Optionally the allowed area may in addition be fine-tuned by the PCF e.g. based on UE location, PEI and network policies. Service area restrictions can be changed due to, e.g. subscription, location, PEI and/or policy change. Service area restrictions may be updated during a Registration procedure.

If the UE has overlapping areas between RAT restrictions, Forbidden areas, Allowed areas and Non-allowed areas, or any combination of them, the UE shall proceed in the following precedence order:

-	The evaluation of RAT restrictions shall take precedence over the evaluation of any other Mobility restrictions;

-	The evaluation of Forbidden areas shall take precedence over the evaluation of allowed areas and non-allowed areas; and

-	The evaluation of Non-allowed area shall take precedence over the evaluation of Allowed areas.

A UE shall override any RAT restrictions, Forbidden area and Non-allowed area restrictions whenever access to the network for regulatory prioritized services like Emergency services and MPS. Also the network shall override any non-allowed area restrictions and RAT restrictions for regulatory prioritized services like Emergency services and MPS.

* * * * End of changes * * * *