SA WG2 Meeting #128-bis	S2-187719

Sophia Antipolis, France, 20 - 24 August 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 0554 | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | AMF requests for the PCF policy update of the URSP rules in the UE after the Allowed and/or Configured NSSAI update |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Wistron |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1 |  |  |  |  |  |  | Date: |  | 2018-08-02 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | According to TS 23.501, clause 5.15.4.2, it describes that, once the UE received the Allowed and/or Configured NSSAI update, it should also receive the URSP update according TS 23.503. However, when referring to TS 23.503, clause 6.6.2.2, there is no mention how and when the PCF is triggered to update the URSP rule in the UE. Furthermore, clause 5.15.4.2 in TS 23.501states that, without the URSP rule, the UE is not allowed to indicate the target S-NSSAI in the PDU Session Establishment procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This is the 23.501 part of the CR correction to have the AMF to request the UE’s serving PCF in the HPLMN for the policy update of the URSP rule in the UE once the Network Slicing subscription change has been successfully updated in the UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE will not able to initiate the PDU Session Establishment until the UE’s serving PCF in the HPLMN updates the URSP in the UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.15.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | The corresponding changes to TS 23.502 and TS 23.503 are in CR0632 and CR0120. |  |  |  |  |  |  |  |  |

#### ***** First Change *****

#### 5.15.4.2	Update of UE Network Slice configuration

At any time, the AMF may provide the UE with a new Configured NSSAI for the Serving PLMN, associated with mapping of the Configured NSSAI to the Configured NSSAI for the HPLMN as specified in clause 5.15.4.1. The Configured NSSAI for the Serving PLMN and the mapping information is either determined in the AMF (if based on configuration, the AMF is allowed to determine the Network Slice configuration for the whole PLMN) or by the NSSF. The AMF provides an updated Configured NSSAI as specified in TS 23.502 [3], clause 4.2.4 UE Configuration Update procedure.

If the HPLMN performs the configuration update of a UE registered in the HPLMN (e.g. due to a change in the Subscribed S-NSSAI(s)), this results in updates to the Configured NSSAI for the HPLMN. Updates to the Allowed NSSAI and/or, if present, to the associated mapping of the Allowed NSSAI to the Configured NSSAI for the HPLMN are also possible if the configuration update affects S-NSSAI(s) in the current Allowed NSSAI.

If the VPLMN performs the configuration update of a UE registered in the VPLMN (e.g. due to a change in the Subscribed S-NSSAI(s), the associated mapping is updated), this results in updates to the Configured NSSAI for the Serving PLMN and/or to the associated mapping of the Configured NSSAI for the Serving PLMN to the Configured NSSAI for the HPLMN. Updates to the Allowed NSSAI and/or to the associated mapping of the Allowed NSSAI to the Configured NSSAI for the HPLMN are also possible if the configuration update affects S-NSSAI(s) in the current Allowed NSSAI.

A UE for which the Configured NSSAI for the Serving PLMN has been updated as described in clause 5.15.4.1 and has been requested to perform a Registration procedure, shall initiate a Registration procedure to receive a new valid Allowed NSSAI (see clause 5.15.5.2.2).

When the subscribed S-NSSAIs change, a UDR flag is set in the HPLMN to make sure the current PLMN (or, if the UE was not reachable, the next serving PLMN) is informed by the UDM that the subscription data for network slicing has changed. The AMF, when it receives the indication from the UDM subscription has changed, indicates the UE that subscription has changed and uses any updated subscription information from the UDM to update the UE. Once the AMF updates the UE and obtains an acknowledgment from the UE, the AMF informs the UDM that the configuration was successful and the UDM clears the flag in the UDR.

If the UE receives indication from the AMF that Network Slicing subscription has changed, the UE locally deletes the network slicing information it has for all PLMNs (i.e. so when it registers in other PLMNs it sends no requested NSSAI and the serving PLMN updates the UE configuration for the Serving PLMN). It also updates the current PLMN network slicing configuration information with any received values from the AMF.  The AMF also requests the UE’s serving PCF of the HPLMN to perform the policy update of the URSP rule in the UE.

The update of URSP rules (which include the NSSP), if necessary at any time, is described in TS 23.503 [45].

#### ***** End of Change *****