3GPP TSG-SA2 Meeting #128bis	S2-188147

# Sophia-Antipolis, France, 20th Aug 2018 - 24th Aug 2018

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 0501 | rev | 2 | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification for pre-configured QoS rule |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1 |  |  |  |  |  | Date: |  |  | 2018-08-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This CR was postponed at last meeting. The comment was we still need the pre-configured QoS rule for the 1a QoS control option of the non-GBR QoS flows without signalling. But if we keep the pre-configured QoS rule, as it contains QFI and Packet Filter Set, the problem is how to make the QFI and Packet Filter Set consistent between UE and network. if there’s no corresponding QoS flow in the network, the pre-configured QoS rule can’t work. One option is to pre-configure the same QoS rule to each UE belonging to a PLMN or DNN or slice, in this case, the QFI and Packet Filter Set can be pre-configured in the corresponding SMF and UPF serving the whole PLMN or DNN or slice to ensure the consistency between UE and network. If a QoS rule is pre-configrued in a specific UE, the SMF can retrieve the corresponding QFI and Packet Filter Set from UDM and provides them to UPF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify the QFI and Packet Filter Set of the pre-configured QoS rule can be configured at SMF and UPF serving the whole PLMN/DNN/slice, or SMF retrieves them from UDM for a specific UE and provides them to UPF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If the QFI and Packet Filter Set is inconsistent between UE and network, there may have no corresponding QoS flow in the network, the pre-configured QoS rule can’t work. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.7.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

**************************************Start of 1st change*****************************************

5.7.1.4	QoS Rules

The UE performs the classification and marking of UL User plane traffic, i.e. the association of UL traffic to QoS Flows, based on QoS rules. These QoS rules may be explicitly provided to the UE (i.e. explicitly signalled QoS rules using the PDU Session Establishment/Modification procedure), pre-configured in the UE or implicitly derived by UE by applying Reflective QoS (see clause 5.7.5). A QoS rule contains the QFI of the associated QoS Flow, a Packet Filter Set (see clause 5.7.6) and a precedence value (see clause 5.7.1.9). Additionally, for a dynamically assigned QFI, the QoS rule(s) and the QoS Flow level QoS parameters (e.g. 5QI, GFBR, MFBR, Averaging Window) are signalled to the UE on per QoS Flow basis. An explicitly signalled QoS rule contains a QoS rule identifier which is unique within the PDU Session and is generated by SMF.

There can be more than one QoS rule associated with the same QoS Flow (i.e. with the same QFI).

A default QoS rule is required to be sent to the UE for every PDU Session establishment and it is associated with a QoS Flow. For IP type PDU Session or Ethernet type PDU Session, the default QoS rule is the only QoS rule of a PDU Session whose Packet Filter Set may contain a packet filter that allows all UL packets, and in this case, the highest precedence value shall be used for the QoS rule.

NOTE 1:	The filter in the Packet Filter Set of the default QoS rule that allows all UL traffic (also known as match-all filter) is described in TS 24.501 [47].

NOTE 2:	How the UE evaluates UL packets against the Packet Filter Set in a QoS rule is described in clause 5.7.1.5.

For Unstructured type PDU Session, the default QoS rule does not contain a Packet Filter Set, and in this case the default QoS rule defines the treatment of all packets in the PDU Session.

As long as the default QoS rule does not contain a Packet Filter Set or contains a Packet Filter Set that allows all UL packets, Reflective QoS should not be applied for the QoS Flow which the default QoS rule is associated with and the RQA should not be sent for this QoS Flow.

For a QoS rule pre-configured in the UE, the network ensures the corresponding QFI and Packet Filter Set are also pre-configured at SMF and UPF. e.g. the same QFI and Packet Filter Set may be pre-configured at each SMF and UPF serving a  specific PLMN or DNN or slice, or the SMF may retrieve the corresponding QFI and Packet Filter Set for a specific UE from UDM and provide them to UPF.

**************************************End of changes*******************************************