SA WG2 Meeting #128bis	S2-188424

20 - 24 August 2018, Sophia Antipolis, France	(Revision of S2-188263/6669)

Source:	Huawei, HiSilicon, Telecom Italia, Orange

Title:	        Solution for Data Collection from OAM using Existing SA5 Services

Document for:	Approval

Agenda Item:	6.11

Work Item / Release:	FS_eNA

Abstract of the contribution: This contribution proposes a solution for data collection from OAM using existing services as defined in SA5.

1 Discussion

Observation:

The issue of data collection from OAM associated with Key Issue 4: Interactions with OAM for Data Collection and Data Analytics Exposure is to be solved. The KI#4 regarding data collection should address how services from SA5 should be reused and which data can be collected.

SA5 has already defined services that can allow consumers to collect information about deployed network slices such as:

- PM (Performance Management) services as defined in TS 28.550 and TS 28.551

- NR and NG RAN or 5GC performance measurement services as defined in TS 28.552 and TS 28.553

- 5G End to end KPI services as defined in TS 28.554

- FS (Fault Supervision) services defined in the TS 28.545 and TS 28.546

Proposal:

This contribution proposes that NWDAF reuse the current data collection services as defined in SA5 for Data collection from OAM in Key Issue #4.

2 Proposal

It is proposed to make the following changes to the TR 23.791.

* * * * Start of Change * * * *

6.X	Solution X: Data Collection from OAM using the existing SA5 services

6.X.1	Description

This solution is for Data Collection from OAM associated with Key Issue 4: Interactions with OAM for Data Collection and Data Analytics Exposure.

6.X.1.1 General

Based on current definitions from SA5, NWDAF shall use as information to be collected:

- NR and NG RAN or 5GC performance measurements as defined in TS 28.552 and TS 28.553

- 5G End to end KPIs as defined in TS 28.554

NWDAF shall use the following services from SA5 to collect the aforementioned information:

- PM (Performance Management) services as defined in TS 28.550 and TS 28.551

- FS (Fault Supervision) services defined in the TS 28.545 and TS 28.546

6.X.1.2 Procedure for the support of Data Collection from OAM

The interactions between NWDAF and OAM for data collection are illustrated in Figure 1, which depends on the use cases.

OAM shall setup the proper mechanism to guarantee the continuous data collection requested by NWDAF

NOTE: Coordination with SA5 is required to clarify if measurements and KPIs required by NWDAF (as defined in the solutions for the key issues) can be provided.

Figure 6.X.1.2-1: Data collection from OAM services

- NWDAF subscribes to OAM services for data collection The subscription request shall optionally indicate to OAM the need to receive UP network slice instance topology information relevant to the analysis of the requested information

- [Optional] It is up to the OAM to identify if already deployed/configured support for data collection will be reused for each request from NWDAF or if new support for data collection needs to be deployed/configured.

- OAM services send notification to NWDAF about subscribed information.

NOTE: The call flow in Figure 6.X.1.2-1 only shows a subscribe/notify model for the simplicity, however both request-response and subscription-notification models should be supported.

6.1.2	Impacts on Existing Nodes and Functionality

Editor's note:	Capture impacts on existing 3GPP nodes and functional elements.

6.1.3	Solution Evaluation

Editor's note:	Use this section for evaluation at solution level.

* * * End of Change * * *