SA WG2 Meeting #129bis	S2-1812816

West Palm Beach, Florida, USA, November 26 - 30, 2018

Source:	Nokia, Nokia Shanghai Bell, Motorola Mobility, Lenovo, Convida Wireless LLC, Telecom Italia, NEC

Title:	KI#3 conclusions

Document for:	Approval

Agenda Item:	6.21

Work Item / Release:	FS_eNS/Rel-16

Abstract of the contribution:

# 1.	Proposal

It is proposed that the following text changes and proposals are added to TR 23.740.

***********************PROPOSED TEXT********************************

## 7.x	Evaluation KI#3

While the 6.3.1 solution was promising “to re-use the secondary authentication introduced in Release 15 to enable slice authentication” , based on inspection of the solution, it seems to require several dedicated developments that impacts all network elements in the CN. The UDM is impacted not only (even if not mentioned) for subscription policy that require secondary AUTH (like in solution 6.3.2) but also to retain status of authentication and SMF that needs to be contacted to revoke the slice authentication. Extensive SMF and AMF interactions specific for this feature are required and also  state maintenance in the system even across the roaming interfaces.

Solutions in 6.3.2  only impacts RM signalling, the solution in 6.3.1 has some drawbacks that seem not resolvable as the UE can stay registered in slices as long as it does not execute SM procedures.

## Because of the assessment here above, solution 6.3.2 should be the basis of normative work.

***********************MORE PROPOSED TEXT***************************

# 8	Conclusions

Editor's note:	This clause will capture conclusions from the study.

Editor's note:	For the Key Issue#1, Mutually exclusive access to Network Slices, the conclusions of the study and the resulting normative specifications should describe whether and how the following aspects are covered:

-	Whether the standard support the possibility for a Network Slice (S-NSSAI) to be associated to more than one group of Network Slices for which the access to the group of Network Slices are Mutually Exclusive from each other.

-	Whether it is possible to deploy an AMF Set which supports Network Slices that are mutually exclusive from each other.

For KI#3 it is concluded that Solution in clause 6.3.2 shall be the basis of normative work.

NOTE: 	Solution 6.3.2 has an open issue that needs resolution in normative phase with CT1 interactions. Base on the outcome of these interactions it will be possible to update the procedure, but the guiding principle is that this is based on AMF-UE interaction with no SMF interaction.

***********************END of PROPOSED TEXT***************************