SA WG2 Meeting #129BIS	S2-1813142

26 – 30 November, West Palm Beach, USA	(revision of S2-1813041)

Source:	Oracle, Deutsche Telekom, Cisco, AT&T, Sprint, Verizon, KDDI, NTT Docomo, China Mobile, ETRI, Affirmed Networks

Title:	Conclusion on KI#3 for Service Framework Implicit Discovery

Document for:	Approval

Agenda Item:	6.19

Work Item / Release:	FS_eSBA / Rel-16

Abstract of the contribution: This contribution proposes conclusions for key issue 3 on service framework implicit discovery.

# 1 Discussion

This contribution uses Service Framework Support Functions SFSF to denote the entity that can perform common service framework functions on behalf of a consumer or producer.  This contribution does not identify all possible common service framework functions.  For the purposes of this contribution, common service framework functions includes a message passing system, service discovery, and service selection functions.  Explicit service discovery and service selection is shown in figure 1.  Implicit service discovery and service selection is shown in figure 2.

Figure 1: Explicit Service Discovery and Selection

Figure 2: Implicit Service Discovery and Selection

This document proposes a conclusion on the implicit discovery via the Service Framework Support Function (SFSF) based on figures 1 and 2 in order to avoid any misinterpretation.

# 2 Proposal

This document proposes the following changes into TR 23.742.

**************** Start change all new text **********

## 8.x	Conclusions for the service framework

Principles related to registration and discovery of services

A.1)	Rel-16 will support implicit discovery, ie. that NF Services are discovered and selected by the Service Framework Support Function without an explicit discovery request from the consumer.

a)	Discovery and selection of a suitable NF Service instance is based on a combination of 3GPP specified attributes as metadata, for example:

-	Service Type of the provider

-	Service Type of the consumer

-	S-NSSAI

-	Service Instance Set ID

-	Service Zone ID

-	Service Area

-	DNN

Note 1:	The list of 3GPP specified attributes which may be used as metadata for discovery will be finalized during the normative process.

Note 2:	The SFSF is not assumed to have specific 3GPP application knowledge in order to perform discovery and selection. The SFSF matches the attributes that are requested by the service consumer in the service request to the attributes that are provided by the service producer at the NF Service instance registration, and identifies a set of suitable candidate NF Service instances based on the match and select an instance.

A.3)	For model D1 (as shown in Annex X) the SFSF may not perform a full service discovery for each request received . For example, a request from a service consumer targeting a previously created resource in a service producer use the resource URI for selection of a member in a specific set of service producer instances without having to discover the set.

********************  End of changes  ********************