SA WG2 Meeting #129BIS	S2-1813351

26 - 30 November 2018, West Palm Beach, Florida	(revision of S2-181xxxx)

Source:	Huawei, HiSilicon, Ericsson, Intel, Mediatek, InterDigital Inc, LGE, OPPO

Title:	Proposed way forward for KI#1

Document for:	Approval

Agenda Item:	6.21

Work Item / Release:	FS_eNS / Rel-16

Abstract of the contribution: This contribution proposes a way forward for the specification work for Rel-16 regarding KI#1 of eNS.

# 1	Discussion

In the spirit of completing the work for FS_eNS, a compromise resulting on the following proposal, a merge of solution #2 and solution #5, is proposed.

# 2	Proposal

It is proposed to update TR 23.742 as follows:

First change

# 8	Conclusions

Editor's note:	This clause will capture conclusions from the study.

For KI#1, it has been decided to move forward with the following solution, based on solution 2 and solution 5, for the specification work with the following informative content:

-	An explanation that, the UE can optionally be locally (pre-)configured with information (by mechanisms not specified in 3GPP specifications) regarding the S-NSSAIs that can (or cannot) be requested simultaneously by the UE;

-	An explanation that, the UE can optionally be locally configured (by mechanisms not specified in 3GPP specifications) to sort S-NSSAIs in the Requested NSSAI in a priority order;

-	An explanation that the AMF/NSSF, when receiving a Requested NSSAI with S-NSSAIs that cannot be allowed simultaneously, can decide to consider the S-NSSAIs in the Requested NSSAI in priority order; and

-	Possibly, an explanation how these mechanisms can allow an operator to fulfil the KI#1 scenarios.

It is expected that this work can be completed via a single CR to 23.501.

End of changes