3GPP TSG-SA WG2 Meeting #129	S2-1910052

Dongguan, China, 15 - 19 October, 2018	rev. of. S2-187948

Source:	ORANGE, AT&T, Interdigital, HUAWEI

Title:	Updates of Solution #9

Document for:	Approval

Agenda Item:	6.11

Work Item / Release:	FS_eNA

Abstract of the contribution: The proposal addresses updates of Solution #9 “Recommendations produced by NWDAF”.

# Discussion

The modifications address the following topics:

| Topic | Reason | Proposal |
| --- | --- | --- |
| Editor's notes: How to define input parameters provided in the recommendation request is FFS. Especially considering that NWDAF is an entity to provide statistics / prediction and is not assumed to run NF logic | The text of Editor’s note is taken into account in the §Impact The constraints in the Recommendations requests (§6.9.1.5) may be used by NFs to guide recommendations produced by the NWDAF depending on the NF and use case. Therefore the NWDAF computations can be generic and independent of NF logic. | - Removal of Editor’s notes (§6.9.1.1, §6.9.1.5) - Addition of text in Solution evaluation (§6.9.2): last to paragraphs: “The arrays…parameters).” - Addition of sentence: “These constraints may be used by NFs to guide recommendations by passing UE subscription profile values.” - Addition of optimization goals: there may be several different goals to achieve, the choice of which depends on NF logic and business case. |
| Editor’s note about parameters, leaving them for further study (§6.9.1.6) | Editor’s note denoting a “further study in the TR” is changed into a NOTE describing the topic as to be addressed in the normative phase | - Removal of Editor’s notes (parameters) - Addition of text “NOTE: The exact definition of … is left for the normative phase.” |
| §Impacts and §Evaluation | 3GPP default Editor’s notes signalling the missing parts | Addition of the §Impact and §Evaluation chapters |
| Various changes | Editor’s note on the possibility to have periodical notification (§6.9.1.5) | Additional text to be align with the NEF and Solution #2 |

# Proposal

It is proposed to modify the following clause in TR 23.791.

**** Start of Change ****

## Solution 9: Recommendations produced by NWDAF

### 6.9.1	Description

#### 6.9.1.1	Service principles

This solution addresses Key issue #1: "Network Data Analytics feedback".

The NWDAF offers a Recommendations service in order to propose recommendations (parameter values) to NFs.

The use of the Recommendations service by an NF is optional.

The purpose of the service is to better assist some NFs in the elaboration of its strategy, in certain use cases.

Each NF may request the NWDAF Recommendations service for instance in case its decision may impact other NFs behaviours.

Each type of recommendation belongs to a given domain (e.g. registration, reachability, mobility, service and application, forwarding, routing, roaming, security, resilience). There may different types of recommendations per domain.

The NWDAF apart from providing statistical/prediction information, shall not intervene in the area of responsibility of each NF, which shall remain solely responsible for its decisions. The NFs may additionally coordinate their decisions with other NFs.

After receiving a recommendation that the NF has requested, the NF shall then, according to its own criteria

(for example applicative and business criteria) and its own service logic, make its optimal choice within its area of responsibility.

The NF consumers of the NWDAF Recommendations service are fully in charge of all decisions concerning their own tuning. As a consequence, they should be able to operate with good performance in most situations, even in case of NWDAF absence or failure.

The NWDAF should strive to ensure consistency in its recommendations, taking into account the previous recommendations performed across the different domains and the other NFs, in order to avoid conflicted decisions.

#### 6.9.1.2	Service definition

The NWDAF should expose a generic service interface NWDAF_RecommendationsService offering the following operations.

Table 6.9.1.2-1

| Operations | Description of each operation of the Recommendations Service |
| --- | --- |
| Subscribe | Subscription to recommendations |
| Unsubscribe | Cancel of a previous subscription |
| Notify | Notification of recommendations to the consumer NF |
| Request | Request to recommendations |

The Recommendations service proposes pre-computed vectors of optimal parameter recommended values to NFs.

A parameter recommended value is a single value or a range, for possible integration in the rules of the consumer NF.

Each consumer NF shall provide in input, for a given type of recommendation, the target objects that are related to the parameter recommend values that will be derived. The NF may impose computational constraints (e.g. ranges of parameters to explore) on the results.

The NWDAF in return shall provide, for each requested recommendation, a list of one or more suggested parameter combinations. For instance, these combinations can be lists of candidate QoS dedicated parameters, or lists of candidate UPFs for selection.

Each combination is a vector of recommended parameter values, possibly with associated impacts (predicted KPIs) and a preference order, consistent between different domains.

#### 6.9.1.3	Recommendations Subscribe/Unsubscribe

This procedure is used by any NF service consumer to subscribe/unsubscribe at NWDAF and to be notified of recommendation information.

Figure 6.9.1.3-1: NWDAF Recommendations Subscribe/Unsubscribe/Notify

1.	The NF service consumer subscribes to or cancels subscription to recommendations by invoking the Nnwdaf_Recommendations_Subscribe or Nnwdaf_ Recommendations _Unsubscribe service operation.

2.	If NF service consumer has subscribed to recommendations, the NWDAF notifies the NF service consumer with the recommendations by invoking Nnwdaf_ Recommendations _Notify service operation.

#### 6.9.1.4	Recommendations Request

This procedure is used by the NF service consumer (e.g. UDM, AMF, SMF, PCF) to request and get from NWDAF recommendations, using Nnwdaf_Recommendations_Request.

Figure 6.9.1.4-1: NWDAF Recommendations Request and Response

1.	The NF service consumer requests recommendations by invoking Nnwdaf_Recommendations_Request service operation.

2.	The NWDAF responds with recommendations to the NF service consumer.

#### 6.9.1.5	Contents of subscribe and request operations

In addition to basic SBI parameters defined in TS 29.520 [10], the requests or the subscription should contain:

-	The desired recommendation on a given domain, expressed as a recRequirement object.

-	Observation period [start..end] in the future on which recommendations are requested.

-	Maximum tolerable delay for the provision of the recommendations.

- 	Event parameters for periodic notification (as defined in Solution#2),

-	Optional preferences on recommendations (as defined in Solution#2),.

Editor's note:	whether preferences on recommendations (e.g. algorithms used, number of iterations, authorization of using subsets or samples in order to reduce computation time) are provided and the parameters regarding to the preferences on analytics/predictions is left for further study.

-	Filters (event matching parameters): already defined in Solution #2 to Key Issue#1.

A recRequirement object is composed of:

-	the recommendation registered ID in the given domain.

-	optional optimization goals: this parameter denotes the objectives of optimization requested, if several optimizations methods are possible. Several simultaneous objectives may be requested and hierachized. The requesting NF should use these goals parameters in order to fit its own strategy.

-	optional constraints per requested parameter, expressed as parameter ranges. These constraints may be used by NFs to guide recommendations by passing UE subscription profile values.

Editor's note:	The possibility of subscribing simultaneously to multiple recommendations is left for further study.

#### 6.9.1.6	Contents of notifications and responses

In addition to the basic SBI parameters defined in TS 29.520 [10], the response to Request or the notification should contain:

-	Identification of the Request /Subscribe operation.

-	Optional confidence score on recommendations.

-	Possible limitations on the observation period and requested delay.

-	Recommendations: candidate recommendations relevant for the observation period, expressed as an array of elementary recommendation objects recResult.

Each recResult is a composed of:

-	Reference to the recommendation registered ID in the recommendation domain

-	The vector of tagged parameter recommended values or value ranges,

-	Optionally: the associated impacts (a set of predicted analytics) if such parameters are chosen

-	Optionally: a preference order in order to suggest a preference among the various recResult objects

A recommended parameter may belong to the following categories.

Table 6.9.1.6-1

| Parameters categories | Example of parameters |
| --- | --- |
| Time | Periodic registration timer, back-off timers |
| QoS | QoS profiles |
| NF | Identification of proposed UPF |
| Load | Load reduction ratio |
| UEs | Identification of UEs suspected of performing a security attack |
| Location | List of relevant TA for the UE e.g. for paging or mobility restriction |
| Activity | Handover/No handover, blacklist UEs |

Each parameter should have a clearly defined semantic and format, and should typically be defined in a 3GPP standard.

NOTE: The exact definition of parameter types and associated impacts is left for the normative phase

The NWDAF may return an error if the parameters (in immediate response to subscription or request), or the traffic conditions do not permit to deliver relevant recommendations in due time. It may also return, in case of error, limitations of requested observation period and tolerable delay. It may also return, in case of coordination between different recommendations, the action it should take to use the response (wait a delay, wait for an event of another domain, etc.).

### 6.9.2	Impacts on Existing Nodes and Functionality

The solution requires the implementation of a new service on the NWDAF.

### 6.9.3	Solution Evaluation

The proposed solution takes benefit of the NWDAF computation capacities (e.g. artificial intelligence, machine learning, storage), in order to obtain in the end more consistent strategies for NFs and AFs due to the central position of the NWDAF which provides global network knowledge. For instance, it can facilitate the decisions related to QoS provisioning (see Solution #3) and adjustment, or NF selection.

The arrays of recommendations provided by the NWDAF are support data for NF decision. These recommendations are based on computational analysis guided by NF specific constrains, enabling the NWDAF to provide processed data beyond sheer statistics and predictions. For instance, the NF may choose a solution of secondary preference order in the array, for reasons relevant to the NF logic. The NF may also simply ignore all recommendations, for the same reason.

**** End of Changes ****