SA WG2 Meeting #129	S2-1810706

Dongguan, China, October 15-19, 2018	(updated revision of)

Source:	Nokia, Nokia Shanghai Bell

Title:	evaluation of solution 6.1.5

Document for:	Approval

Agenda Item:	6.21

Work Item / Release:	FS_eNS/Rel-16

Abstract of the contribution:

# 1.	Proposal

It is proposed the following text is added to TR 23.740

***********************PROPOSED TEXT********************************

### 6.1.5.5	Evaluation

Editor's note:	This clause provides an evaluation of the solution.

This solution assumes the Mutual exclusivity deployment of AMF. So, this poses constraints on the deployments if they need to support mutual exclusivity.

This solution does not propose a way for the UE to learn the mutual exclusion rules that is testable.

Since low priority slices tend to be rejected in this proposal regularly, when mutual exclusion is used, this inherently requires a dynamic priority allocation scheme is needed. Then each time priority is changed new rules need to be learned by the UE as possibly different AMFs are selected and in those AMFs the rules of coexistence the UE learns are different as they are linked to the new priority.

This solution is too open ended and not testable so operators can hardly rely on this to have deterministic behaviour.

***********************END of PROPOSED TEXT**************************