3GPP TSG-SA WG2 Meeting #129	S2-1811533

Dongguan, China, 15 - 19 October, 2018

Source:	ORANGE, AT&T, HUAWEI, Convida Wireless LLC, Ericsson, Nokia

Title:	Updates of Solution #2 and Solution #1 (merging together)

Document for:	Approval

Agenda Item:	6. 11

Work Item / Release:	FS_eNA

Abstract of the contribution: The proposal updates and merge of Solutions #1 and #2.

# Discussion

It was agreed during SA2#129 to merge Solutions 1 and 2 in TR 23.791.

The present document implements this decision by:

- replacing the content of clause 6.1 by the merge of S2-1810050 (for Solution #2), S2-1810669 (for Solution #1) and S2-1811076 (for Solution #1), to which additional modifications agreed during SA2#129 are made.

- voiding clause 6.2.

# Proposal

It is proposed to modify the following in TR 23.791.

**** Start of Change ****

## 6.1	Solution 1: Network Data Analytics Feedback

### 6.1.1	Description

#### 6.1.1.0	General

This is a solution to:

-	Key Issue#1: Analytic Information Exposure to 5GS NF

-	Interactions with OAM for Data Analytics Exposure issue associated with Key Issue#4.

This solution is to define the framework for network data analytics feedback and the detailed parameters of the network data analytics will be defined in other solutions.

The solution extends the features of NWDAF as exposed in clause 4.19 of TS 23.502 by enabling statistics for data or events collected in the past and based on these statistics, predictions for data or events in the future. It integrates Event Filter Information, Event Reporting Information, and Target of Event Reporting as defined for network exposure framework in clause 4.15.1 of TS 23.502.

#### 6.1.1.1	Network data analytics Subscribe/Unsubscribe

This procedure is used by any NF service consumer) to subscribe/unsubscribe at NWDAF to be notified on analytic information, using existing Nnwdaf services defined in TS 23.502 [3]. Any entity can consume this service as far as they follow SBI defined in 23.502[3].

Figure 6.1.1.1-1: Network data analytics Subscribe/unsubscribe

1.	The NF service consumer subscribes to or cancels subscription to analytic information by invoking the Nnwdaf_EventsSubscription_Subscribe/ Nnwdaf_EventsSubscription_Unsubscribe service operation.

2.	If NF service consumer subscribes to analytic information, the NWDAF notifies the NF service consumer with the analytic information by invoking Nnwdaf_EventsSubscription_Notify service operation.

#### 6.1.1.2	Network data analytics Request

This procedure is used by the NF service consumer (e.g. UDM, AMF, SMF, PCF) to request and get from NWDAF analytic information, using Nnwdaf_AnalyticsInfo_Request defined in TS 23.502 [3].

Figure 6.1.1.2-1: Network data analytics Request

1.	The NF service consumer requests analytic information by invoking Nnwdaf_AnalyticsInfo_Request service operation.

2.	The NWDAF responds with analytic information to the NF service consumer.

#### 6.1.1.3	How a NF requests or subscribes to analytics

A NF subscribes or requests analytics using the Nnwdaf service defined in clause 6.1.1.1 that needs to allow new EventIds to resolve the existing key issues, note that each key issue will include specific EventID and those are listed as examples here:

Table 6.1.1.3-1

| Event ID | Event Filter | Description (examples of patterns) |
| --- | --- | --- |
| UE Behavior parameters Expected Moving Trajectory | List of UE identities, Time, Date | UE´s expected geographical movement (e.g. as described in TS 23.502 [3] clause 14.5.6.3). |
| UE Behavior parameters Communication pattern |  | UE expected communication (e.g. communication of low latency applications at a given time and date, or |
|  |  | Time and date when UE does not have any communication) |
| NW performance Pattern | NW area, Time, Date | NW expected performance pattern (e.g. Estimated load in a network area at a given time and date). |

NOTE:	If Subscription information includes a UE identity or a list of UE identities, privacy issues should be taken into account, e.g. user consent.

Each analytics defined in other solutions should reuse the following same framework to define the parameters of the analytics request to the NWDAF.

#### 6.1.1.3.1	Content of Analytics requests/subscribe

##### 6.1.1.3.1.1	General

In addition to basic SBI parameters defined in TS 29.520 [10], the requests or the subscription to analytics should contain:

-	Type of analytics: Event ID as defined in TS 23.502 [3] sub-clause 4.15.1 , possibly fit to use cases (e.g. service quality of network slice, UE expected behaviour, CP or UP related analytics, etc).

- 	Analytics filtering information: Event Filter Information (as defined in TS 23.502 clause 4.15), this set of parameter types and values enables to select which type of analytics information are requested (e.g. subset of all available statistics produced by NWDAF for the given Event ID value). For example, Event ID #01, has the Event Parameter Type “service quality of network slice” and the parameter values (i.e. available statistics) are: service satisfaction level, number of registrations, etc. It provides Event Parameter Types and Event Parameter Value(s) to be matched against.

-	Target of Event Reporting (as defined in TS 23.502 clause 4.15.1): the object targeted by analytics/predictions (e.g. slice, region, NF set, NF Id, UEs, group of UE(s), any UEs). The Target of event reporting defined in TS 23.502 [3] sub-clause 4.15.1 is reused (and extended) for that purpose.

-	Observation period: time interval [start..end] covered by the computation, either in the past or in the future, expressed as positive or negative offsets to the present. An interval in the past is a request for statistics. An interval in the future is a request for predictions. A default value means statistics in the recent past, with a time range defined by the NWDAF.

NOTE: The available statistics depend on the use cases handled by the NWDAF.

-	A Notification Target Address (+ Notification Correlation ID) as defined in TS 23.502 [3] sub-clause 4.15.1, allowing the Event Receiving NF to correlate notifications received from the NWDAF with this subscription.

-	Event Reporting Information (as described in the TS 23.502 [3] Table 4.15.1-1)

This parameter already supports the configuration of the Mode of Reporting (e.g reporting up to a maximum number of reports allowing a one shot analytics request, periodic reporting along with periodicity, reporting up to a maximum duration) or the request to notify immediately the current status of the subscribed statistics, if available, to the requesting NF.

-	Optional preferences on analytics (e.g. computation constraints).

NOTE: the exact definition of preferences on analytics/predictions is left for the normative phase.

##### 6.1.1.3.1.2	Target of Event Reporting

In TS 23.502 clause 4.15.1, the target of event reporting could indicate entities such as specific UEs, or PDU session, a group of UE(s) or any UE (i.e. all UEs). The entities that can be indicated by the target of event reporting are extended for the generation of analytics. The target of event reporting is a set of value assertions (i.e., Target Type = value) which determines which entities are concerned for the production of relevant analytics.

Each parameter may or may not be support wildcards. In case of wildcards, it shall be possible to specify either an empty wildcard meaning "all values", or a constrained wildcard by denoting a range of values.

The parameters may be organized as a set of additional value assertions (equality assertions or more improved)

Table 6.1.1.3.1.2-1 describes the level roles and examples of parameters associated with a target of event reporting.

Table 6.1.1.3.1.2-1

| Level | Level role | Example of Target Type | Value assertion (examples) |
| --- | --- | --- | --- |
| 0 | slice ID | S-NSSAI | = "MBB" |
| 1 | NF ID | NF, NF set, NF region | = "all NFs" |
| 2 | user ID | SUPI/PEI/GPSI, group ID | = "All UE-groups" |
| 3 | Area/location | TAI, Cell ID, geographical area, area type (rural/urban) | = "Cell-3" |
| 4 | Application | DNN, AFid, | = "MMS" |
| 5 | Activity |  |  |

NOTE 1: The exact definition of parameters is left for the normative phase.

NOTE 2: the need and usage of NF related Target Types is to be defined in conjunction with SA WG5.

In complement, the output mode mode may be used to determine whether the provided statistics are aggregated for all entities that are targeted or individually listed i.e. provided for each individual Target. For at most one single Target Type and one single parameter within a subscription, it should be possible to specify if it is individually listed. This permits to collect an array of statistics on one single dimension, in order for the consumer NF to compare statistics and facilitate its decisions.

When level aggregation is chosen, the cumulative or mean value of the measurement shall be provided, depending on the type of statistics. For instance, if the statistics "registration counter" is requested aggregated, only one aggregated counter is provided which totalizes counters on the various. Aggregation rules should be specified per statistics type.

When individually listed, all individual values of the target parameter corresponding to the wildcard are requested. For instance, if the statistics "number of registrations" is requested as an individually list at level UserGroup-ID, an array of statistics is provided, one per each value of UE-groups (e.g. list of <UE-group-counter>=<number of registrations>).

Table 6.1.1.3.1.2-2 describes a simplified example of content of Subscription/Request with event parameter value of the Event Filter Information and Target of Event Reporting including explicit list.

Table 6.1.1.3.1.2-2: Simplified example of content of Subscription/Request

| Event Parameter value | Target of Event reporting |
| --- | --- |
| Number of registrations (N) | Slice ID = ”SliceA” and AFId= “App1“ and [<UE-group1>; <UE-group2 >; <UE-group3>] |
| Service satisfaction level (R) |  |

#### 6.1.1.4	How NWDAF provides analytics

A NF subscribes or requests analytics using the Nnwdaf service defined in clause 6.1.1.1 that needs to allow new EventIds to resolve the existing key issues. Each subscription or request for an EventId provides a pattern.

The NWDAF functionality does not overlap the existing NF functionality. Each pattern enhances existing NF functionality such as Mobility Management (by AMF), Session Management (by SMF), Background Data Transfer (in PCF).

Each analytics defined in other solutions should reuse the same following framework to define the parameters of the analytics request to the NWDAF. This framework follows the framework already defined in TS 23.502 [3] clause 4.15.

#### 6.1.1.4.1	Content of Analytics responses/notifications

In addition to basic SBI parameters defined in TS 29.520 [10], the response or the notification to analytics should contain:

-	Identification of the request/subscription including:

- Notification Correlation Id (provided in the subscription request)

- The Event ID (one of the Event Id provided in the request),

-	Target Object Values, which contains values associated with the subscribed/requested analytics e.g. UE ID (SUPI and if available GPSI), NF ID, slice Id etc..

- The analytics output include:

-	Related statistics (or arrays of statistics) on the requested observation period, per Target Object Value;

-	Optional additional information:

-	Timestamp of analytics generation, which allows consumers to decide until when the received information shall be used. For instance, an NF can deem a received notification from NWDAF for a given feedback as invalid based on this timestamp.

-	Probability assertion: level of certainty, degree of confidence in statistics/prediction.

Table 6.1.1.4.1-1 describes a Simplified example of content of notification/response with Target Object Values.

Table 6.1.1.4.1-1: Simplified example of content of notification/response

| statistics for N-SSAI="SliceA" and AFId= "App1" | Array of statistics per Target Object Value in the case of explicit list |
| --- | --- |
| Number of registrations (N) | <UE-group1, N=1973>; <UE-group2, N=29382>; <UE-group3, N=293> |
| Service satisfaction level (R) | <UE-group1, R=90%>; <UE-group2, R=95%>; <UE-group3, R=85%> |

### 6.1.2	Impacts on Existing Nodes and Functionality

NWDAF: New EventIds are provided. No impacts on the existing Nnwdaf interface, except for the slicing information that should become optional, as other event filters should be allowed.

Consumer NF: For a NF such as PCF or NSSF that are consumers of Nnwdaf service, the impacts are to support new EventIds. For other new consumers, the impacts are listed the solutions to each key issue.

The solution is compatible with the existing NWDAF services as exposed in clause 4.19 of TS 23.502 (NnwdafAnalyticsInfo and Nnwdaf_EventSubscription). The content of the operations for subscribe/request and notify/response of such services are extended. The solution extends the features of NWDAF as exposed in TS 23.502, by enabling both statistics in the past and predictions in the future.

It integrates Event Filter Information, Event Filter information, and event target, parameters as defined for network exposure framework in clause 4.15 of TS 23.502 or clause 5.3 of TS 29.518

### Usage of non UE targets (NFs or groups of NFs) is optional. It needs to be defined in conjunction with SA WG5. Non UE targets (e.g. NFs) may typically be requested to help optimizing NF selection.

### 6.1.3	Solution Evaluation

The solution provides application-agnostic analytics.

It enables to perform comparisons of statistics in the past and predictions in the future related to various domains, leaving all decisions to the consumer NF.

This solution preserves the current defined services for analytics exposure defined in NWDAF, i.e., Nnwdaf_EventSubscription and Nnwdaf_AnalyticsInfo, as it indicates that the procedures for analytics exposure are the same as in Solution 1.

This solution keeps the alignment with the Network Exposure defined in TS 23.502 clause 4.15. Nevertheless, this solution defines in detail the content of subscribe/request and notify/response messages associated with such services.

The proposed content of these messages requires the current definition of the NWDAF service operations in TS 23.502 to be extended to support such solution.

### The proposed extensions of the NWDAF service operations allows NWDAF to provide analytics that go beyond network slice load analytics as defined in Rel-15 (TS 23.501) and supports thus the analytics identified as required in the Use Cases defined in clause 5.1.

## 6.2	Void

### NOTE: The content of this clause was merged into clause 6.1.

**** End of Changes ****