3GPP TSG-SA2 Meeting #130	S2-1900758

# Kochi, India, 21st Jan 2019 - 25th Jan 2019							 (revision of S2-1900613)

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 0967 | rev | 01 | Current version: | 15.4.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI-#5 specific AF influence on traffic routing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NEC, NTT DOCOMO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | ETSUN |  |  |  |  |  | Date: |  |  | 2019-01-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | There are deployment scenarios studied and concluded in ETSUN TR 23.726 where a signle SMF may not always control all the UPFs, and thus requiring addition, modification or removal of an intermediate SMF or I-SMF. Agreed solutions to address these cases were to introduce a new interface between SMF and I-SMF, with required functionalities including those related to Handling AF influence on traffic routing. Corresponding changes were discussed and solution #15 for KI#5 was agreed and included in ETSUN TR 23.726. These needs to be captured in clause 4.3.6 of TS 23.502. Below changes are made in Ver-01, based on the received comments: - Changed the clause-structure i.e. instead of under 4.3.6, move them to sub-clause 4.x.y - Keep the figure more general e.g. remove AF in the flow and focus on interface 16a - Add a note for further details addition to 16a interface - Included NTT DOCOMO as source company: this is merged tdoc |  |  |  |  |  |  |  |  |
| Summary of change: |  | Additional text/steps related to Handling AF influence on traffic routing, those agreed in ETSUN TR 23.726, specific to KI#5, solution #15 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | ETSUN conclusions related to KI#5 not captured in TS 23.502 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.6.2, 4.3.6.3, 4.3.6.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

***** 1st Change *****

4.x.y	PCF Services for Deployments with I-SMF

This clause provides PCC related details of N16a interface between SMF and I-SMF.

4.x.y.1 AF Influence on Traffic Routing for Deployment with I-SMF

Below figure 4.x.y-1 shows procedure related to provisioning of PCC rules to I-SMF over the 16a interface.

Figure 4.x.y-1

When a new I-SMF is inserted, the I-SMF sends DNAI list supported by I-SMF to SMF.

In cases where step 1a in figure 4.x.y-1 is triggered in response to PCF receiving AF request, either over N5 or via NEF, below steps 3 and 4 are followed, in additional to those explained in 4.3.6.1.

Step 3:   SMF provides PCC rules to I-SMF based on Npcf_SMPolicyControl_UpdateNotify received from PCF. PCC rules allows I-SMF in e.g. traffic routing related decisions, usage reporting, I-UPF selection, QoS enforcement to support scenarios with UL-CL/BP controlled by I-SMF.

4.	I-SMF may take appropriate actions to reconfigure the User plane in its service area of the PDU Session such as:

-	Adding, replacing or removing a UPF in the data path to e.g. act as an UL CL or a Branching Point e.g. as described in clause 4.3.5.

-	Updating the UPF in the target DNAI with new traffic steering rules

Subscribe to notifications from the AMF for an Area Of Interest via Namf_EventExposure_Subscribe service operation

NOTE: Further details on steps 3 and 4 to be added, if required.

4.x.y.2  I-SMF Notification to SMF

Below figure 4.x.y-2 shows procedures related to I-SMF providing event notification, etc to SMF over the 16a interface.

Figure 4.x.y-2

Clause 4.3.6.3 provides details on notification of user plane management events. In case AF has subscribed for user plane management event notifications, then steps 2a and 4a in figure 4.x.y-2 are followed as below for early as well as late notifications:

1a. A condition for an AF notification, as described in 4.3.6.2, has been met. I-SMF sends notification to the SMF as shown in step 2a.

2a. If early notification is requested by SMF on be half of AF, as described in 4.3.6.2, then I-SMF notifies the SMF of user plane management events by invoking Nsmf_EventExposure_Notify service operation. SMF then includes these as inputs to its notification to AF.

4a. In case of late notification I-SMF sends to the SMF of user plane management events e.g. newly added UL CL and I-UPF details by invoking Nsmf_EventExposure_Notify service operation. SMF then includes these as inputs to its notification to AF.

***** End of Changes *****