3GPP TSG-SA WG2 Meeting #130	S2-1900790

# Kochi, India, 21 – 25 January 2019 												(revision of S2-1900088)

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 0745 | rev | 1 | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Mobility restrictions for wireline access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5WWC |  |  |  |  |  | Date: |  |  | 2019-01-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Mobility restrictions are not supported for wireline access |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add support for mobility restriction when using wireline access |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.4.1.1, 5.3.4.1.2, 5.3.4.1.2a (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

##### 5.3.4.1.1	General

Mobility Restrictions restrict mobility handling or service access of a UE. The Mobility Restriction functionality is provided by the UE (only for mobility restriction categories provided to the UE), the radio access network and the core network.

Mobility Restrictions only apply to 3GPP access and wireline access, they do not apply to other non-3GPP accesses.

For 3GPP access, Service Area restrictions and handling of Forbidden Areas for CM-IDLE state and, for CM-CONNECTED state when in RRC Inactive state are executed by the UE based on information received from the core network. Mobility Restrictions for CM-CONNECTED state when in RRC-Connected state are executed by the radio access network and the core network.

For wireline access, Service Area restrictions and handling of Forbidden Areas for CM-CONNECTED state are executed by the AMF based on information received from UDM and PCF.

In CM-CONNECTED state, the core network provides Mobility Restrictions to the radio access network within Mobility Restriction List.

Mobility Restrictions consists of RAT restriction, Forbidden Area, Service Area Restrictions and Core Network type restriction as follows:

-	RAT restriction:

Defines the 3GPP Radio Access Technology(ies), a UE is not allowed to access in a PLMN. In a restricted RAT a UE based on subscription is not permitted access to the network for this PLMN. For CM-CONNECTED state, when radio access network determines target RAT and target PLMN during Handover procedure, it should take per PLMN RAT restriction into consideration. The RAT restriction is enforced in the network, and not provided to the UE.

-	Forbidden Area:

In a Forbidden Area, the UE, based on subscription, is not permitted to initiate any communication with the network for this PLMN. The UE behaviour in terms of cell selection, RAT selection and PLMN selection depends on the network response that informs the UE of Forbidden Area.

NOTE 1:	The UE reactions to specific network responses are described in TS 24.501 [47].

-	Service Area Restriction:

Defines areas in which the UE may or may not initiate communication with the network as follows:

-	Allowed Area:

In an Allowed Area, the UE is permitted to initiate communication with the network as allowed by the subscription.

-	Non-Allowed Area:

In a Non-Allowed Area a UE is service area restricted based on subscription. The UE and the network are not allowed to initiate Service Request or SM signalling to obtain user services (both in CM-IDLE and in CM-CONNECTED states). The UE shall not use the entering of a Non-Allowed Area as a criterion for Cell Reselection, a trigger for PLMN Selection or Domain selection for UE originating sessions or calls. The RRC procedures while the UE is in CM-CONNECTED with RRC Inactive state are unchanged compared to when the UE is in an Allowed Area. The RM procedures are unchanged compared to when the UE is in an Allowed Area. The UE in a Non-Allowed Area shall respond to core network paging or NAS Notification message from untrusted non-3GPP access with Service Request and RAN paging.

NOTE 2:	When the services are restricted in 5GS due to Service Area Restriction, then it is assumed that the services will be also restricted in all RATs/Systems at the same location(s) using appropriate mechanisms available in the other RATs/Systems.

-	Core Network type restriction:

Defines whether UE is allowed to connect to 5GC for this PLMN.

NOTE 3:	The Core Network type restriction can be used e.g. in network deployments where the E-UTRAN connects to both EPC and 5GC as described in clause 5.17.

For a given UE, the core network determines the Mobility Restrictions based on UE subscription information, UE location and local policy. The Mobility Restriction may change due to e.g. UE's subscription, location change and local policy. Optionally the Service Area Restrictions or the Non-Allowed Area may in addition be fine-tuned by the PCF e.g. based on UE location, PEI and network policies. Service Area Restrictions may be updated during a Registration procedure or UE Configuration Update procedure.

NOTE 4:	The subscription management ensure that for MPS service subscriber the Mobility Restrictions is not included.

If the network sends Service Area Restrictions to the UE, the network sends only either an Allowed Area, or a Non-Allowed Area, but not both at the same time, to the UE. The network does not send any Allowed Area or Non-Allowed Area to the RG (UE) for wireline access. If the UE has received an Allowed Area from the network, any TA not part of the Allowed Area is considered by the UE as non-allowed. If the UE has received a Non-Allowed Area from the network, any TA not part of the Non-Allowed Area is considered by the UE as allowed. If the UE has not received any Service Area Restrictions for 3GPP access, any TA in the PLMN is considered as allowed.

If the UE has overlapping areas between Forbidden Areas, Service Area Restrictions, or any combination of them, the UE shall proceed in the following precedence order:

-	The evaluation of Forbidden Areas shall take precedence over the evaluation of Service Area Restrictions.

The UE and the network shall override any Forbidden Area, Non-Allowed area restrictions and Core Network type restriction whenever access to the network for regulatory prioritized services like Emergency services and MPS.

**** Next Change ****

##### 5.3.4.1.2	Management of Service Area Restrictions for 3GPP access

This clause describes Service Area Restrictions for 3GPP access. For Service Area Restrictions when using wireline access, see clause 5.3.4.1.2a.

For 3GPP access, a Service Area Restriction may contain one or more (e.g. up to 16) entire tracking areas each of which belongs to either the registered PLMN or its equivalent PLMN, or all tracking areas of the registered PLMN. The UE's subscription data in the UDM includes a Service Area Restriction which may contain either Allowed or Non-Allowed Areas–specified by using explicit tracking area identities and/or other geographical information (e.g., longitude/latitude, zip code, etc). The geographical information used to specify allowed or non-allowed area is only managed in the network, and the network will map it to a list of TAs before sending Service Area Restriction information to the PCF, NG-RAN and UE. The Allowed Area may also be limited by a maximum allowed number of tracking areas, or the allowed area may alternatively be configured as unlimited i.e. it may contain all tracking areas of the PLMN. The registration area of a UE in the non-allowed area should consist of a set of TAs which belongs to a non-allowed area of the UE. The registration area of a UE in the allowed area should consist of a set of TAs which belongs to an allowed area of the UE. The AMF provides the Service Area Restriction in the form of TA(s), which may be a subset of full list stored in UE's subscription data, to the UE during the Registration procedure.

NOTE:	As the finest granularity for Service Area Restrictions are at TA level, subscriptions with limited geographical extent, like subscriptions for Fixed Wireless Access, will be allocated one or a few TAs and will consequently be allowed to access services in a larger area than in e.g. a FWA system.

The UDM stores the Service Area Restrictions of a UE as part of the UE's subscription data. The PCF in the serving network may (e.g. due to varying conditions such as UE's location, application in use, time and date) further adjust Service Area Restrictions of a UE, either by expanding an allowed area or by reducing a non-allowed area or by increasing the maximum allowed number of tracking areas. The UDM and the PCF may update the Service Area Restrictions of a UE at any time. For the UE in CM-CONNECTED state the AMF updates the UE and RAN immediately. For UE in CM-IDLE state the AMF may page the UE immediately or store the updated service area restriction and update the UE upon next signalling interaction with the UE.

During registration, if the Service Area Restrictions of the UE is not present in the AMF, the AMF fetches from the UDM the Service Area Restrictions of the UE that may be further adjusted by the PCF. The serving AMF shall enforce the Service Area Restrictions of a UE. A limited allowed area given by a maximum allowed number of tracking areas, may be dynamically assigned by the AMF adding, any not yet visited (by the UE) tracking areas to the Allowed area until the maximum allowed number of tracking areas is reached.

When the AMF assigns a limited allowed area to the UE, the AMF shall provide the UE with Service Area Restrictions which consist of either Allowed Areas or Non-Allowed Areas. The Allowed Areas included in the Service Area Restrictions can be pre-configured and/or dynamically assigned by AMF.

For a UE in CM-CONNECTED state the AMF shall indicate the Service Area Restrictions of this UE to the RAN, using a Mobility Restriction List.

The UE shall store the received Service Area Restrictions and, if there is previously stored Service Area Restrictions, replace them with the newly received information. If the Service Area Restrictions include a limited allowed area, the Service Area Restrictions are applicable for the Tracking areas indicated in Service Area Restrictions. If the Service Area Restrictions included an unlimited allowed area, the received Service Area Restrictions are applicable for the registered PLMN and its equivalent PLMNs. The RAN uses the Service Area Restrictions for target cell selection in Xn and N2 based handover.

Upon change of serving AMF due to mobility, the old AMF may provide the new AMF with the Service Area Restrictions of the UE that may be further adjusted by the PCF.

The network may perform paging for a UE to update Service Area Restrictions with Generic UE Configuration Update procedure (see in TS 23.502 [3] clause 4.2.4).

In the case of roaming, the Service Area Restrictions are transferred from the UDM via the serving AMF to the serving PCF in the visited network. The serving PCF in the visited network may further adjust the Service Area Restrictions.

**** Next Change ****

The below sections should go to the new TS

##### 5.3.4.1a	Mobility Restrictions

##### 5.3.4.1a.1	General

Mobility Restrictions restrict service access of an RG depending on RG location.

For an RG connecting over NG-RAN, the Mobility Restriction functionality as described in TS 23.501, clause 5.3.4.1 applies.

For an RG connecting over wireline access, the Mobility Restriction functionality is described in this clause.

Mobility Restrictions for wireline access consists of Forbidden Area and Service Area Restrictions, as described in the following subsections.

##### 5.3.4.1a.2	Management of Forbidden Area in wireline access

In a Forbidden Area, the RG, based on subscription, is not permitted to initiate any communication with the network for this PLMN.

The UDM stores the Forbidden Area for wireline access in the same way as for 3GPP access, with the difference that Line ID is used to describe the Forbidden Area.

Forbidden Area is enforced by AMF, based on subscription data and the Line ID received from W-AGF. The 5G-RG and W-AGF (for FN-RG) behaviour depends on the network response (cause code from AMF) that informs the RG that communication is forbidden.

##### 5.3.4.1a.3	Management of Service Area Restrictions in wireline access

A Service Area Restriction for wireline access may contain one or more Line IDs. The RG's subscription data in the UDM includes a Service Area Restriction which may contain either Allowed or Non-Allowed Areas–specified by using explicit Line IDs and/or other geographical information (e.g., longitude/latitude, zip code, etc). The geographical information used to specify allowed or non-allowed area is only managed in the network, and the network will map it to a list of Line IDs before sending Service Area Restriction information to the PCF.

The UDM stores the Service Area Restrictions of a RG as part of the RG's subscription data. The PCF in the serving network may (e.g. due to varying conditions such as RG's location, application in use, time and date) further adjust Service Area Restrictions of a RG, either by expanding an allowed area or by reducing a non-allowed area. The UDM and the PCF may update the Service Area Restrictions of a RG at any time.

During registration, if the Service Area Restrictions of the RG is not present in the AMF, the AMF fetches from the UDM the Service Area Restrictions of the RG that may be further adjusted by the PCF. The serving AMF shall enforce the Service Area Restrictions of a RG. The AMF receives the Line ID where the RG is connected from the W-AGF via N2.

The network does not send any Allowed Area or Non-Allowed Area to the RG or W-AGF (for FN-RG) for wireline access. In case the RG or W-AGF initiates communication in an Allowed Area, the network accepts the communication as allowed by the subscription. In case the 5G-RG or W-AGF initiates Service Request or SM signalling in a Non-Allowed Area, the AMF rejects the request with a suitable cause code indicating that the RG/W-AGF should not retry Service Request and SM signalling while being connected to the same line.

Editor’s note: BBF need to be consulted whether it is feasible for the 5G-RG to identify whether the same Line is used (e.g. after a disconnect and re-connect) or whether the 5G-RG can only notice that a Line has been disconnected and a Line has been connected but without identifying which Line.

Upon change of serving AMF due to mobility, the old AMF may provide the new AMF with the Service Area Restrictions of the RG that may be further adjusted by the PCF.

**** End of Changes ****