3GPP TSG-SA WG2 Meeting #131	S2-1901773

# Santa Cruz-Tenerife, Spain, 25 Feb – 1 March, 2019  	 (builds from S2-1900854)

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 0955 | rev |  | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Home ID for cable access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Charter Communications, CableLabs |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5WWC |  |  |  |  |  | Date: |  |  |  |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | As noted in CR S2-19008754, as agreed upon in SA2 #130, the following EN is added: It is FFS whether a specific description of the Home ID for Cable scenarios is needed. If the SUPI is based on IMSI then Home Network identifier (Home ID) is based on MCC/MNC. There is no reason to change the format of Home Network when SUPI is using IMSI. For NAI based SUPI, 23.003 has defined the corresponding format of SUCI in TS 23.003, section 28.7.3, which takes for form of username@realm as specified in subclause 2.2 of IETF RFC 7542 [126], where the realm part shall be identical to the realm part of the Network Specific Identifier. In this case, the home ID is contained in the realm part. There is no reason to change this NAI format as well. All in all, there is no specific reason to change the Home ID description as defined in TS 23.003 for Cable. It is proposed to remove that editor’s note in the final version of S2-1900854. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following revisions are proposed: - Home ID for cable access is based on TS 23.003. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of Home ID clarity for cable access. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.6.x.4 (new as agreed upon in SA2 #130)) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | It is expected this CR will be merged to the final form of CR based off the endorsed S2-1900874. The delta from S2-1900874 is highliged in green. |  |  |  |  |  |  |  |  |

******************************* start  1st changes *********************

#### 5.9.X.4 	SUPI and SUCI for FN-CRG support

The SUPI for an FN-CRG subscription shall contain either an IMSI or a HFC_Identifier, based on operator configuration. If the SUPI contains a HFC_Identifier, and since the HFC_Identifier is not globally unique, the SUPI also needs to contain an operator identifier of the operator administrating the HFC_Identifier value.

The SUCI provided to the network for FN-CRG support always contains a HFC_Identifier together with an operator identifier of the operator administrating the HFC_Identifier value. If the SUPI associated with the subscription for FN-CRG contains an IMSI, the HFC_Identifier included in the SUCI acts as pseudonym of the SUPI and UDM performs a mapping to a SUPI containing an IMSI. If the SUPI associated with the subscription for FN-CRG contains a HFC_Identifier, there is no need for a mapping in UDM.

As described in TS 23.003 [19], the SUCI also contains an identifier of the Home network for both IMSI and NAI based SUPI.

The format of the HFC_Identifier is an octet string and may may contain a cable modem MAC address or an overall HFC account identifier, as defined by CableLabs in [W].

Editor’s note: Additional details regarding SUCI, including whether or not a HFC_Identifier in SUCI is concealed, is to be deteremined by SA3. The text above about SUCI for wireline access may need to be updated based on SA3 outcome.

**** End of Changes ****