3GPP TSG-SA WG2 Meeting #133	S2-1905030

# Reno (NV), USA, 13-17 May 2019															rev of S2-190xxxx

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 1298 | rev | - | Current version: | 16.0.2 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Unified Access Control with NPN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Vertical_LAN |  |  |  |  |  | Date: |  |  | 07/05/2019 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | It is not clear whether any extensions to UAC is needed for NPN. UAC is applied per PLMN and besides the standardized access categories the operator can enable different Operator-defined access category criteria type values as defined in 24.501 which includes support for S-NSSAI. As for SNPN the NID is an extension to the PLMN ID, the UAC is applied specifically for the SNPN. For PNI NPN, the PLMN operator can enable UAC for the S-NSSAI that is used for the NPN. So, no extensions are needed, but the above can be clarified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is clarified that existing UAC and access categories can be used. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not clear whether extensions to UAC is needed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.30.2.5, 5.30.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

****** Start of changes ******

#### 5.30.2.5	Network access control

If a UE performs the registration or service request procedure in an SNPN identified by a PLMN ID and a locally managed NID and there is no subscription for the UE, then the AMF shall reject the UE with an appropriate cause code to temporarily prevent the UE from automatically selecting and registering with the same SNPN.

If a UE performs the registration or service request procedure in an SNPN identified by a PLMN ID and a universally managed NID and there is no subscription for the UE, then the AMF shall reject the UE with an appropriate cause code to permanently prevent the UE from automatically selecting and registering with the same SNPN.

NOTE 1:	The details of rejection and cause codes will be defined in TS 24.501 [47].

In order to prevent access to SNPNs for authorized UE(s) in case of network congestion/overload, Unified Access Control information is configured per non-public network (i.e. as part of the subscription information that the UE has for a given non-public network).

NOTE 2:	As the SNPN is identified by PLMN ID and NID, the UAC is enabled solely for the access to the SNPN in the cell.

****** Next changes ******

#### 5.30.3.4	Network and cell (re-)selection, and access control

The following is assumed for network and cell selection, and access control:

-	The CAG cell shall broadcast information such that only UEs supporting CAG are accessing the cell;

NOTE:	The above also implies that cells are either CAG cells or normal PLMN cells.

-	In order to prevent access to NPNs for authorized UE(s) in case of network congestion/overload, existing mechanisms defined for Control Plane load control, congestion and overload control in clause 5.19 can be used, as well as the access control and barring functionality described in clause 5.2.5. For Unified Access Control, the operator-defined access categories with access category criteria type (as defined in TS 24.501[x]) set to the S-NSSAI used for an NPN can be used.

-	For aspects of automatic and manual network selection in relation to CAG, see TS 23.122 [17];

-	For aspects related to cell (re-)selection, see TS 38.304 [50];

-	The Mobility Restrictions shall be able to restrict the UE's mobility according to the Allowed CAG list (if configured in the subscription) and include an indication whether the UE is only allowed to access CAG cells (if configured in the subscription);

-	During transition from CM-IDLE to CM-CONNECTED, if the UE is accessing the 5GS via a CAG cell, the NG-RAN shall provide the CAG Identifier to the AMF:

-	The AMF shall verify whether UE access is allowed by Mobility Restrictions:

-	If the CAG Identifier received from the NG-RAN is part of the UE's Allowed CAG list, then the AMF accepts the NAS request;

-	If the CAG Identifier received from the NG-RAN is not part of the UE's Allowed CAG list, then the AMF rejects the NAS request with an appropriate cause code, whereas the UE removes that CAG Identifier, if it exists, from its Allowed CAG list, as defined in TS 24.501 [47]. The AMF shall then release the NAS signalling connection for the UE by triggering the AN release procedure; and

-	If the UE is accessing the network via a non-CAG cell and the UE's subscription contains an indication that the UE is only allowed to access CAG cells, then the AMF rejects the NAS request with an appropriate cause code, whereas the UE updates its local configuration, as defined in TS 24.501 [47]. The AMF shall then release the NAS signalling connection for the UE by triggering the AN release procedure.

-	During connected mode mobility procedures:

-	Based on the Mobility Restrictions received from the AMF:

-	Source NG-RAN shall not handover the UE to a target NG-RAN node if the target is a CAG cell and the related CAG Identifier is not part of the UE's Allowed CAG list;

-	Source NG-RAN shall not handover the UE to a non-CAG cell if the UE is only allowed to access CAG cells;

-	Update of Mobility Restrictions:

-	When the AMF receives the Nudm_SDM_Notification from the UDM and the AMF determines that the Allowed CAG list or the indication whether the UE is only allowed to access CAG cells have changed;

-	The AMF shall update the Mobility Restrictions in the UE and NG-RAN accordingly; and

-	If the UE is currently accessing a CAG cell and the related CAG Identifier has been removed from the Allowed CAG list or if the UE is currently accessing a non-CAG cell and the indication that the UE is only allowed to access CAG cells has been set in the subscription, then the AMF shall release the NAS signalling connection for the UE by triggering the AN release procedure.

****** End of changes ******