3GPP TSG-SA WG2 Meeting #132 	S2-1905056

Reno, NV, USA, May 13 - 17

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 1310 | rev | - | Current version: | 16.0.2 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | eSBA PCF discovery and selection + editorial |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eSBA, 5GS_Ph1 |  |  |  |  |  | Date: |  |  | 2019-05-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | eSBA. |  |  |  |  |  |  |  |  |
| Summary of change: |  | eSBA additions and editorials. eSBA: - according to DP S2-…. - Factors to send to SCP for UE and AM policies in delegated discovery. - Letting the NFs (AMF and SMF) to decide if same PCF shall be used and not configurations in SCP. i.e. retaining these decissions in the NFs also for eSBA. - If AMF wants to re-use the AM and UE policy associations it already has the resource URI, and thus no special handling needed in case of SCP. This CR also takes in changes from CR 1042 (S2-1904798) and makes editorial movements of text. The changes proposed in CR 1042 is marked in green and the moved text in yellow. This CR invalidates CF 1042, since it includes both the changes from from the CR 1042 and in addition adds eSBA functionality. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No delegated discovery and selection |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.7.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | CR 1042 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

*** BEGIN CHANGES ***

#### 6.3.7.1	PCF discovery and selection for a UE or a PDU Session

PCF discovery and selection functionality is implemented in AMF, SMF and SCP, and follows the principles in clause 6.3.1. The AMF uses the PCF services for a UE and the SMF uses the PCF services for a PDU Session.

When NF service consumer performs discovery and selection, the following applies:

-	The AMF may utilize the NRF to discover the candidate PCF instance(s) for a UE. In addition, PCF information may also be locally configured in the AMF. The AMF selects a PCF instance based on the available PCF instances obtained from the NRF or locally configured information in the AMF, depending on operator's policies.

In the non roaming case, the AMF selects a PCF instance for AM policy association and selects the same PCF instance for UE policy association. In the roaming case, the AMF selects a V-PCF instance for AM policy association and selects the same V-PCF instance for UE policy association.

-	The SMF may utilize the NRF to discover the candidate PCF instance(s) for a PDU Session. In addition, PCF information may also be locally configured in the SMF. The SMF selects a PCF instance based on the available PCF instances obtained from the NRF or locally configured information in the SMF, depending on operator's policies.

The following factors may be considered at PCF discovery and selection for Access and Mobility policies and UE policies:

-	SUPI; the AMF NF consumer selects a PCF instance based on the SUPI range the UE's SUPI belongs to or based on the results of a discovery procedure with NRF using the UE's SUPI as input for PCF discovery.

-	S-NSSAI

In the case of delegated discovery and selection in SCP, the AMF shall include the available factors as discovery and selection parameters in the first request. The following also applies:

-	In case AMF decides based operator policies that H-PCF is not needed, the AMF first establishes the Access and Mobility policy association. SCP discovers and selects a PCF instance. The SCP adds the selected PCF instance in the returned response. Then AMF establishes the UE policy association by adding the received PCF ID in the discovery and selection parameters in the request. SCP selects the indicated PCF.

-	In case AMF decides based on operator policies that H-PCF is needed, the AMF first establishes the Access and Mobility policy association indicating in the discovery and selection parameters that a H-PCF is needed. SCP discovers and selects a H-PCF and a (V-)PCF. Forwards the request to selected PCF. SCP adds the selected H-PCF ID and V-PCF ID in the returned response. Then AMF establishes the UE policy association by adding H-PCF ID in the message body and the received V-PCF ID in the discovery and selection parameters to the request. SCP selects the indicated V-PCF.

In the case of delegated discovery and selection in SCP and the V-PCF received a H-PCF ID, the V-PCF establishes the UE policy association to H-PCF by adding the H-PCF ID in the discovery and selection parameters in the request.

The following factors may be considered at PCF discovery and selection for a PDU session:

a)	Local operator policies.

b)	Selected Data Network Name (DNN).

c)	S-NSSAI of the PDU Session.

d)	SUPI; the SMF NF consumer selects a PCF instance based on the SUPI range the UE's SUPI belongs to or based on the results of a discovery procedure with NRF using the UE's SUPI as input for PCF discovery.

e)	PCF selected by the AMF for the UE.

In the case of delegated discovery and selection in SCP, the SMF shall include the available factors in the first request.

Editor's note:	It still needs to be clarified which actual set of parameters the sentence above is referring to.

The selected PCF instance for the Access and Mobility and UE policies need may be the same or different as the selected PCF insance(s) for PDU session policies.

In the following scenarios, information about the PCF instance that has been selected i.e. the PCF ID may be forwarded to another NF. If the NF service consumer performs discovery and selection, this NF may use this PCF instance. In the case of delegated discovery and selection, this NF may include PCF ID in the request and the SCP may use this information to select the associated PCF instance (discovery may still be needed depending on what level of information is sent by the AMF, e.g. the address of the PCF instance may not be present):

-	During AMF relocation, the target AMF may receive Access and Mobility policy association and UE policy associations from the source AMF to enable the usage of the same PCF by the target AMF, and the target AMF may decide based on operator policy either to use these association or establish new policy associations to  a new PCF.

-	The AMF may, based on operator policies, forward the selected PCF ID to SMF instance(s) during the PDU Session Establishment procedure(s) to enable the usage of the same PCF for the AMF and the SMF instance(s). The SMF may decide based on operator policy either to use the same PCF or select a new PCF. If SMF decides to use the same PCF, and there is delegated discovery and selection. The SMF adds the PCF ID to the discovery and selection parameters in a first request. The SCP shall select the indicated PCF instance.

In the roaming case, the AMF may, based on operator policies, e.g. roaming agreement, select the H-PCF in addition to the V-PCF for a UE by performing the PCF discovery and selection as described above. The AMF sends the H-PCF ID of the selected H-PCF instance to the V-PCF during the policy association establishment procedure.

*** END CHANGES ***