SA WG2 Meeting #135 	S2-1909480

14 - 18 Oct 2019, Split, Croatia

Source:	TNO, Airbus DS SLC, SES S.A.

Title:	Update of evaluation of solutions for Key Issue #5

Document for:	Approval

Agenda Item:	8.1

Work Item / Release:	FS_5GSAT_ARCH / Rel-16

Abstract of the contribution: this paper proposes an update to the evaluation of solutions for Key Issue #5. It advocates the use of the new Solution #X which is a combination of Solution #3 and Solution #5.

# Discussion

The evaluation provided for the two original solutions (Solution #3, Solution #5) has been updated to include the new combined solution (Solution #X). Based on this evaluation it will be concluded to base normative work on Solution #X.

# Proposal

It is proposed to update clause 7.1 (Evaluation of solutions for Key Issue #5) as follows:

FIRST CHANGE

## 7.1	Evaluation of solutions for Key Issue #5 - QoS with satellite backhaul

For Key Issue #5 - "QoS with satellite backhaul" three solutions have been proposed:

-	Solution #3: "Backhaul triggered QoS adaptation";

-	Solution #5: "QoS control for satellite backhaul scenario";

-	Solution #X: "Backhaul QoS handling based on AMF and UPF information".

Solution #X is a combination of Solution #3 and Solution #5.

Solution #5 is based on the assumption that the AMF can be aware of the backhaul characteristic of the AN during the N2 setup procedure. During a subsequent PDU Session Establishment this characteristic is then signalled to the SMF and potentially to the PCF which can use this for providing appropriate QoS to the PDU Session. This solution is applicable for the deployment scenario where an AN node has a single type of connection to the 5GC.

Solution #3 is based on the assumption that one or more UPFs are involved in the setup of PDU session establishment and that a UPF is able to determine the type of QoS limitations (e.g. latency) that the connections to AN nodes and/or other UPFs are exhibiting. Solution #3 does not assume that an AN node has a single type of backhaul and hence it can be used in other situations, e.g. in case of hybrid backhaul and in case of the use of multiple UPFs for the same PDU Session. For the case applicable for Solution #5 , this solution provides the SMF for an method for providing the appropriate QoS to the PDU Session after a UPF with QoS limitation is selected during PDU Session Establishment, which requires the SMF to initiates an SM policy Association modification procedure.

All solutions require changes to SMF and PCF functionality and to the service-based interface between SMF and PCF. Solution #5 further assumes changes to AMF functionality and to the service based interface between AMF and SMF. Solution #3 further assumes changes to UPF functionality and to the N4 protocol. Solution #X may require changes to the AMF functionality and to the interface between AMF and SFM if AMF based information is available. Solution #X may require changes to the UPF functionality and to the N4 protocol if UPF based information is available. Solution #X combines the two sources of information by applying them in different phases of the PDU Session Establishment procedure.

Summarizing the above results in the following table.

Table 7.1-1: Summary

|  | Solution #3 | Solution #5 | Solution #X | Remarks |
| --- | --- | --- | --- | --- |
| Applicable before UPF selection | No | Yes | Yes |  |
| Applicable after UPF selection | Yes | No | Yes |  |
| Suitable for hybrid backhaul | Yes | No | Yes | Hybrid means a combination of terrestrial and satellite |
| Suitable for multi-UPF deployments | Yes | Yes | Yes |  |
| Changes to AMF needed | No | Yes | Maybe | AMF needs to send backhaul info to the SMF, if AMF information is available |
| Changes to SMF needed | Yes | Yes | Yes | SMF needs to be able to act upon AMF and UPF based information on backhaul QoS and if needed interact with PCF |
| Changes to PCF needed | Yes | Yes | Maybe | PCF needs to be able to receive AMF and UPF based information on backhaul QoS and act on this, possibly in combination with AF interaction. If PCF is used. |
| Changes to UPF needed | Yes | No | Maybe | UPF needs to send information on QoS Limitation to the SMF, if UPF information is available |
| Changes to N4 protocol/interface needed | Yes | No | Maybe | If UPF information is available. |
| Changes to SBI between AMF and SMF needed | No | Yes | Maybe | SBI = Service Based Interface, i.e. it refers to the SBA clients and servers that make use of the SBA architecture, e.g. AMF as SBA client and SMF as SBA server If AMF information is available. |
| Changes to SBI between SMF and PCF needed | Yes | Yes | Maybe | SBI = Service Based Interface, i.e. it refers to the SBA clients and servers that make use of the SBA architecture, e.g. SMF as SBA client and PCF as SBA server. If PCF is used. |

END OF FIRST CHANGE