3GPP TSG-SA2 Meeting #135	S2-1910211

# Split, Croatia, 14th Oct 2019 - 18th Oct 2019									(Revision of 1908817)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0009 | rev | 2 | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifications to UE mobility and Abnormal behaviour analytics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Spirent, ORANGE, Huawei, China Mobile, NEC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA |  |  |  |  |  | Date: |  |  | 2019-09-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Table 6.7.2.2-1 “UE Mobility information collected from 5GC” discusses a stationary UE that may re-select between neighbour cells due to radio coverage fluctuations leading to multiple re-registrations if the selected cells belong to different registration areas. The term “stationary UE” is not defined anywhere (e.g. IoT is not the only stationary class) and furthermore, there is no reason to limit the feature to UE’s (or UE modules) that are expected to be stationary.The frequent reselection of adjacent cells is not uniquly characteristics to stationary UEs. This phenomenon could apply to any UE, stationary and mobile ones. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR indicates that the stationary qualification of a UE is just an example. Also provided are operator configurable parameters (N, M) to define an abnormal behaviour. For example, a few ping-pong hanovers (N=3) in a day (M=24 hours) may be accetable to an opeartor as a normal behaviour. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Re-Registration pehnomena will be artificially restricted to a one class of UEs, while in reality it will happen to other UEs and the analytic function will igonre such valid cases. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7.2.2, 6.7.5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First change * * * *

6.7.2.2	Input Data

The NWDAF supporting data analytics on UE mobility shall be able to collect UE mobility information from OAM, 5GC and AFs. The detailed information collected by the NWDAF could be MDT data from OAM, network data from 5GC and/or service data from AFs:

-	UE mobility information from OAM is UE location carried in MDT data;

-	Network data related to UE mobility from 5GC is UE location information as defined in the Table 6.7.2.2-1;

Table 6.7.2.2-1: UE Mobility information collected from 5GC

| Information | Source | Description |
| --- | --- | --- |
| UE ID | AMF | SUPI |
| UE locations (1..max) | AMF | UE positions |
| >UE location |  | TA or cells that the UE enters |
| >Timestamp |  | A time stamp when the AMF detects the UE enters this location |
| Type Allocation code (TAC) | AMF | To indicate the terminal model and vendor information of the UE. The UEs with the same TAC may have similar mobility behavior. The UE whose mobility behavior is unlike other UEs with the same TAC may be an abnormal one. |
| Frequent mobility re-registration | AMF | A UE (e.g. a stationary UE) may re-select between neighbour cells due to radio coverage fluctuations. This may lead to multiple re-registrations if the cells belong to different registration areas. The number of re-registrations N within a period M may be an indication for abnormal ping-pong behaviour, where N and M are operator’s configurable parameters |

-	Service data related to UE mobility provided by AFs is defined in the Table 6.7.2.2-2;

Table 6.7.2.2-2: Service Data from AF related to UE mobility

| Information | Description |
| --- | --- |
| UE ID | Could be GPSI or external UE ID |
| Application ID | Identifying the application providing this information |
| UE trajectory (1..max) | Timestamped UE positions |
| >UE location | Geographical area that the UE enters |
| >Timestamp | A time stamp when UE enters this area |

Depending on the requested level of accuracy, data collection may be provided on samples (e.g. spatial subsets of UEs or UE group, temporal subsets of UE location information).

* * * Second change * * * *

6.7.5.3	Output Analytics

The NWDAF services as defined in the clauses 7.2 and 7.3 are invoked to notify consumer NFs. A new Analytics ID named "Abnormal behaviour" is defined.

Corresponding to the Analytics ID, the analytics result provided by the NWDAF is defined in Table 6.7.5.3-1. When the level of an exception trespasses above or below the threshold, the NWDAF shall notify the consumer of the exception with an appropriate exception ID if the exception has an identity within the list of exception IDs indicated by the consumer or matches the expected analytics type indicated by the consumer.

Table 6.7.5.3-1: Abnormal behaviour Analytics

| Information | Description |
| --- | --- |
| UE ID | Could be SUPI, Internal-Group-Identifier, external UE ID, TAC which indicates UE being subject to the risk(s) identified by the Exception ID(s) |
| Exceptions (1..max) |  |
| >Exception ID | The risk detected by NWDAF |
| >Exception Level | Measured level, compared to the threshold |
| >Exception trend | Measured trend (up/down/unknown/stable) |
| >Additional measurement | Specific information for each risk |

If PCF subscribes notifications on "Abnormal behaviour", the NWDAF shall send the PCF notifications about the risk, which may trigger the PCF to update the AM/SM policies.

The NWDAF also sends the notification directly to the AMF or SMF, if the AMF or SMF subscribes the notification, so that the AMF or SMF may, based on operator local policies defined on a per S-NSSAI or per (DNN,S-NSSAI), take actions for risk solving. The following Table 6.7.5.3-2 gives examples of AM/SM policies and corresponding actions for solving each risk.

Table 6.7.5.3-2: Examples of policies and actions for risk solving

| Exception ID and description | AM/SM policy | Actions of NFs |
| --- | --- | --- |
| Unexpected UE location | Add the area of current UE location into mobility restriction | PCF may extend the Service Area Restrictions. AMF may extend the mobility restriction |
| Unexpected long-live/large rate flows | Decrease the MBR for the related QoS flow | SMF updates the QoS rule. PCF, if dynamic PCC applies for corresponding DNN, S-NSSAI, updates PCC Rules that triggers SMF updates the QoS rule. |
| Unexpected wakeup | Apply MM back-off timer to the UE | AMF applies MM back-off timer to the UE |
| Suspicion of DDoS attack | Release the PDU session and Apply SM back-off timer | PCF may request SMF to release the PDU session. SMF may release the PDU session and applies SM back-off timer |
| Wrong destination address | Update the packet filter of the related QoS flow to block the wrong SDF | PCF updates the packet filter in the PCC Rules that triggers the SMF to update the related QoS flow and configures the UPF |
| Ping-pong UE | NWDAF notifies the AMF or AF (Service Provider) | AMF may adjust a UE (e.g. a stationary UE) registration area. |
| Too frequent Service Access/Abnormal traffic volume | NWDAF notifies AF (Service Provider) |  |

* * * End of changes * * * *