3GPP TSG-WG SA2 Meeting #136AH 	S2-2000411

# Incheon, South Korea, January 13 – 17, 2020	(revision of S2-200xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.287 | CR | 0074 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on groupcast L2 IDs for privacy |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eV2XARC |  |  |  |  |  | Date: |  |  | 2020-01-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | For groupcast mode V2X communication that requires privacy support, it is not clear how destination Layer-2 ID is to be changed over time. This topic has already raised discussion in SA3, as captured from TR 33.836: 5.4.3 Potential security requirements 5G system shall ensure that the group IDs conversion to L2 IDs are protected from linkability and traceability attacks for eV2X groupcast communications. In order to protect L2 IDs from traceability attacks as mentioned above, the destination L2 ID shall be changed over time, as a destination Layer-2 ID used by a group of UEs (e.g. vehicles), might also expose the group and group members to be tracked or identified by any other UEs. Meanwhile, if only one ID of Source Layer 2 ID and Destination Layer ID pair is changed, the other, unchanged ID might also expose the group or the member to be tracked, and the attacker might easily find the newly changed ID. Hence the source Layer-2 ID and destination Layer-2 ID shall be changed all at once. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a clause clarify the details of the Layer-2 IDs update for groupcast mode V2X communication. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Groupcast mode V2X communication cannot meet the SA3 requirements and provide privacy support. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.6.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 5.6.1.3	Identifiers for groupcast mode V2X communication over PC5 reference point

For groupcast mode of V2X communication over PC5 reference point, the V2X application layer may provide group identifier information. When the group identifier information is provided by the V2X application layer, the UE converts the provided group identifier into a destination Layer-2 ID. When the group identifier information is not provided by the V2X application layer, the UE determines the destination Layer-2 ID based on configuration of the mapping between service type (e.g. PSID/ITS-AID) and Layer-2 ID, as specified in clause 5.1.2.1.

NOTE 1:	The mechanism for converting the V2X application layer provided group identifier to the destination Layer-2 ID is defined in Stage 3.

The UE self-selects a source Layer-2 ID.

For groupcast mode V2X communication that requires privacy support in the current Geographical Area, in order to ensure that any UE of the groupcast cannot be tracked or identified by any other UEs beyond a certain short time-period required by the application, the source Layer-2 ID and destination Layer-2 ID shall be changed all at once over time.

NOTE 2:	The mechanism for updating source Layer-2 ID and destination Layer-2 ID for groupcast mode V2X communication is defined in Stage 3.

Editor's note:	Further updates of the identifiers description may be required based on RAN WG feedback.

# * * * * End of changes * * * *