SA WG2 Meeting #S2-138e	S2-2003194

20 – 24  April, 2020, e-Meeting	(revision of S2-200xxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.287 | CR | 0136 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to Link Identifier Update Procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Interdigital, LG Electronics, Samsung, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eV2XARC |  |  |  |  |  | Date: |  |  | April 9, 2020 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | According to the privacy requirements defined by SA3 in clause 5.3.2.2 of TS33.356, vulnerabilities arising from trackability and linkability attacks should be minimized. SA3 hence has described the Link Identifier Update procedure in clause 5.3.3.2 of TS33.356 which satisfies these privacy requirements. To fulfill these privacy requirements, it is mandatory that both UEs change their Layer-2 ID and the Security information (i.e. KD session-ID) at the same time by executing the Link Identifier Update procedure. Therefore, it is being clarified that the change of security information needs to happen along with the change of Layer-2 ID to respect the privacy requirements |  |  |  |  |  |  |  |  |
| Summary of change: |  | Following changes are proposed in this CR: - Both Peer UEs are mandated to change their Layer-2 ID and security information during the execution of the Link Identifier Update procedure to meet the privacy requirements. - Clarification that UE shall continue to use the old identifiers and security information during the procedure if it has data to transmit |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The current version of the procedure is vulnerable to trackability and linkability thus UEs may be tracked and traced |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

#### 6.3.3.2	Link identifier update for a unicast link

Figure 6.3.3.2-1 shows the link identifier update procedure for a unicast link. Due to the privacy requirements, identifiers used for unicast mode of V2X communication over PC5 reference point (e.g. Application Layer ID, Source Layer-2 ID and IP address/prefix) shall be changed over time as specified in clauses 5.6.1.1 and 5.6.1.4. A UE may also initiate the change of link identifier for other reasons, e.g. detected a conflict of the Layer-2 IDs. This procedure is used to update and exchange new identifiers between the source and the peer UEs for a unicast link before using the new identifiers, to prevent service interruptions.

If a UE has multiple unicast links using the same Application Layer IDs or Layer-2 IDs, the UE needs to perform the link identifier update procedure over each of the unicast link.

Figure 6.3.3.2-1: Link identifier update procedure

0.	UE-1 and UE-2 have a unicast link established as described in clause 6.3.3.1.

1.	UE-1 decides to change its identifier(s), e.g. upon change of the Application Layer ID  or upon expiry of a timer. UE-1 generates its new Layer-2 ID and sends a Link Identifier Update Request message to UE-2 using the old identifiers.

The Link Identifier Update Request message includes the new identifier(s) to use (including the new Layer-2 ID, new Security Information, optionally the new Application Layer ID, and optionally new IP address/prefix if IP communication is used). The new identifier(s) shall  be cyphered if the link requires privacy protection. After sending the Link Identifier Update Request message, if the UE1 has data to send, UE-1 keeps sending data traffic to UE-2 with the old identifiers until UE-1 sends the Link Identifier Update Ack message to UE-2.

NOTE 1:	The timer is running on per Source Layer-2 ID.

NOTE 2:	When one of the two UEs acts as IPv6 router as described in clause 5.2.1.5 and IP address/prefix also needs to be changed, corresponding address configuration procedure would be carried out after the Link Identifier update procedure.

2.	Upon reception of the Link Identifier Update Request message, if the privacy configuration as specified in clause 5.1.2.1 requires privacy protection for this link, UE-2 shall change its identifier(s). UE-2 shall respond with a Link Identifier Update Response message which includes the new identifier(s) to use (including the new Layer-2 ID, new Security Information, optionally the new Application Layer ID, and optionally a new IP address/prefix if IP communication is used). The new identifier(s) shall be cyphered if the link requires privacy protection. The Link Identifier Update Response message is sent using the old identifiers. UE-2 continues to receive traffic with the old Layer-2 ID from UE-1 until UE-2 receives traffic with the new Layer-2 ID from UE-1. After sending the Link Identifier Update Response message, UE-2 keeps sending data traffic to UE-1 with the old identifiers, if the UE-2 has data to send, until UE-2 receives the Link Identifier Update Ack message from UE-1.

3.	Upon reception of the Link Identifier Update Response message, UE-1 shall respond with a Link Identifier Update Ack message. The Link Identifier Update Ack message  includes the new identifier(s) from UE-2, as received on the Link Identifier Update Response message. The Link Identifier Update Ack message is sent using the old identifiers. UE-1 continues to receive traffic with the old Layer-2 ID from UE-2 until UE-1 receives traffic with the new Layer-2 ID from UE-2.

4.	The V2X layer of UE-1 passes the PC5 Link Identifier for the unicast link and the updated Layer-2 IDs (i.e. new Layer-2 ID for UE-1 for the source and new Layer-2 ID of UE-2 for the destination) down to the AS layer. This enables the AS layer to update the provided Layer-2 IDs for the unicast link.

UE-1 starts using its new identifiers and UE-2's new identifiers for this unicast link.

5.	Upon reception of the Link Identifier Update Ack message, the V2X layer of UE-2 passes the PC5 Link Identifier for the unicast link and the updated Layer-2 IDs (i.e. new Layer-2 ID of UE-2 for the source and new Layer-2 ID for UE-1 for the destination) down to the AS layer. This enables the AS layer to update the provided Layer-2 IDs for the unicast link.

UE-2 starts using its new identifiers and UE-1's new identifiers for this unicast link.

NOTE 3:	The Security Information in the above messages also needs to be updated at the same time as the Layer-2 IDs. This is defined in TS 33.536 [26].

* * * End of Changes * * *