SA WG2 Meeting #139E	S2-2003663

Elbonia, June 1- 12, 2020	(revision of S2-200xxxx)

Source:	Apple

Title:	KI #3, New Sol: Limitation of data rate per network slice in UL and DL per UE based on RAN enforcement

Document for:	            Approval

Agenda Item:	8.4

Work Item / Release:	FS_eNS_Ph2 / Rel-17

Abstract: This contribution proposes a solution for Key Issue #3 – Limitation of data rate per network slice in UL and DL per UE in TR 23.700-40.

Key issue #3 is about to study whether and how to limit the data rate of UE for a Network Slice, ensuring that the aggregate of the PDU sessions that use the slice are rate limited to the rate defined for the Network Slice in DL and UL.

* * * * Start of Changes * * * *

## 6.0	Mapping Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

| Solution#'s | Solution Titles | Key Issue#'s |
| --- | --- | --- |
| 1 | PCF measurement based Network Slice SLA control for Maximum Number of UEs parameter | 1 |
| 2 | Max number of UEs per Network Slice control at registration | 1 |
| 3 | AMF/NSSF based counting of UEs in a Network Slice | 1 |
| 4 | NWDAF enhancements for supporting of network slice quota on the maximum number of UEs | 1 |
| 5 | NWDAF enhancements for supporting of network slice quota on the maximum number of PDU Sessions | 2 |
| 6 | PCF-based counting of PDU Sessions in a Network Slice | 2 |
| 7 | Support of Network Slice SLA for Maximum Number of PDU sessions parameter | 2 |
| 8 | AMF and O&M based solution | 1, 2 & 4 |
| 9 | Monitoring multiple quotas of number of UEs/PDU Sessions per S-NSSAI at NWDAF | 1, 2 & 4 |
| 10 | Max number of PDU Sessions per Network Slice control via NSQ function | 2 |
| 11 | Handling maximum number of sessions using NF status | 2 |
| Y | Limitation of data rate per network slice in UL and DL per UE based on RAN enforcement | 3 |

* * * * Next Change (all new text) * * * *

## 6.Y	Solution #Y: Limitation of data rate per network slice in UL and DL per UE based on RAN enforcement

### 6.Y.1	Introduction

This solution addresses the Key Issue #3: limitation of data rate per network slice in UL and DL per UE.

It is based on the assumption that the data rate limit per network slice in UL and DL per UE is stored in UDM and can be retrieved by other NFs when needed.

### 6.Y.2	High-level Description

In this solution, the data rate limit per network slice in UL and DL per UE is enforced by RAN according to the aggregated bit rate from all the non-GBR QoS flows belonging to that network slice per UE. NG-RAN shall first reserve the cumulative GFBR needed for the GBR flows, and then initiate the enforcement for the non-GBR traffic.

### 6.Y.3	Procedures

The following changes to the existing procedure defined in TS 23.502 [6] are proposed:

Figure 6.Y.3-1: TS 23.502 Figure 4.2.2.2.2-1 Registration procedure

Step 14a-c: In addition to the existing registration procedure defined in TS 23.502 [6] Figure 4.2.2.2.2-1, AMF shall retrieve the data rate limit per network slice in UL and DL per UE from UDM in step 14 and provide it to RAN.

Figure 6.X.3-2: TS 23.502 Figure 4.3.2.2.1-1 UE-requested PDU Session Establishment for non-roaming and roaming with local breakout

In the PDU Session establishment procedure (as defined TS 23.502 [6] clause 4.3.2.2), RAN has already received QoS profiles and S-NSSAI according to the existing procedures. In this solution, RAN should further associate the received QoS profiles and the corresponding QFIs with the S-NSSAI at step 11.

RAN shall thereafter enforce the aggregated flow bit rate from all the non-GBR QoS flows associated with a particular S-NSSAI to the data rate limit per network slice in UL and DL per UE received from AMF at any particular measurement window. NG-RAN shall first reserve the cumulative GFBR needed for the GBR flows, and then initiate the enforcement for the non-GBR traffic.

### 6.Y.4	Impacts on existing entities and interfaces

AMF:	Interacts with UDM to retrieve the data rate limit per network slice in UL and DL per UE from UDM, and further provide it to RAN.

### 6.Y.5	Evaluation

Editor's note:	This clause provides an evaluation of the solution.

* * * * End of Changes * * * *