3GPP TSG-SA WG2 Meeting #139e 	S2-2003932

Electronic Meeting,  1 - 12 June 2020	(revision of S2-xxxxx)

Title:	[draft]LS on sensitive Mission Critical information

Response to:

Release:	Release 17

Work Item:	FS_5MBS

Source:	SA2

To:	SA6

Cc:	SA1

Contact Person:

#### Name:	Francois Piroard

E-mail Address:	francois.piroard@airbus.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

SA2 has discussed the use of 5GS multicast and broadcast transport solutions for Mission Critical (MC) services, as part of the FS_5MBS study.

The 5GS multicast transport solutions under consideration in TR 23.757 make the assumption that the UEs interested in receiving a specific information flow need to make an explicit request (e.g. by indicating the underlying TMGI) to either NG-RAN or 5GC, depending on the solution. RAN or 5GC triggered switching between unicast and multicast relies on an accurate count of interested UEs in each information flow, therefore SA2 understands that in the context of MC communications defined by SA6, the use of multicast transport creates a one to one association between a TMGI and an MC Service group.Thus, by expressing interest in the same TMGI the UEs participating in MC communications implicitly indicate their affiliation with the same MC service group to the NG-RAN or to 5GC. This discloses the group structure of the MC system to the 5GS, where analysis of interests in TMGIs would allow to determine which UEs are part of the same MC service group.

Several companies in SA2 expressed a concern that in certain MC scenarios the MC organization may not have a full trust relationship with the Mobile Network Operator and the exposure of UE’s interest in a specific TMGI to the NG-RAN or to 5GC would amount to exposure of sensitive information and may not be acceptable.

Question: SA2 would like to seek confirmation from SA6 about the existence of Mission Critical scenarios where UE’s explicit expression of interest in a specific TMGI to either NG-RAN or 5GC would be unacceptable.

SA2 would like to note that 5GS broadcast transport solutions with Application Server-triggered switching between unicast and multicast (like in EPS) can still be used in such scenarios.

2. Actions:

To SA6 group.

ACTION: 	SA2 kindly asks SA6 group to whether there are Mission Critical scenarios where UE’s explicit expression of interest in a specific TMGI to either NG-RAN or 5GC would be unacceptable.

3. Date of Next TSG SA WG2 Meetings:

SA2#140e	16 August – 2 September		Electronic meeting

SA2#141	12 – 16 October 2020		TBD