3GPP TSG-WG SA2 Meeting #140E e-meeting  	S2-2005055r02

# Elbonia, August 19 – September 02, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR |  | rev |  | Current version: | 16.5.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Procedures of utilizing backhaul related information during the PDU session establishment procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | TNO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_ARCH |  |  |  |  |  | Date: |  |  | 2020-08-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Usage of satellite access for backhauling might introduce additional latency compared to e.g. fiber backhauls. The existence of a satellite link in (part of) the backhaul can be identified by AMF or UPF. This information subsequently needs to be transferred to SMF and PCF in order to select/adapt the choice of QoS Profile. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adaptation of procedures/functions that account for possible usage of satellite links in backhaul that might affect QoS Profile selection/adaptation process. Besides, 5GC aware of the satellite category can determine whether a PDU session establishment/modification request can be accepted. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | In case 5GC is not aware of the existence of satellite link(s) in the backhaul might result in degradation of QoS provided to the end-user. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.x.x (new), 4.x.x.x(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** Begin of Change ***

### 4.x.x Support of integrating satellite access into 5GS

### 4.x.x.x UE Requested PDU Session Establishment in case of satellite access

This clause specifies additional steps introduced in the UE Requested PDU Session Establishment procedure (clause 4.3.2.2) specifically for the Non-roaming and Roaming with Local Breakout (clause 4.3.2.2.1) in cases satellite access is used in 5GS, in particular:

In step 1, after reception of the UE to AMF NAS message, the AMF may determine whether the UE is accessing the network via satellite backhaul as defined in TS 23.501, clause 5.4.X. The AMF shall determine the satellite category (GEO, MEO, LEO).

In step 3, if available, the AMF sends the satellite category to the SMF. The SMF may check the consistency of the DNN/S-NSSAI and the satellite category information provided by the AMF. Based on this check, the SMF may decide to reject the PDU Session Establishment request in step 5 (e.g. due to the fact that the DNN/S-NSSAI is only used for some specific low latency service).

If the SMF performs an SM Policy Association Establishment procedure in step 7b, then the SMF may send the satellite category to the PCF. The satellite category shall be indicated in the new (optional) input Backhaul Type in the Npcf_SMPolicyControl_Create operation sent to the PCF. The PCF may take the satellite category information into account when generating the PCC rules (to be provided to the SMF) and may make other policy decisions, e.g. it may also request the SMF to reject the PDU Session Establishment request.

*** End of Change ***

#### -