3GPP TSG-WG SA2 Meeting #140E e-meeting 	S2-2005415

Elbonia, August 19 – September 1, 2020	(revision of S2-200xxxx)

Title:	[DRAFT] LS on RAN impact of FS_5MBS Study

Release:	Release 17

Work Item:	FS_5MBS, NR_MBS-Core

Source:	[Huawei to be] SA2

To:	RAN2, RAN3

Cc:

Contact Person:

Name:	Meng Li

Tel. Number:

E-mail Address:	Raymond DOT limeng AT huawei DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	Draft TR 23.757 v0.4.0

1. Overall Description:

SA2 has been discussing on Rel-17 FS_5MBS solutions since SA2 #135 meeting. Based on the agreements made at the SA2 #140 e-meeting, SA2 agreed to send the updated version of the Technical Report TR 23.757 to RAN to facilitate work of RAN.

SA2 would like to kindly inform RAN2 and RAN3 the following interim agreements in SA2:

-	SA2 assumes the RAN is aware of QoS requirements for an MBS Session.

-	[Depends on the outcome of SA2] SA2 agrees the Architecture X in section A.X is used as the baseline architecture.

-	[Depends on the outcome of SA2] SA2 agrees that shared N3 tunnel will be used for DL MBS data delivery, both “multicast transport based on that NG-RAN sends IGMP/MLD Join to a multicast router or a MB-UPF” and “setting up point-to-point unidirectional N3 tunnel from MB-UPF to NG-RAN” could be used.

-	[Depends on the outcome of SA2] SA2 agrees one shared N3 tunnel is associated with only one MBS session.

SA2 would like to kindly ask RAN2 and RAN3 the following questions:

-	Some solutions assume the UE can receive MBS data in CM-IDLE state. Does RAN2 and RAN3 have any progress or assumption to enable UE receiving MBS data in CM-IDLE state?

-	Some solutions assume the UE needs to be notified in CM-IDLE state that a MBS session is started. Does RAN2 and RAN3 have any progress or assumption to use group paging or legacy session start notification?

-	Some solutions assume the UE can indicate session join and session leaving over radio in RRC-INACTIVE state. Does RAN2 and RAN3 have any progress or assumption to support this feature?

-	Some solutions assume the UE always needs to indicate session leaving, while some solutions assume the UE can silently leave a session in CM-IDLE/RRC-INACTIVE state. Does RAN2 and RAN3 have any progress or assumption on the requirement (e.g. accurate counting in RAN/CN is mandatory)?

-	Some solutions assume RAN is aware of the service area of a MBS session before data delivery of the MBS session for service area restriction. Does RAN2 and RAN3 have any progress or assumption to support this feature?

-	Some solutions provide service continuity based on MBS data forwarding at the Xn/N2 handover procedure from S-RAN to the T-RAN to address potential data loss due to asynchronous transmission between the S-RAN and the T-RAN. Does RAN2 and RAN3 have any progress or assumption to support the lossless data transmission during UE mobility, and what are the options in the consideration if any?

-	Some solutions assume RAN can trigger CN delivery method switch between shared delivery method (using shared N3 tunnel) and individual delivery method (using PDU session). Does RAN2 and RAN3 have any progress or assumption to have this feature?

2. Actions:

To RAN2 and RAN3 group.

ACTION:

SA2 respectfully requests RAN2 and RAN3 to take the above information into account and provide feedback, if necessary.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #141E		October 12 – 23, 2020			Elbonia

TSG-SA2 Meeting #142E		November 16 – 20, 2020			Elbonia