SA WG2 Meeting #140E	S2-2005845

19 August - 02 September 2020

Source:	Nokia, Nokia Shanghai Bell

Title:	KI #1: Evaluation and conclusion

Document for:	Discussion/Approval

Agenda Item:	8.5

Work Item / Release:	FS_IIoT / Rel-17

Abstract of the contribution: This contribution gives the evaluations for the solutions of KI#1 and proposes the conclusion for KI#1.

1		Discussion

This contribution proposes conclusions and way forward for Key Issue #1 for Uplink Time Synchronization.

2		Proposal

The following change is proposed for TR 23.700-20.

# * * * * First change * * * *

7.X	Key Issue #1: Uplink Time Synchronization

Solution #1 is the only solution addressing KI#1 Uplink Time Syncthonization.

# * * * * Second change * * * *

8.X	Key Issue #1: Uplink Time Synchronization

- To enable support for Uplink Time Synchronization, following are some interim principles to be considered for the way forward:

- The following interim agreement are applied for transmission of the gPTP messages (carried within Sync message for one-step operation or Follow_up message for two-step operation)

  - DS-TT which is attached by one or more TSN GMs will perform exactly the same operations for UL gPTP messages as what the NW-TT performs for the DL gPTP messages as specified in clause 5.27.1.2.2 of TS 23.501 [2].

  - The PDU session of DS-TT port receiving the source GM’ gPTP messages is used to forward the gPTP messages to the UPF/NW-TT.

    - In case of synchronizing TSN end stations behind UPF/NW-TT, the NW-TT performs exactly the same operations for UL gPTP messages as what the DS-TT performs for the DL gPTP messages as specified in clause 5.27.1.2.2 of TS 23.501 [2].

    - In case of synchronizing TSN end stations behind other UE(s), UPF then forwards the gPTP messages to the UEs via all PDU sessions terminating in this UPF except for the PDU session of the source (“avoids play back to the source DS-TT port”) . The other UE(s) perform the operation as specified in clause 5.27.1.2.2 of TS 23.501 [2].

- The UPF/NW-TT may run the BMCA to determine the port roles for the DS-TT ports connected to the same 5GS TSN bridge

Editor's note:  whether NW-TT expose the BMCA outcome to the TSN AF is FFS.

Editor's note:	How to gaurantee the 5GS delay for (g)PTP message is less than 10ms recommended by IEEE 802.1AS [6] clause B2.2 is FFS.