SA WG2 Meeting #141E 	S2-2007116

12 - 23 October 2020, Online

Source:	Ericsson

Title:		KI#12: Evaluation and Interim Conclusion

Document for:	Approval

Agenda Item:	8.1

Work Item / Release:	FS_eNA_Ph2 / Rel-17

Abstract of the contribution: Solution Evaluation and interim conclusion for 23.700-91-040 of eNA Key Issue #12

# 1 Discussion

This contribution analyses solution #4 described in the TR 23.700-91 regarding “what kind of information is made available to the NWDAF for the data analytics (e.g. the information collected for assisting RFSP index) and from which provided NFs”.

It proposes that NWDAF provides the Observed Service Experience without the knowledge of the subscriber category, and then the NF consumer decides if the Observed Service Experience is good enough depending on the subscriber category e.g. a bronze user, but not enough for e.g. a gold user, and as such a RFSP index value change is needed for the gold users.

* * * * Start of Changes * * * *

# 7	Overall evaluation

## 7.X	Evaluation of solutions for Key Issue #12 - NWDAF-assisted RFSP policy

The following questions needs to be answered: What kind of information is made available to the NWDAF for the data analytics (e.g. the information collected for assisting RFSP index) and from which provided NFs?

The data to be collected by NWDAF depends on the type of AnalyticsIds that are proposed to help determination of the RFSP index value to be assigned to a UE.

Solution #4 proposes that the RFSP index value is assigned using both the estimation of the “Service that a UE uses” and its “Observed Service Experience”:

- In order to determine the “Service in use” the NWDAF collects input data from the AF related to the service that the UE uses.

- In order to determine the “Observed Service Experience” the NWDAF collects the subscriber category from UDR or from PCF (e.g. gold user), the subscriber category may help the NWDAF to determine which the Observed Service Experience is (e.g. good, fair, excellent, poor) so that for two UEs that are located in an area where the OAM measurements in RAN provide e.g. the same level of interferences and noise, the Observed Service Experience for a gold user may not be good enough, but it may be for a bronze user.

The subscriber category should not be considered input data to NWDAF to determine the Observe Service Experience of a UE. Instead, the NF consumer, e.g. the PCF, takes the Observe Service Experience (e.g. good, far, excellent, poor) and then determines using the subscriber category if it is good enough or a change of RFSP index value is needed.

# 8.  Conclusions

## 8.X 	Key Issue #12: NWDAF-assisted RFSP policy

Solution#4 is defined in the normative work, except for the input data for Observed Service Experience that does not need any extensions compared with TS 23.288 Rel-16.

NOTE: NWDAF does not collect subscriber data as input for Observed Service Experience Analytics

* * * * End of Changes * * * *