3GPP TSG-WG SA2 Meeting #141E e-meeting 	S2-2006872

Elbonia, October 12 – 23, 2020	(revision of S2-200xxxx)

Source:	Huawei, HiSilicon

Title:	Evaluation and conclusion on KI#2 for Ethernet traffic

Document for:	Approval

Agenda Item:	8.6

Work Item / Release:	FS_ATSSS_ph2 / Rel-17

Abstract: This paper evaluates the solutions on KI#2 for Ethernet traffic and proposes to conclude on some solutions.

- Introduction/Discussion

This paper is to evaluate the KI#2 for the Ethernet traffic and conclude on the solutions. This key issue 2 aims to study on whether and how to support additional steering functionality(ies).

As the steering functionality is defined per SDF to support the steering modes, the relationship between the steering mode and the steering functionality is described in the following Table 1-1. Note that the Rel-16 ATSSS-LL functionality works together with the link performance measurement PMF, which is enhanced by the solution 3, so the ATSSS-LL with ePMF is treated as a consolidate solution to compare with solution 1 and 13. Additionally, the traffic splitting per packet modes as defined in the TR are not treated as supported by the solution 1, 13 and ATSSS-LL functionality as not relying on the application layer to perform the packet reordering.

Table 1-1: The relationship between the steering functionality and the steering mode

|  | Rel-16 Steering modes |  |  |  | Rel-17 Steering modes (proposed in the TR) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Active-Standby | Smallest-Delay | Load-balancing | Priority-based | New modes for traffic splitting per flow, e.g. Autonomous, or UE assisted mode | New modes for traffic splitting per packet, e.g. Autonomous, or RTT difference based mode | New mode for redundant transmission, i.e. Redundant steering mode |
| Solution 1 QUIC-LL | √ | √ | √ | √ | ? | X | X |
| Solution 13 proxy QUIC | √ | √ | √ | √ | ? | X | X |
| ATSSS-LL with ePMF | √ | √ | √ | √ | √ | X | X |
| Solution 6 MPQUIC-LL | √ | √ | √ | √ | √ | √ | √ |
| Solution 14 proxy MPQUIC | √ | √ | √ | √ | √ | √ | √ |
| Solution 7 MPQUIC-based | X | X | X | X | X | X | X |
| Solution 8 QUIC-based | X | X | X | X | X | X | X |

- “√” means the steering modes can be supported by the steering functionality.

- “X” means the steering mode cannot be supported by the steering functionality without relying on the application layer to perform the packet reordering.

- “?” means it is unclear on whether and how to support this steering mode. For example, it is unclear on how to support the UE assisted traffic steering mode as defined in solution 12.

It can be seen from the above Table 1-1, the new solution 1 and 13 can support the Rel-16 steering modes and some new modes proposed in the TR. All these steering modes can also be supported by the Rel-16 ATSSS-LL functionality with ePMF, so it is proposed to compare the solution 1 and 13 with the existing Rel-16 ATSSS-LL functionality plus ePMF in the following subclause 1.1.

For traffic splitting per packet, only the solution 6 MPQUIC-LL and solution 14 MP-QUIC proxy solution claim to support this mode for Ethernet traffic. It will be discussed in the following subclause 1.2.

  - Steering functionality for traffic splitting per Ethernet flow

As mentioned above, the solution 1, solution 13 and Rel-16 ATSSS-LL functionality with ePMF are evaluated together. The following criteria are applied:

- Impact on the protocol stack, see below.

- System impact: similar system impact as for IP/UDP traffic, see details in S2-2006871.

- Link performance measurement evaluation: similar evaluation as for IP/UDP traffic, see details in S2-2006871.

- Multiplex issues introduced by the new solution 1 and 13: see details in S2-20xxxx, with the difference on encapsulating/decapsulating the Ethernet packets from/to single QUIC packet.

The user plane protocol stack for the QUIC-LL Solution 1 and Proxy QUIC Solution 13 is the same and is depicted in Figure 1.

Figure 1: user plane protocol stack

The user plane protocol stack for the ATSSS-LL functionality with ePMF is depicted in Figure 2.

Figure 2: user plane protocol stack for user data packet and PMF

Observation 1: In the user plane protocol stack, both solution 1 and 13 add three additional layers, i.e. QUIC/IP/UDP, between the UE and UPF. For the ATSSS-LL functionality with ePMF, there is no IP/UDP layer, since the PMF protocol as defined in Rel-16 is directly over the Ethernet layer instead of IP/UDP layer.

In addition, the Ethernet PDU session may be applied by the terminals which cannot support the IP protocol. Therefore, it is unreasonable to enforce such terminals to support the IP/UDP layer only for ATSSS. And obviously, the handling of QUIC/IP/UDP layer will reduce Ethernet packet processing efficiency.

  - Steering functionality for traffic splitting per packet

So far, only the MPQUIC-LL solution 6 and MP-QUIC proxy solution 14 claim to support traffic splitting per packet for Ethernet traffic without relying on the application layer. The same problem (i.e. introducing additional MPQUIC/IP/UDP layer in the user plane protocol stack for both UE and UPF) exists for these solutions, as described above for solution 1 and 13.

Observation 2: Same as Observation 1.

2. Text Proposal

It is proposed to capture the following changes in the TR 23.700-93.

# * * * * First change * * * *

7	Evaluation

Editor's note:	This clause will provide a general evaluation of the solutions.

7.1  Evaluation on KI#2

The steering functionality is proposed to support the steering modes for the IP/UDP and Ethernet traffic.

7.1.y  Steering functionality for Ethernet flow

Apart from the similar evaluation on the transport resource consumption and support for link performance measurements and so on, same as solution 1 QUIC-LL, solution 13 proxy QUIC also introduces an additional QUIC/IP/UDP layer in the user plane protocol stack, for both UE and UPF. For the Rel-16 ATSSS-LL functionality with ePMF solution, there is no such QUIC/IP/UDP protocol impact.

For the other solutions, i.e. the MPQUIC-LL solution 6 and MP-QUIC proxy solution 14, both claim to support traffic splitting per packet for Ethernet traffic without relying on the application layer. The same problem (i.e. introducing additional MPQUIC/IP/UDP layer in the user plane protocol stack for both UE and UPF) exists for these solutions.

In the case the UE applying Ethernet PDU session does not support the IP protocol, all the QUIC/MPQUIC based solutions do not work.

# * * * * End of changes * * * *