SA WG2 Meeting #141e	S2-2006937r01

# October 12th – 23rd, 2020 ; Elbonia                   	(revision of S2-2006937)

Source: 	Nokia, Nokia Shanghai Bell

Title: 	KI 1: Update to Solution #13

Document for: 	Agreement (P-CR)

Agenda Item: 	8.3

Work Item / Release:	FS_enh_EC / Rel-17

Abstract of the contribution: KI 1: Update to Solution #13

# 1 Discussion

Update of the solution:

- .

# 2 Proposal

It is proposed to update TR 23.748 as follows

FIRST CHANGE

## 6.13	Solution #13: 5GC support for UE selection of the DNS to use

### 6.13.1	Description

#### 6.13.1.1	Overview

The operator may negotiate with a Third party (typically a Corporate represented by an AF) dedicated DNN(s) and/or S-NSSAI(s) for the traffic of UE(s) of this third party. UE(s) of the third party may be identified by a group identifier.

This solution addresses policies that the third party would want to get enforced for the traffic of a specific UE or a group of UE(s) of this third party including for cases where traffic of UE(s) of this third party is subject to traffic offload.

The applications (FQDN(s)) reached by UE(s) of this third party may correspond to:

- corporate applications only reachable via a specific (DNN, S-NSSAI) negotiated with the operator ; corresponding URSP rules (URSP rules referring to domains of these corporate applications) shall only point to this specific (DNN, S-NSSAI)

- corporate applications reachable via a (DNN, S-NSSAI) but only in some location (DNAI) ; e.g. the corporate applications are only accessible when the UE is in some location corresponding to the corporate premises,

- internet applications not reachable via a specific (DNN, S-NSSAI) negotiated with the operator but that should be only reachable via a general purpose (DNN, S-NSSAI)  ; e.g. traffic of UE(s) of this third party targeting Internet applications is not to be sent to a specific (DNN, S-NSSAI) negotiated with the operator as this traffic is not expected to cross the Intranet of the corporate.

- internet applications reachable via both a specific (DNN, S-NSSAI) negotiated with the operator and via a general purpose (DNN, S-NSSAI) for which the third party may want to set preferences between these 2 kinds of connectivity. These preferences may depend on the UE location,

- etc…

The cases a), b), … above may correspond to different corporate that have different policies.

In the case of the usage of a specific (DNN, S-NSSAI) negotiated with the operator, the third party may wish to control the DNS resolver used by the UE (the IP address of the DNS server sent to the UE and the corresponding security related information)

Some configuration of URSP upon AF request is already (in Rel-16) possible for 5G VN group data but here the data configuration is not meant for 5G VN group members but to corporate users or users of a specific application that are not meant to only communicate with each other.

The solution relates to KI 1 and addresses how an AF can provide the 5GC with information about the relative precedence of data (PDU) Sessions (e.g. relative precedence of (DNN, S-NSSAI)) to use to reach different domains (sets of FQDN) possibly depending on the DNAI where traffic of this PDU Session may be offloaded

The solution relies on:

-	5GC can get from AF information on the domains (set of FQDN(s)) supported on a DN / local access to a DN (DNAI); these domains may be associated with a set of DN priority information. This information may be translated by NEF and is stored in UDR for further possible PCF consumption. This is further described in clause 6.13.2.1; The DN priority information indicates the relative priority (DN priority value) of one or any (DNN, S-NSSAI) for the domains (set of FQDN(s)) indicated by the AF.

The AF can associate the same set of FQDN(s) with different DN priority values for different (DNN, S-NSSAI)

NOTE 1: 	this is to cover case d) above where the AF of a corporate would try to configure the following for the UE(s) of this corporate: internet applications are reachable with lower priority via a specific corporate (DNN, S-NSSAI) negotiated with the operator and are reachable with Higher priority via a general purpose (DNN, S-NSSAI) (generic Internet access of the operator),

Information on the domains (set of FQDN(s)) supported on a DN / local access to a DN (DNAI) is later on called "DN priorities for appDomains". It consists of a list of rules that each associates a FQDN filter with DN priority information:

-	the AF identifier and an AF Transaction ID

NOTE: 	The AF Transaction ID allows the AF to manage (update / delete) the records it has created. The PCF can use the AF identifier to manage potentially conflicting rules

-	A FQDN filter corresponds to a (possibly set of) FQDN (with possible wildcarding such as such as "*.example.com") and is associated with a filtering /rule priority.

-	one or more set of DN priority information that each may correspond to:

-	a (DNN, S-NSSAI). This may be provided by the AF or determined by the NEF based on the AF identity when it is not provided by the AF and the AF provides only one instance of DN priority information.

-	a default DN priority value to be used for the FQDN(s) identified by the FQDN filter when DN priority for DNAI is not provided or does not apply.

-	for the “DNS alternative” in clause 6.13.1.3: a default DNS Server configuration information; this information is optional and is associated with the default DN priority above. . This.

-	a list of DN priority for DNAI that each associate a DN priority value and a DNAI for the FQDN(s) identified by the FQDN filter / rule when the DNAI applies to the PDU Session.

-	for the “DNS alternative” in clause 6.13.1.3: an optional list of DNS Server configuration information each associated with a DNAI and a DN priority for DNAI described above.

The DNS Server configuration information corresponds to the DNS Server related information sent to the UE. It may include the address of the DNS server as well as DNS server security information as specified in TS 24.501 and TS 33.501. This information may apply only for specific (DNN, S-NSSAI) negotiated with the operator

NOTE 2: 	the DNAIs that apply to a session may depend among other on the DNN and S-NSSAI, the user location and network topology.

-	An optional spatial Validity condition that indicate where the rule is to apply. This may correspond to a geographical area (i.e. geographical zone identifier) or a (set of) DNAI

NOTE 3:	For example, for a DN related with corporate access, the FQDN(s) corresponding to corporate services can be associated with a higher DN priority value than FQDN(s) for internet-based services (when the corporate also has an access to the Internet thus allowing also access to internet based services). The AF(s) corresponding to different corporate or different services can’t co-ordinate the DN priority values between themselves. The co-ordination between DN priority values set by different AF is done by the PCF as explained in clause 6.13.2.1

This "DN priorities for appDomains" information may be used by 5GC at 2 levels:

1)	URSP alternative: to set the Rule Precedence of URSP rules whose Traffic descriptor / Domain descriptor corresponds to such domains (sets of FQDN). The solution uses the existing Rel-16 URSP rules without change for UE perspective. Only change is how the PCF derive the URSP rules based on "DN priorities for AppDomains". URSP(s) delivered to UE(s) follow Rel-16 specifications; UE use URSP(s) for matching a PDU Session, as defined in Rel-16.

2)	DNS alternative: DNS configuration on the UE i.e. which DNS server a multi Homed UE will consider for a DNS look-up.;

The solution addresses following Connectivity Models:

-	Multiple PDU sessions including some more local PDU Sessions and PDU Sessions to corporate DN: the information on the domains (set of FQDN(s)) supported on a DN may then be received from a third party DN and has to be mapped with operator policies.

-	Session Breakout.

#### 6.13.1.2	URSP Alternative

The alternative is defined to allow the AF to influence PCF decisions for URSP rules for one UE, a group of UEs, etc… as described in Solution #1

On the UE interface this alternative reuses Rel-16 interface (URSP configured by the PCF) without change for UE perspective (or with changes as specified in solution 1 about UE taking into account URSP when resolving a FQDN via DNS). The difference with Rel-16 lies on the fact that the AF API can provide guidance on URSP building that:

- 	is not related with 5G VN group,

-	allows the AF to provide different rules for the same domains (sets of FQDN(s)) with different DN priority values for different (DNN, S-NSSAI): typically a DN priority value for the (DNN, S-NSSAI) dedicated to the corporate that the AF represents and another DN priority value for any other (DNN, S-NSSAI)

As described in 6.13.1.1, AF may provide DN priority information specific to a DNAI. If so, PCF determines the DNAIs applicable for the PDU session to build the URSPs. Any Spatial Validity conditions in terms of a (set of) DNAIs provided by AF are adapted to fit the Location Criteria in the RSD part supported in Rel-16 URSP rules.

An example of How the DN priority per Appdomain can influence the URSP rule generation in PCF is the following

A corporate wishes for the UE its employees (members of a group for corporate users) that

- They use a specific DNN “My-corporate” when the UE try reach *.corporate.com.

- They preferably use another DNN for other Internet applications: they can use the specific DNN “My-corporate” but with lower priority

- With the exception that when traffic offload to DNAI xx takes place, the members of a group for corporate users should now use specific DNN “My-corporate” for all applications (this is because in this case a direct and cheaper connectivity to the Internet via the corporate Intranet is used)

The corporate via a NEF API associates following DN priorities for appDomains rules with the Group of corporate users

-	domain *.corporate.com is associated with DNN “My-corporate” with high default priority,

-	other domains (filter with lower rule priority) is associated with DNN “My-corporate” with low default priority, and high priority for DNAI xx

The PCF acquires Application Data from the UDR as described clause 6.13.2.1. The PCF uses the Application Data to compose the URSP rules for the UE as follows:

- 	appDomains is used to set the Destination FQDNs or a regular expression in the Domain descriptor in the Traffic descriptor in the URSP rule;

- 	DN priority value is used to set the Rule precedence value of the URSP rule;

- 	DNN and S-NSSAI are used to set the DNN selection and Network Slice selection Route selection components in the Route Selection Descriptor of the URSP rule, respectively;

- 	DNAI or geographical zone identifier is mapped to the list of Cell IDs, RAN node IDs or TAI list in the Location Criteria in the Route Selection Descriptor of the URSP rule.

The PCF stores the composed URSP rules of the UE to the UDR as part of UE's Policy Set entry and updates the UE as described in clause 6.1.2.4 in TS 23.503 [4].

#### 6.13.1.3	DNS Alternative

A UE that has multiple PDU Sessions will receive multiple DNS server address and needs to know which of these DNS servers it needs to use to translate a target FQDN.

NOTE 1:	RFC 6731 [26] specifies how a DHCP server can express preferences and domains for DNS server information it provides to DHCP clients. However, RFC 6731 [26] does not specify how the DHCP server acquires specific knowledge of domains and networks. An administrator may choose to utilize the different preference values, for instance by manual configuration (RFC 6731 [26] clause 3). And 3GPP has not defined whether and how to use RFC 6731 [26] (for example considering Session Breakout and Multiple PDU sessions defined in clause 4.2 of this TR).

NOTE 2:	In the case of multiple PDU Sessions for a UE, information on (different) DNS servers that the UE can use are advertised on each of these different PDU Sessions. This currently may use NAS (PCO), RA (Router Advertisement messages), or DHCP to transfer this information to the UE but 3GPP currently does not define how to guide the UE to select the right DNS server (considering e.g. relative priorities). Clause 7.1.4 further clarifies the procedure and includes recommendations that have been agreed for normative phase (as per clause 9.1)

At PDU Session establishment, the PCF transforms this DN priorities for appDomains information into PDU Session related Policy used by SMF to send DNS configuration information to the UE

The DNS configuration received from the SMF does not provide the trigger to establish PDU Sessions but it provides the DNS priorities after/during the PDU session establishment.

The UE uses the DNS information received from the SMF (NAS, DHCP, etc…) to configure its IP stack.

NOTE 3: whether the EAS information associated with the old DNS configuration needs be cleaned is to be solved as part of KI 2

The DNS Alternative is specifically meant for a UE that has access to one DNN (one single PDU Session) but is multi-homed (per IPV6 multihoming) or because an UL CL has been inserted to the PDU Session.

### 6.13.2	Procedures

#### 6.13.2.1	Configuring DN priorities for appDomain information on PCF

Figure 6.13.2.1-1: Configuring DN priorities for appDomains information on PCF

- An Application Function (AF) invokes NEF API (e.g. Nnef_ServiceParameter API) to communicate DN priorities for appDomains (as defined in clause 6.13.1) to Network Exposure Function (NEF). This information may be associated ("target") with an individual UE or with a group of UE (for example all UE(s) of a corporate)  or with any UE.

2.	The NEF may translate the AF identity into a target (DNN, S-NSSAI). The NEF stores the information (after possible translation by NEF) including in the UDR (corresponding data set and subset are indicated in step 5).

3.	The UDR responds to NEF.

4.	The NEF responds to the AF.

5.	The PCF may acquire (GET or SUBSCRIBE to notifications on change of) Application Data from UDR including the DN priorities for appDomains information.

The PCF(s) that have subscribed to notification on modifications of AF requests (Data Set = Application Data; Data Subset = DN priorities for appDomains,) receive(s) a Nudr_DM_Notify notification of data change from the UDR. Internal Group Identifier or SUPI may be used as Data Key to cover the case where the AF requests "targets" an individual UE or a group of UE.

6.	the PCF applies this information to determine policies to build URSP or policies related with DNS priority selection in the UE.

The PCF may receive different policies from different AF (e.g. receive policies from a third party AF corresponding a corporate group of users and also apply policies applying to any UE received from different AF(s) managing Edge computing application deployments). The PCF needs to build an unique map of relative priorities between (DNN, S-NSSAI) for a given domain (range of FQDN) (to be sent to the UE as URSP, see § 6.13.2.2 or as information sent to the SMF for a PDU Session see § 6.13.2.3). For this purpose, the PCF considers local policies to resolve the potential conflicts, policies that take into account the identity of the AF that has provided a DN priorities for appDomains policy.

#### 6.13.2.2	Usage of DN priorities for appDomains information to configure URSP

1.	The UE Registers

2.	The AMF selects a PCF and establishes an AM Policy Association with the PCF by invoking Npcf_AMPolicyControl_Create operation (providing the IMSI Group of the UE.

All steps above are as for TS 23.502 [3] clause 4.2.2.2.2 (with no intended changes)

3.	If the PCF does not have relevant policy data (e;g; policy data related with for the Group the UE is belonging to), it sends a corresponding request to the UDR by invoking Nudr_DataRepository_Query operation. The PCF may also request notifications from the UDR about changes on the corresponding policy information (including DN priorities for appDomains for an IMSI group by invoking Nudr_DataRepository_Subscribe.

4.	The UDR responds to the PCF with Nudr_DataRepository_Query response including DN priorities for appDomains that includes the list specific domains (and networks).

The DN priorities for appDomains information configured on PCF by mechanisms described in the § 6.13.2.2 and recalled above may be used to generate USRP sent to the UE; URSPs sent to UE are Rel-16 URSPs.

#### 6.13.2.3	SMF influence on UE preferences between multiple Recursive DNS Servers

This clause applies to the DNS Alternative.

Figure 6.13.2.3-1 illustrates the SMF influence on UE preferences between multiple Recursive DNS Servers and the delivery of DNS Configuration information to UE as part of PDU Session Establishment procedure (defined in TS 23.502 [3] clause 4.3.2).

Figure 6.13.2.3-1:  SMF influence on preferences and domains and networks for multiple Recursive DNS Servers.

1.	The UE initiates a UE Requested PDU Session Establishment procedure to a (DNN, S-NSSAI).

2.	The AMF selects an SMF and sends Nsmf_PDUSession_CreateSMContext Request to SMF.

3.	SMF selects a PCF.

4.	SMF establishes a SM Policy Association with the PCF by invoking Npcf_SMPolicyControl_Create operation providing the (DNN, S-NSSAI) of the PDU Session.

All steps above are as for TS 23.502 [3] clause 4.3.2.2.1 (with no intended changes)

5.	If the PCF does not have policy data for the (DNN, S-NSSAI) of the PDU Session, it sends a corresponding request to the UDR by invoking Nudr_DataRepository_Query operation. The PCF may request notifications from the UDR about changes on the corresponding policy information (including DN priorities for appDomains mapping to DNN and S-NSSAI) by invoking Nudr_DataRepository_Subscribe.

6.	The UDR responds to the PCF with Nudr_DataRepository_Query response including DN priorities for appDomains that includes the list specific domains (and networks).

Steps 5 and 6 may be omitted if the PCF had subscribed to notification of this data change as defined in step 4 of TS 23.502 [3] Figure 4.3.6.2-1or in step 5 of Figure 6.13.2.1-1.

PCF decides on DNS preferences for (DNN, S-NSSAI) using the DN priorities for appDomains information received from UDR but also local policies (see also step 7 and § 6.13.2.1). The output of this decision is called DNS Configuration information. The PCF can then use the DNS Configuration information to provide it to SMF

7.	The PCF responds to the SMF with Npcf_SMPolicyControl_Create response including DNS Configuration information.

The PCF is responsible to transform "DN priorities for appDomains" information into "DNS configuration information" to be provided to SMF as part of PCC rules. “DNS configuration information” has the same format than DN priorities for appDomains" but

-	with DN priority values that have been normalized based on PCF

locally configured policies related with (DNN, S-NSSAI)

-	with only information suitable for (DNN, S-NSSAI) of the PDU Session.

AF may provide DN priority information specific to a DNAI. If so, PCF determines the DNAIs applicable for the PDU session to build the DNS Configuration. PCF also takes into account any Spatial Validity conditions in terms of a (set of) DNAIs provided by AF

Editor's note:	It is FFS if and how roaming can be supported, e.g. how to support URSP in LBO roaming, or how to support AF influence for traffic routing in HR roaming.

8.	SMF communicates the DNS Configuration information in PDU Session Establishment Response by:

-	including DNS Configuration information in Extended PCO IE (8.a). in this case it may include DNS server security information as specified in TS 23.501 § 5.6.10.1; or

-	when acting as DHCP server by including in DHCP RDNSS option when communicating with UE using DHCP (8.b); or

-	in RA (per RFC 8106).

If PCC does not apply to a PDU Session, the SMF determines locally the DNS Configuration information

The SMF may take into account DNS configuration information into account when selecting a LDNSR for a PDU Session.

If the PCF determines that DNS Configuration information has changed (because e.g. DN priorities for appDomains information has changed) it can provide a new DNS Configuration information as part of a Npcf_SMPolicyControl_Update. The SMF may provide the corresponding information to the UE:

-	Via NAS PCO sent in a network initiated PDU Session modification procedure; or

-	Via Router Advertisement ('RA' per IETF RFC 8106) (in IPv6 case);

-	Via DHCP FORCERENEW and DHCP INFORM defined in DHCP reconfigure extension (RFC 3203) that are subject to authentication of DHCP message as defined in RFC 3118 forcing the UE to get again the DNS configuration via DHCP. This alternative may not be supported by the UE in which case the SMF may be unable to transfer an update of DNS Configuration information unless it executes a SSC mode 3 procedure.

### 6.13.3	Impacts on services, entities and interfaces

-	NEF: new API information to support and to convert into policy data stored in UDR. 

The NEF service Nnef_ServiceParameter is enhanced to allow the AF to influence PCF decisions for URSP rules. In addition to the enhancements described in Solution #1, the service is enhanced in this solution to allow the AF to provide a DN priority for the associated appDomains (i.e. list of FQDNs), and to allow the AF to provide a DNAI in association with the appDomains. The AF shall provide also the AF Service ID in the AF request which shall be stored to the UDR by the NEF along the AF request information in order to distinguish multiple AFs providing rules for the same DNN, S-NSSAI combination.

-	UDR: new Application Data format (DN priorities for appDomains) to manage.

-	PCF: retrieve new UDR Application Data and,

-     either (DNS alternative) (SM PCF) determines DNS Configuration information to be provided to SMF via PDU Session level policy data.

-    or (URSP alternative) (AM PCF) uses it as guidance to build the URSPs to send to the UE. URSPs sent to UE are Rel-16 URSPs

-	SMF: (DNS alternative) send DNS Configuration information via PCO / DHCP / Router Advertisement to the UE.

-	UE: (DNS alternative) new DNS Configuration information for the PDU Session.

The content of the DNS configuration that is provided to the UE has the same format as the DHCP RDNSS option (when sent in PCO or via the DHCP RDNSS option or as defined in RFC 8106 in IPv6 case) ..

NEXT CHANGE (3)