SA WG2 Meeting #141E e-meeting	S2-2007025r03

Elbonia, October 12 – 23, 2020	(revision of S2-200xxxx)

Source:	China Mobile

Title:	KI#2 -Update Sol#57 by partially merging with Sol# 26 and 59 due to UE mobility

Document for:	Approval

Agenda Item:	8.1

Work Item / Release:	FS_eNA_Ph2 / Rel-17

Abstract: This contribution proposes to update solution 57 partially merging with 26 and 59 for Key Issue 2. In this merged solution, it is proposed to define a new solution for re-selection of serving NWDAF due to UE mobility change without consumer NF change.

1. Discussion

There are 4 solutions to address the NWDAF re-selection, one scenario of the KI#2 - Multiple NWDAF instances:

- Solution 10: Handling of mixed and distributed NWDAF deployments

- Solution 26: Reselection of NWDAF

- Solution 57: Reselection of NWDAF due to mobility change

- Solution 59: Analytics handover

Sol#57, 59, 26 also propose the NWDAF to derive the re-selection of NWDAF. All 3 solutions suggest one key step that NWDAF triggers the re-selection for target NWDAF, Sol#57 also suggests the key step that NWDAF subscribes the UE mobility event from AMF, Sol#59 supplements the procedures that NWDAF derives handover based on the UE mobility prediction.

Proposal:

This contribution proposes to update solution 57 partially merging with 26 and 59 due to UE mobility without consumer NF change.

2. Text Proposal

It is proposed the following changes to TR 23.700-91 [2].

# * * * * First change * * * *

6.57	Solution #57(partially merging Sol# 59, 26): Re-selection of NWDAF due to mobility change

6.57.1	Description

This solution is for Key Issue #2: Multiple NWDAF instances.

The consumer NF should select an appropriate NWDAF among multiple NWDAF instances to make subscriptions for data analytic. There will be some cases that serving NWDAF of one consumer NF may need to change due to UE mobility event or the prediction of UE mobility change without consumer NF change. Upon the change of UE location, AMF is required to inform the serving NWDAF, and the serving NWDAF decides whether to continue analysis service or start NWDAF re-selection. It is also required to inform the related consumer NF of the NWDAF re-selection event and also to update subscriptions between the NWDAF and the consumer NF. The analytic data and stored input data can be transferred from the previous serving NWDAF instance to new NWDAF, if applicable, to facilitate the new NWDAF to produce the analytics.

6.57.2	Procedures

6.57.2.1	Re-selection of NWDAF without AMF change

Figure 6.57.2.1-1: Procedure for re-selection of NWDAF without AMF change

1.	Consumer NF sends an analytics subscription to the NWDAF1 by invoking an Nnwdaf_AnalyticsSubscription_Subscribe service; the NWDAF1 subscribes to fetch the network data or management data from the NF(s) or the OAM and the NF(s) or the OAM deliver the network data or management data, as defined in 3GPP TS 23.288 [5].

2.	The NWDAF1 subscribes the UE mobility event notification from AMF by invoking an Namf_EventExposure_Subscribe service.

3.	UE mobility event is detected by AMF and the AMF notifies the UE mobility event to the NWDAF1 by invoking the Namf_EventExposure_Notify service if the TAI of the UE changes. NWDAF1 determines if the UE location is in the serving area of NWDAF1. If yes, the NWDAF1 could continue the analytics service, otherwise, the NWDAF1 triggers the re-selection of NWDAF.

4.	The NWDAF1 interacts with NRF by sending Nnrf_NFDiscovery_Request to discover an appropriate NWDAF2 for the UE.

5.	The NWDAF1 invokes the Nnwdaf_AnalyticsRelocation_Request or enhanced Nnwdaf_AnalyticsSubscription_Subscribe_Request service to the NWDAF2 to trigger relocation. The service includes subscription information, consisting of Subscription ID, Analytics ID, Target of Reporting and Event Reporting Information. The NWDAF2 responds to the NWDAF1 with a Nnwdaf_AnalyticsRelocation_Response or Nnwdaf_AnalyticsSubscription_Subscribe_Responce service operation which includes information on the subscription selected by the NWDAF2. The analytic data and stored input data can be transferred from the previous serving NWDAF instance to new NWDAF, if applicable, to facilitate the new NWDAF to produce the analytics.

6.	The NWDAF1 sends Nnf_EventExposure_Unsubscribe to the NF(s) or the OAM to unsubscribe the fetching the network data.

7.	The NWDAF2 invokes the Nnwdaf_AnalyticsSubscription_Notify service to the consumer NF to notify the release of the subscriptions of NWDAF1 and take over the analytics subscription for the consumer NF.

8.	The NWDAF2 invokes the Nnf_EventExposure_Subscribe service to the source NF(s) or OAM to subscribe the data in the source NF(s) or OAM.

6.57.2.2	Re-selection of NWDAF with AMF change

Figure 6.57.2.2-1: Procedure for re-selection of NWDAF with AMF change

1~2.	Steps 1~2 are executed as defined in clause 6.57.7.1.

3.	The AMF2 invokes the Namf_Communication_UEContextTransfer service operation to the AMF1 to fetch the UE context which includes the subscription of the UE mobility event from AMF1.

4~9.	Steps 3~8 are executed as defined in clause 6.57.2.1.

6.57.2.3 Prepared analytics handover procedure

The NWDAF re-selection could be prepared if source NWDAF anticipates that it will soon not be able to continue the analytics tasks that it has been requested for previously, source NWDAF could prepare the handover procedure to possible target NWDAFs based on the prediction of UE mobility, the possible target NWDAFs may start preparing data collection and analytics generation phase based on information received from source NWDAF, once the UE is confirmed to move to certain area, the source NWDAF could start the analytics handover.

For details, please refer to Sol #59 clause 6.59.2.2.

6.57.3	Impacts on Existing Nodes and Functionality

NWDAF:

- Supports a subscription transfer procedure.

- Transfers and receives stored analytics and input data.

Consumer NF:

- Supports subscription renewal to new NWDAF.

# * * * * End of changes * * * *