3GPP TSG-WG SA2 Meeting #141E e-meeting 	S2-2007048r03

Elbonia, October 12 – 23, 2020	(revision of S2-200xxxx)

Source:	Huawei, HiSilicon

Title:	KI#4, evaluations and conclusions update

Document for:	Approval

Agenda Item:	8.2

Work Item / Release:	FS_eNPN / Rel-17

Abstract: This contribution introduces evaluations and conclusions update on boarding and provisioning for SNPN subscription for KI#4

# 1. Introduction/Discussion

This contribution aims to provide the evaluations and conclusions for the Solution #5, #6, #27, #29, #30, #31, #33, #34, #35, #36, #37, #39 and #40. These solutions address the key issue #4, especially for provisioning of NPN credentials (i.e. for primary authentication) and other information to enable SNPN access. The following table provides an overall analysis and comparison for these solutions in the following dimensions:

- UP based solution or CP based solution or support both

- Primary authentication and authorization: whether this is specified and if yes, which entity is responsible for the primary AA

- Onboarding registration: whether the functionality is specified to restrict only for onboarding and remote provisioning

- Configuration PDU session: whether the PDU session is specified to provide restricted connectivity to the PS

- O-SNPN discovery: whether discovery and selection of the onboarding SNPN is specified

- subscription provisioning at UE: whether this is specified

- subscription provisioning at SNPN: whether subscription at UDM of the SO-SNPN is specified

| Solution | UP or CP based or both | primary AA | Onboarding registration | configuration PDU Session | O-SNPN discovery | subscription provisioning at UE | subscription provisioning at SNPN | remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sol#5: UE Onboarding and provisioning for an SNPN | UP-based solution | By DCS | Support | Support | Support | Support | Not specified | Address PS discovery |
| Sol#6: Control Plane-Based UE Onboarding and Provisioning Solution | CP-based solution | Not specified | Support | NA | Not specified | Support | Support |  |
| Sol#27: Common UP/CP onboarding solution for SNPN | both | By DCS/PS | Support | Support | Support | Support | Support | UP or CP decision |
| Sol#29: Discovery of the Onboarding Network | Not specified | Not specified | Not specified | Not specified | Support | Not specified | Not specified |  |
| Sol#30: UE Onboarding via Control Plane | CP-based solution | By DCS | Support | NA | Not specified | Support | Support | PS needs subscribe to UE events at O-SNPN |
| Sol#31: UE onboarding and provisioning for SNPN subscription | UP-based solution. | By DCS or PLMN | Support | Support | Support | Support | Not specified | the non-3GPP Identities/credentials are provisioned |
| Sol#33: UE Onboarding and remote provisioning based on eSIM | UP-based solution | By PLMN | Support | Support | NA | Support | Not specified | UICC is used at UE |
| Sol#34: PLMN assisted On-boarding Procedures | both | By PLMN | NA | NA | NA | Support | NA | SNPN subscription is updated at the PLMN UDM, this is trigged by UE location event |
| Sol#35: Solution for On Boarding for SNPN Compatible with SO's Existing Provisioning Server | both | By SO-SNPN | Support | Support | Support | Support | Support | SO-SNPN can also support the DCS |
| Sol#36: Providing provisioning details to UEs using local and central provisioning server | Not specified | Not specified | Not specified | Not specified | Not specified | Not specified | Not specified | Architecture for PS (Local PS, Central PS) |
| Sol#37: UE Onboarding and remote provisioning for SNPN | Not specified | Not specified | Not specified | Not specified | Not specified | Not specified | Not specified | Architecture for OPF (merging PS and DCS) (vOPF, default OPF, target OPF) |
| Sol#39: UE onboarding using PLMNs | UP-based solution | By PLMN | Support | Support | NA | Support | Support |  |
| Sol#40: Solution for UE onboarding and remote provisioning | UP-based solution | Not specified | Not specified | Support | Support | Support | Support |  |

When a SNPN supports as Onboarding Network (ON), all related solutions propose to support the functionality to restrict usage to only on-boarding service, among these solutions:

-	Sol#5 proposes that O-SNPN supports onboarding functionalities, the UE registers to O-SNPN for onboarding indicating that the registration is regardless of UE subscription, the UE provides an RRC indication that can be used by the NG-RAN to select an AMF for onboarding and an indication in the Registration Request indicating that the registration is for restricted onboarding service only. Based on the local configuration policy in the 5GC, the network can impose maximum time duration for the Configuration PDU session or Onboarding Registration, upon expiry of which the session is released or the de-registration is triggered.

-	Sol#27 proposes the similar mechanism as solution #5, with more description on onboarding SUPI.

-	Sol#30 proposes the similar mechanism as solution #5, #27.

-	Sol#31 proposes the similar mechanism as solution #5, #27, #30.

-	Sol#35 proposes the similar mechanism as solution #5, #27, #30, #31.

Observation 1: When a SNPN supports as Onboarding Network, the O-SNPN supports the onboarding functionalities.

Proposal 1: A SIB indication is used by O-SNPN to indicate the support of onboarding. During UE registration, the UE includes the “onboarding” registration type, “onboarding” SUPI in registration request message, and provides an RRC indication that can be used by the NG-RAN to select an AMF supporting onboarding functionality. The onboarding functionalities should include a Maximum time duration, upon expiry of which the Configuration PDU session is released or the de-registration is triggered.

When PLMN supports as Onboarding Network (ON):

-	Sol#31 proposes that UE selects and registers to the onboarding PLMN using default operator profile (subscription and credentials to the onboarding PLMN for the UE to be "uniquely identifiable and verifiably secure"), and the AMF restricts this registration only for onboarding service only based on UDM indication, SMF restricts the PDU session for remote provisioning only based on UDM subscription. The UE shall initiate de-registration from the on-boarding network after finishing the remote provisioning or the on-boarding network shall initiate the de-registration based on timer configured for on-boarding service.

-	Sol#33 proposes the similar mechanism as solution #31, with the following differences:

-	no details on how to support the functionality to restrict usage to only on-boarding service

-	further describe that new NPN operator profile can be provided at the eUICC inside the device on top of the established PDU Session by using eSIM-based UE onboarding procedure between eUICC and eUICC platform as specified in GSMA

-	Sol#34 proposes that the PS initiates the onboarding procedures to PLMN via NEF by providing the GPSI of target UE, Onboarding Information, NPN type, and optional Triggering Events, then the normally registered UE can receive the Onboarding Information for SNPN or PNI-NPN via Control Plane or User Plane based remote provisioning. The UE shall initiate de-registration or CAG cell selection after finishing the remote provisioning.

-	Sol#39 proposes the similar mechanism as solution #31, with the following differences:

-	further describe the attach procedure to any onboarding PLMN using the default operator profile, that is used only for UE onboarding, e.g., contain a default Subscribed S-NSSAI and a default DNN that are appropriate to reach the PS

-	further describe the procedure on identifying the UE and the subscription/configuration/credential for remote provisioning using GPSI at PS

Observation 2: When PLMN supports as Onboarding Network and UE initiates the onboarding, a default PLMN operator profile needs to be used to support the functionality to restrict usage to only on-boarding service.

Proposal 2: When PLMN supports as Onboarding Network and UE initiates the onboarding, a default PLMN operator profile needs to be used to support the functionality to restrict usage to only on-boarding service. UDM indicates that this operator profile is only for onboarding service. A default subscribed DNN, S-NSSAI is supported for the PDU session to reach to the PS.

Observation 3: When PLMN supports as Onboarding Network and PS initiates the onboarding, PLMN needs to receive the onboarding request from PS via NEF and provisions the UE via control plane based remote provisioning or askes the UE to trigger the User Plane based remote provisioning.

Proposal 3: Use Sol#34 for PS-initiated onboarding and remote provisioning.

For the Configuration PDU session, all related solutions propose that this PDU session is used only for provisioning of the new subscription/credential.

-	Sol#5 uses a well-known or pre-configured S-NSSAI or DNN, or a combination of S-NSSAI and DNN, which is used just for provisioning purposes and has limited connectivity capabilities. The AMF selects a designated SMF, which in turn selects a designated PSA that provides a data connection restricted only to the Provisioning Server. The PCF may in addition provision URSP rules for the UE that restrict communication only to the provisioning server and/or specific applications.

-	Sol#27 is similar as Sol#5, with more description on subscription profile (e.g., a list of triplets (IP address, port number, protocol)) that restrict the connectivity of the Onboarding PDU Session. The SMF can get the subscription profile for onboarding PDU Session from local configuration, PS or AMF.

-	Sol#31 is similar as Sol#5, with more details on subscription profile (e.g. port ID, valid period, allowed destination address list and QoS information). Further, it describes how the SMF gets from the UDM the restriction information of the onboarding PDU Session in case the ON is PLMN or PNI-NPN.

-	Sol#33 is similar as Sol#31 in case the ON is PLMN.

-	Sol#35 is similar as Sol#5.

-	Sol#39 is similar as Sol#31 in case the ON is PLMN.

-	Sol#40 is similar as Sol#5.

Observation 4: The PDU session within the ON should be used only for remote provisioning.

Proposal 4: When User Plane is used for provisioning of SO-SNPN credentials, a restricted PDU session is supported to be dedicated for the remote provisioning based on local configuration or UDM subscription.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-07 V0.5.0.

# * * * * First change * * * *

## 7.4	Key Issue #4: UE Onboarding and remote provisioning

### 7.4.X	Evaluations for SNPN case

This clause provides evaluations for the solutions that address the SNPN case: provisioning of NPN credentials (i.e. for primary authentication) and other information to enable SNPN access.

This following table provides an overview for the solutions that address the SNPN case:

| Principle | What it solves | System impacts | Pros/Cons | Solutions |
| --- | --- | --- | --- | --- |
| Control Plane remote provisioning | Use the Control Plane remote provisioning | Impacts on AMF receiving data to be provisioned and forwarding it to the UE | Pros: efficient Cons: | Sol#6, #27, #30, #34, #35 |
| User Plane remote provisioning | Use the User Plane remote provisioning | A PDU session is established only for provisioning of NPN credentials (i.e. for primary authentication) and other information to enable SNPN access | Pros: Cons: | Sol#5, #27, #31, #33, #34, #35, #39, #40 |
| Primary authentication and authorization towards DCS via O-SNPN | O-SNPN supports the primary authentication and authorization towards DCS for UE using default credential | O-SNPN determines “onboarding registration” and interacts with the DCS to validate the UE | Pros: Cons: | Sol#5, #27, #30, #31 |
| Primary authentication and authorization towards DCS via SO-SNPN | SO-SNPN supports the primary authentication and authorization itself or towards DCS for UE using default credential | SO-SNPN interacts with the DCS to validate the UE | Pros: Cons: | Sol#35 |
| Primary authentication and authorization at PLMN | PLMN performs the primary authentication and authorization for UE using default operator profile | default operator profile is used | Pros: reuse existing mechanism Cons: | Sol#31, #33, #34, #39 |
| PS discovery | Discovery and selection of the target PS/SO-SNPN | Onboarding network discovers and selects the target PS or SO-SNPN | Pros: Cons: | Sol#5, #27, #30, #31, #36, #37, #39 |
| DCS discovery | Discovery and selection of the target DCS | Onboarding network discovers and selects the target DCS | Pros: Cons: | Sol#5, #27, #30, #31, #35 |
| O-SNPN discovery | Discovery and selection of the O-SNPN | UE discovers and selections the O-SNPN based on RAN broadcast information | Pros: Cons: | Sol#5, #27, #29, #31, #35, #40 |
| Subscription provisioning at UE | Store the NPN credentials (i.e. for primary authentication) and other information to enable SNPN access at ME or USIM/UICC | Once the UE receive the new NPN credentials (i.e. for primary authentication) and other information to enable SNPN access, it stores it at ME or USIM/UICC | Pros: Cons: | Sol#5, #6, #27, #29, #30, #31, #33, #34, #35, #39, #40 |
| Subscription provisioning at SNPN | UDM of SO-SNPN is provisioned with UE credentials/subscription data when remote provisioning is successfully performed | PS provides the new NPN credentials (i.e. for primary authentication) and other information to enable SNPN access to SNPN UDM | Pros: Cons: | Sol#6, #27, #30, #35, #39, #40 |

### 7.4.Y	Evaluations for PNI-NPN case

This clause provides evaluations for the solutions that address the SNPN case: provisioning of NPN credentials for access to specific slice(s) and/or PDU Sessions offering NPN services, i.e. for Network Slice Specific Authentication and Authorization and/or secondary authentication for PDU Sessions, among these solutions.

# * * * * End of changes * * * *