# 3GPP TSG SA2 Meeting #141E	S2-2007204r04

# October 12 – 23, 2020, Electronic meeting 	(revision of S2-200xxxx)

Source:	NTT DOCOMO, Nokia, Nokia Shanghai Bell

Title:	KI#1: Sol#1: Updates on Solution #1 for support of PTP/UDP/IP

Document for:	Discussion/Approval

Agenda Item:	8.5

Work Item / Release:	FS_IIoT / Rel-17

Abstract of the contribution: the document updates the Solution #1 for support of PTP over UDP/IP.

1		Discussion

1) It needs to be clarified how Solution #1 supports PTP over UDP/IP and IP type PDU Sessions. In addition, some clarifications to BMCA methods 1-4 are required.

2) Method 2 has an open EN:

Editor's note:	Whether NW-TT and DS-TT can send Announce messages only when there are chages including time out events and if there is a need for this optimization is FFS.

It is proposed in Method 2 to reduce signalling over the air and forward the received Announce messages from DS-TT to NW-TT only if the Announce is better than the current Announce the Master port is sending.

3) It is clarified that in Method 4 the Announce messages are generated from the NW-TT separately for each DS-TT port.

2		Proposal

The following change is proposed for TR 23.700-20.

*** Start Change ***

6.1	Solution #1: Uplink Time Synchronization for TSN

6.1.1	Introduction

The solution is proposed to solve Key Issue #1: the UL Time Synchronization. In this key issue, one or more TSN GM(s) are attached to the device side and are used to synchronize the TSN end stations behind the 5GS (i.e. NW-TTs) and behind the other UEs (i.e. DS-TTs). Based on clause 5.27.1 of TS 23.501 [2], the following principles are also applied to or UL TSN Time Synchronization:

-	5GS as a TSN bridge plays as a time-aware system.

-	Only TSN TTs at the edge of 5GS need to support the desired operations in the IEEE 802.1AS [6].

-	Two separate distribution time mechanisms are used for 5GS internal clock and TSN system clocks as defined in clause 5.27.1.2 of TS 23.501 [2].

-	All gPTP messages generated from TSN GM(s) are transmitted using user-plane resources in 5GS.

-	All the gNBs, DS-TT/UEs, and UPF/NW-TTs in the 5GS are time synchronized to the same 5GS GM.

.

Therefore, this solution proposes that similar operations defined for DL TSN Time Synchronization can be re-used for the UL TSN Time Synchronization i.e. for the TSN GM(s) attached to the device side.

Since UL TSN Time Synchronization will be distributed to the TSN end stations attached to 5GS (i.e. NW-TTs) and attached to the other UEs (i.e. DS-TTs), TSN GM(s) attached to the device will generate the UL gPTP messages and then DS-TT can perform exactly the same operations for the received DL gPTP messages as NW-TT performs for the DL gPTP messages defined in clause 5.27.1.2.2 of TS 23.501 [2]. Then, the modified UL gPTP messages will be further forwarded via the user-plane established between the devices (i.e. UE) which has TSN GM(s) attached to and the target UPFs.

After the NW-TT receives the modified gPTP messages for the case where delivery to end stations behind the 5G system (NW-TT) is required, the NW-TT can perform exactly the same operations as DS-TT performs for the received DL gPTP messages defined in clause 5.27.1.2.2 of TS 23.501 [2]. Finally, NW-TT can forward to the UL gPTP messages to the TSN end stations.

The main difference between the UL and DL Time Synchronization is that the UL gPTP messages are also required by the TSN end stations behind the other UEs (i.e. DS-TTs). For delivery of gPTP messages to TSN end stations behind other UEs, the UPF will forward the UL gPTP messages transparently to other UEs but not send it back to the source UEs. The DS-TT in the other UE can perform exactly the same operations as defined in clause 5.27.1.2.2 of TS 23.501 [2]. Finally, DS-TT can forward to the gPTP messages to the TSN end stations as described in the figure 6.1.2-1.

NOTE:	The UPF forwards the gPTP message to the Ethernet connected to the N6 interface. This enables other devices on N6 and also other DS-TT/UE connected to a different UPF to receive the gPTP message.

NOTE:	The gPTP messages are multicast data packet. The UPF/NW-TT prevents the occurrence of gPTP message loops.

6.1.2	Functional Description

Distributing the UL gPTP messages from TSN GM attached to the device to all TSN end stations behind the UPFs (NW-TTs) and behind the other UEs (DS-TTs) can be achieved by using the following steps (referred to Figure 6.1.2-1)

-	DS-TT which is attached by one or more TSN GMs will perform exactly same operations for UL gPTP messages as NW-TT perform the operations for the DL gPTP messages as specified in clause 5.27.1.2.2 of TS 23.501 [2].

-	In case of synchronizing TSN end stations behind 5G System (NW-TT), NW-TT will perform exactly the same operations for UL gPTP messages as what DS-TT performs for the DL gPTP messages as specified in clause 5.27.1.2.2 of TS 23.501 [2]. After NW-TT on the UPF side receives the UL gPTP messages, the NW-TT forward the information to the TSN end stations, while adding 5GS residence time to the correction field and removing TSi timestamp from the suffix field.

-	In case of synchronizing TSN end stations behind other UE(s), the UPF will forward the received UL gPTP message transparently to the other UEs to distribute the gPTP messages further to the TSN end stations behind other UEs (other DS-TTs) by applying the same forwarding principles as described in clause 5.27.1.2.2 of TS 23.501 [2] except that the PDU session of the source device connected to the corresponding DS-TT port is not included in the forwarding.

NOTE:	Normal Ethernet behaviour is that the frame is not sent back to the source. So, sending back a frame on the incoming interface would be against Ethernet principles and results in risking creation of forwarding loops.

In case of synchronizing TSN end stations behind 5G System (NW-TT), all gPTP messages are transmitted using the user-plane resources in 5GS and follows the principle that one PDU session is established per DS-TT port for a UPF. To support the multiple TSN working domains attached to the device, the mechanism defined in clause 5.27.1.3 of TS 23.501 [2] can be re-used.

In case of synchronizing TSN end stations behind other UE(s), all gPTP messages are transmitted using the user-plane resources in 5GS and follow the principle that two PDU sessions are used.

To support the multiple TSN working domains attached to the device, the mechanism defined in clause 5.27.1.3 of TS 23.501 [2] can be re-used.

Figure 6.1.2-1: The distribution of UL Time Synchronization Information with the same UPF

6.1.3	Procedures

6.1.3.1 	Synchronizing end stations behind NW-TT

The procedure for synchronizing TSN end stations behind 5G System (NW-TT) with the TSN GM in the network attached to the device is described in Figure 6.1.3-1. It includes BMCA-related steps (step 1~3) because after those steps, the sync message forwarding tree is confirmed. In this case, TSN Node 0 should send sync messages to 5GS TSN Bridge, and the 5GS TSN Bridge should send sync messages to TSN Node 2. The remaining steps (step 4~8) follow principles in clause 5.27.1 of TS 23.501 [2], with the Ingress TT of DS-TT1 and the Egress TT of NW-TT.

Figure 6.1.3.1-1: Procedure for synchronizing TSN end stations behind 5G System (NW-TT) with the TSN GM in the network attached to the device side via 5G System

1.	PDU Session Establishment includes UE MAC / Port Info for updates to TSN AF. After this step, TSN Node 0, 5GS logical bridge (DS-TT1/UE1, NW-TT/UPF) and TSN Node 2 can exchange Ethernet frames.

2.	At BMCA step, TSN nodes exchange Announce messages and build the Sync forwarding tree for TSN Sync messages. The Sync forwarding tree is per TSN working clock domain. BMCA procedure (6.1.3.3 for gPTP) is performed when a NW-TT port or a DS-TT port receives an Announce message.

3.	[Optional] If the operator decides to follow the recommendation to limit the bridge residence time to 10 ms, as defined in IEEE 802.1AS [6], clause B2.2 (stated as recommendation), then Based on the Sync frame forwarding information the TSN AF may trigger QoS setup for the Sync frame delivery. PDB1 which is the UL PDB for the QoS flow of PDU session for UE1, plus UE1-DS-TT residence time, should be less than 10ms, as the residence time in a node should be less than 10ms. This step can be skipped if step 1 satisfies the QoS requirements.

NOTE 1:	The PDB value for the case of dedicated synchronization communication (i.e. a QoS flow dedicated for transmitting gPTP messages) can be pre-configured by the operator at the PCF. In this case, the PCF sets up QoS parameter (i.e. PDB) to fulfil DS-TT to UPF delay to be less than 5 ms, such that the total limit is achieved, without the need for the PCF to learn if this concerns a DS-TT/UE to NW-TT/UPF communication or a DS-TT/UE to DS-TT/UE communication.

NOTE 2:	As a result of steps 2 and 3, BMCA Announce messages are delivered in a default QoS flow but only time synchronization messages are delivered in the new QoS flow.

4.	Sync message is delivered from the previous TSN node to DS-TT/UE1. It has time stamp of TSN GM clock. It also includes correction and rateRatio fields. The rateRatio is (GM clock frequency) / (local clock frequency).

5.	DS-TT/UE1 updates the Sync message. It adds the link delay between the DS-TT/UE1 and the previous TSN node to the correction field. It also updates the rateRatio field with the previous rateRatio multiplied by neighborRateRatio. The neighborRateRatio = (local clock frequency of the previous TSN Node) / (local clock frequency). It also attaches Ingress Timestamp based on 5G GM clock to the Sync message.

6.	Sync message is delivered from DS-TT/UE1 towards NW-TT/UPF. It has time stamp of TSN GM clock. It also includes correction, rateRatio and Ingress Timestamp fields.

7.	NW-TT regenerates the Sync message. It calculates the residence time as Egress 5G GM Time - Ingress 5G GM TS) * rateRatio in the sync message. It updates the correction field as the previous correction field value + the residence time. Then, it removes the Ingress timestamp.

8.	Sync message is delivered from the NW-TT to TSN Node 2. It has time stamp of TSN GM clock. It also includes correction and rateRatio fields.

6.1.3.2 	Synchronizing end stations behind DS-TT

The procedure for synchronizing TSN end stations behind other UE(s) with the TSN GM in the network attached to the device is described in Figure 6.1.3.2-2. It also includes BMCA-related steps (step 1~3). After those steps, the sync message forwarding tree is confirmed. In this case, TSN Node 0 should send sync messages to 5GS TSN Bridge, and the 5GS TSN Bridge should send sync messages to TSN Node 3. The remaining steps (step 4~8/8a) follow principles in clause 5.27.1 of TS 23.501 [2], with the Ingress TT of DS-TT1 and the Egress TT of DS-TT2.

Figure 6.1.3.2-1: Procedure for synchronizing TSN end stations behind other UE(s) with the TSN GM in the network attached to the device side via 5G System

1.	PDU Session Establishment includes UE MAC / Port Info for updates to TSN AF. After this step, TSN Node 0 and 5GS logical bridge (DS-TT1/UE1, NW-TT/UPF) can exchange Ethernet frames.

1a.	For UE-to-UE case, PDU Session Establishment includes UE MAC / Port Info for updates to TSN AF. After this step, TSN Node 3 and 5GS logical bridge (DS-TT2/UE2, NW-TT/UPF) can exchange Ethernet frames. Local switching at the UPF is also enabled.

2.	At BMCA step, TSN nodes exchange Announce messages and build the Sync forwarding tree for TSN Sync messages. The Sync forwarding tree is per TSN working clock domain. BMCA procedure (6.1.3.3 for gPTP) is performed when a NW-TT port or a DS-TT port receives an Announce message.

3.	[Optional] If the operator decides to follow the recommendation to limit the bridge residence time to 10 ms, as defined in IEEE 802.1AS [6], clause B2.2 (stated as recommendation), then Based on the Sync message forwarding information the TSN AF may trigger QoS setup for the Sync message delivery. PDB1, which is the UL PDB for the QoS flow of PDU session for UE1, should be less than 10ms, as the residence time in a node should be less than 10ms.

3a.	[Optional] If the operator decides to follow the recommendation to limit the bridge residence time to 10 ms, as defined in IEEE 802.1AS [6], clause B2.2 (stated as recommendation), then, Based on the Sync message forwarding information, the TSN AF may trigger QoS setup. As the TSN AF can know DS-TT1/UE1 forwards Sync messages to DS-TT2/UE2 via local switching at the UPF, it sets the QoS should meet the sum of PDB1, UE1/DS-TT1 residence time, local switching delay, UE2/DS-TT2 residence time and PDB2 should be less than 10ms. PDB2 is the DL PDB for the QoS flow of PDU session for UE2. For this, the TSN AF may trigger the QoS change for PDU session for UE1.

NOTE 3:	Calculation of the UE-UE delay can be consistent with the UE-UE TSC communication solution chosen.

NOTE 4:	The PDB value for the case of dedicated synchronization communication (i.e. a QoS flow dedicated for transmitting gPTP messages) can be pre-configured by the operator at the PCF. In that case, the PCF sets up QoS parameter (i.e. PDB) in all cases fulfilling DS-TT to UPF delay to be less than 5 ms, such that the total limit is achieved.

NOTE 5:	Based on BMCA results, the TSN AF informs the SMF of the associated PDU sessions. That is, the TSN AF informs the SMF of the PDU Session for the DS-TT port with Slave role and the PDU Session for the DS-TT port with Master role.

NOTE 6:	As a result of steps 2 and 3, BMCA Announce messages are delivered in a default QoS flow but only time synchronization messages are delivered in the new QoS flow.

Editor's note:	Calculation of the UE-UE delay, consistent with the UE-UE TSC communication solution chosen, needs to be updated for this step.

Editor's note:	If SMF needs to know then how the SMF knows the UEs that need to communicate and which PDU sessions to associate is FFS.

4.	Sync message is delivered from the previous TSN node to DS-TT/UE1. It has time stamp of TSN GM clock. It also includes correction and rateRatio fields. The rateRatio is (GM clock frequency) / (local clock frequency).

5.	DS-TT/UE1 regenerates the Sync message. It adds the link delay between the DS-TT/UE1 and the previous TSN node to the correction field. It also updates the rateRatio field with the previous rateRatio multiplied by neighborRateRatio. The neighborRateRatio = (local clock frequency of the previous TSN Node) / (local clock frequency). It also attaches Ingress Timestamp based on 5G GM clock to the Sync message.

6.	Sync message is delivered from DS-TT/UE1 towards NW-TT/UPF. It has time stamp of TSN GM clock. It also includes correction, rateRatio and Ingress Timestamp fields.

6a.	For UE-to-UE case, the NW-TT/UPF performs local switching and forwarding for the Sync message. Before local switching, the NW-TT/UPF can perform multiplication if needed.

6b.	For UE-to-UE case, Sync message is delivered from the NW-TT/UPF to DS-TT/UE2. It has time stamp of TSN GM clock. It also includes correction, rateRatio and Ingress Timestamp fields.

7a.	For UE-to-UE case, DS-TT/UE2 updates the Sync message. It calculates the residence time as Egress 5G GM Time - Ingress 5G GM TS) * rateRatio in the sync message. It updates the correction field as (the previous correction field value) + (the residence time). Then, it removes the Ingress timestamp.

8a.	For UE-to-UE case, Sync message is delivered from the DS-TT/UE2 to TSN Node 3. It has time stamp of TSN GM clock. It also includes correction and rateRatio fields.

6.1.3.3 BMCA procedure

One of the following alternative methods for the BMCA procedure may be implemented. The BMCA state machines required in IEEE 802.1AS [6] clause 5.4.1, may run in different parts of the 5G system.

Method 1:

When a NW-TT port or a DS-TT port receives an Announce message, if the Announce information is better than the current best master information it knows about (per clock domain), the port reports the Announce information to TSN AF using PMIC signalling. NW-TT and DS-TT also inform TSN AF if a previously reported best clock is not available anymore (determined based on lack of Announce or Time Sync messages from that clock). Based on the received Announce information and local information of 5GS, the TSN AF runs the PortStateSelection state machine and decides the BMCA port roles for the ports in the 5GS bridge as described in IEEE 802.1AS-Rev [6] and configures each port role (per clock domain) accordingly using PMIC signalling. The port that is configured to Slave role, receives Sync and Announce messages from a node outside 5GS and sends the Sync and Announce messages to other port(s) inside 5GS. The port that is configured to Master role, receives Announce messages from the port with Slave role inside the 5GS bridge, increases the stepsRemoved field of the Announce messages with one, replaces the value of sourcePortIdentity field of Sync/Announce messages with its portIdentity, and runs the PortAnnounceTransmit state machine to send Announce messages to a node outside the 5GS. When DS-TT port configured to Passive role receives a Sync message from outside of 5GS, it discards the message. Optinally, NW-TT and DS-TT with port(s) with Master role(s) can generate Announce messages based on information from TSN AF. In this case, Announce messages are not exchanged inside 5GS. (Details are described in Solution #18 in clause 6.18.)

NOTE 1:	Depending on the solution to expose the time syncronization service, the BMCA port roles can be assigned either by TSN AF or NEF.

Method 2:

In this alternative, NW-TT receives Announce messages from any NW-TT and DS-TT ingress port (DS-TT forwards all Announce messages to NW-TT) and NW-TT runs the PortStateSelection state machine and decides the BMCA port state.

Upon completion of BMCA, if the TSN AF (or NEF) has subscribed for the BMCA result reports, and if the NW-TT determines that the result of the BMCA has change (which may cause the (g)PTP grandmaster must be activated or deactivated  in DS-TT(s)), NW-TT reports the BMCA result to TSN AF using BMIC signaling. Based on this, TSN AF configures the port states in DS-TT(s) using PMIC signaling. If TSN AF is not deployed then NEF terminates PMIC and BMIC signaling, i.e. NEF receives the BMCA result from NW-TT and configures configures the port states in DS-TT using PMIC signaling.

NOTE 2:	NEF terminating BMIC/PMIC signaling does not have any impact on PCF/SMF as existing Rel-16 functionality is reused for this.

Port states of NW-TT ports are configured locally by NW-TT based on the BCMA result. When a DS-TT configured to Master role receives Announce messages from the NW-TT via 5GS user plane, it increases the stepsRemoved field of the Announce messages with one, replaces the value of sourcePortIdentity field of Sync/Announce messages with its portIdentity, and runs the PortAnnounceTransmit state machine to send Announce messages to a node outside the 5GS. When DS-TT port configured to Passive role receives a Sync message from outside of 5GS, it discards the message. If the NW-TT time outs to receive Announce message from the Slave port in DS-TT or NW-TT and has available Passive port(s) in DS-TT or NW-TT, the NW-TT selects a Passive as new Slave port and update port state(s).

Method 2b:

Optionally, Mothod 2 can be optimized to filter the Announce messages in the ingress port.

For BMCA, each TSN node sends Announce messages to its neighbors. When a TSN node receives an Announce message, from its neigher TSN node, it can locally calculate the ststus of the receiving port basd on the priority priority vector. A TSN node with multiple ports, the status of each port and priority vector need to be aggregated to make a centralized decision on the TSN node level. If the aggregation is done via User Plane, for each port to send the received Announce message to a central unit inside the TSN node may be a solution. On the other hand, for each port to send the changed Announce message to the central uniti inside the TSN node will do the same effects in terms of the central unit, because repeating the same Annouce messages from a port to the central unit does not make any differences. If a port detects a receipt timeout event for an Announce message from its neighbour, it means the port updates its local decision and the port priority vector. In that case, the change needs to be delivered to the central unit of the TSN node. The port may also send an Announce message that was expected to arrive with an additional field to inform receipt timeout event.

Method 3:

Support BMCA via the interworking between the DS-TT and NW-TT:

-	When DS-TT received any announce messages from the port(s) of the DS-TT, if the message Priority Vector of the announce message is better than the Port Priority Vector of the received port, the DS-TT sends the received announce messages to NW-TT via UE.

Editor's note:	It is FFS how a Passive port would learn the Port Priority Vector.

-	The NW-TT/UPF learns the received port for the announce message based on the port number associated with the PDU session sending the announce message. The NW-TT determines the best grand master based on the received announce messages, the port number of the received port on the DS-TT(S) and NW-TT, and the System Priority Vector. If the determined grand master is updated, the NW-TT decides the port state and announce message for next hop accordingly.

-	If the port state of a NW-TT port is master port, the NW-TT forwards the determined announce message to the NW-TT port directly. For each DS-TT port which is a Master port, the NW-TT/UPF runs the PortAnnounceTransmit state machine to send Announce messages towards the PDU session associated with the DS-TT port. Upon receiving the announce message the DS-TT forwards it to the port associated the PDU session receiving the announce message and update the Port Priority Vector accordingly.

-	When DS-TT port configured to Passive role receives an Announce message or a Sync message from outside of 5GS, it forwards it to to NW-TT who discards the message.

Method 4:

Support BMCA process in NW-TT (Details are described in Solution #17).

-	There is a BMCA function in the NW-TT. It runs the BMCA state machine and keeps the state of all ports.

-	When the DS-TT port or NW-TT port receives the Announce message, it forwards the message to BMCA function in NW-TT via U-plane.

-	The BMCA function runs the BMCA state machine to qualify the message, determines the best grand master and determines the DS-TT port and NW-TT port states.

-	If the Announce message is received from the slave port, the BMCA function updates the Announce message and generates an Announce message separately for each DS-TT and NW-TT port which state is Master.

-	If the Announce message is reveived from a non-slave port, the BMCA function discards this message.

-	Upon completion of BMCA, if the TSN AF (or NEF) has subscribed for the BMCA result reports, and if the NW-TT determines that the result of the BMCA has changed which may cause the (g)PTP grandmaster in 5GS (e.g. DS-TT) must be activated or deactivated, , the NW-TT reports the BMCA result to TSN AF (or NEF) using BMIC signaling. Based on this, TSN AF enables or disables the GM in DS-TT using PMIC signaling. Solution #9 describes the details how to activate/deactivate the grandmaster in DS-TT.

6.1.4	Impacts on services, entities and interfaces

For synchronizing TSN end stations behind 5G System (NW-TT) with the TSN GM in the network attached to the device, the impacts on UE, SMF, PCF, TSN-AF and UPF are like the following.

-	The Ingress TT is DS-TT.

-	The Egress TT is NW-TT.

For synchronizing TSN end stations behind other UE(s) with the TSN GM in the network attached to the device side via 5G System, the impacts on UE, SMF, PCF, TSN-AF and UPF are like the following.

-	The Ingress TT is DS-TT of a UE.

-	The Egress TT is DS-TT of the other UE.

-

-	[Optional] The TSN AF needs to trigger QoS setup procedures in case the operator decides to follow the recommendation to limit the bridge residence time to 10 ms

-	The UPF needs to perform local switching for Sync messages without NW-TT interaction.

*** End of Changes ***