3GPP TSG-SA/WG2 Meeting #141e	S2-2007365r04

# Elbonia, 12-23 Oct, 2020

Source:	ZTE, CATT, China Mobile, Nokia, Nokia Shanghai Bell

Title:	KI#5 evaluation and conclusion

Document for:	Agreement

Agenda Item: 	8.3

Work Item / Release:	FS_enh_EC / Rel-17

Abstract of the contribution: This contribution proposes conclusion for KI#5

1	Proposal

It is proposed to agree the changes:

/********************Start of Change***************/

7.2	Evaluation of solutions for Key Issue #5

There are two solutions on KI#5. One difference is in solution#50 the target DNAI(s) is applied to existing PDU session while in solution#12 the requested DNAI(s) is applied to subsequent new PDU session. In both solutions the target DNAI(s) is sent to AMF via Nsmf_PDUSession_SMContextStatusNotify service operation. After the AMF receives the target DNAI(s) it can insert a proper I-SMF for current PDU session(solution#50), or select a proper new I-SMF/SMF which can serve the requested DNAI(s) for subsequent PDU session(solution#12).

/*******************Next Change****************/

9	Conclusions

9.x	Conclusions for Key Issue #5

The SMF uses Nsmf_PDUSession_SMContextStatusNotify operation to send the target DNAI(s) to the AMF to assistant the I-SMF/SMF selection. The AMF then uses the target DNAI(s) for I-SMF/SMF selection for existing or subsequent PDU Session. Clause 6.12.2.2 in Solutions #12 and solution#50 are recommended into normative with below considerations:

- Normative work for KI5 shall only impact N11 (AMF-SMF interface) and N16a (to relay information). No PCF impact and no N4 impact are to be considered

- There is no I-SMF chaining induced by these solutions

- SMF decides whether it is required to send the target DNAI(s) to the AMF for triggering I-SMF (re)selection.

- AMF may retrieve the list of SMFs supporting the requested DNAI(s) from NRF, and considers the last DNAI(s) received from the SMF to decide the proper I-SMF.

- Nsmf_PDUSession_SMContextStatusNotify operation is used by normative extensions due to these solutions. They will use 2 different notifications: DNAI for the current PDU Session / DNAI for the subsequent PDU Session for the AMF to be able to distinguish between I-SMF selection for the PDU Session with SSC mode 1(situation of sol 50) and SMF selection for PDU Session with SSC mode 2/3 (situation of of solution 12 ).

/*******************End of Change****************/