3GPP TSG-WG SA2 Meeting #141E e-meeting 	S2-2007461r02

Elbonia, October 12 – 23, 2020	(revision of S2-200xxxx)

Source:	Huawei, Hisilicon, Toyota

Title:	KI#2: Conclusion on AF based EAS re-discovery

Document for:	Discussion/Approval

Agenda Item:	8.3

Work Item / Release:	FS_enh_EC / Rel-17

Abstract of the contribution: This contribution proposes conclusion on C3 solutions (AF based EAS re-discovery).

- Discussion

Following the categories of solutions described in the way forward paper S2-2007008, this paper addresses the evaluation and conclusion of C3 solutions, especially on the AF based re-discovery aspects.

The following solutions support EAS relocation in which the AF reselects the target EAS, e.g. when UP path change notification is received from the 5GC, or, due to application reasons such as EAS load re-balance:

| Solution #25 | The EAS has AnyCast addresses that are routed to. To find the old EAS for sessions that are still to be served by that the flow state (IP, port/proto) is transferred to the new UPF |
| --- | --- |
| Solution #27 (EAS Reselection part) | The AF reselects the new EAS based on UP path change notification, and the application layer mechanisms are used to notify the UE with the new EAS IP address, for example via HTTP redirection. |
| Solution #28 | The solution is to support the application change independent of UE mobility. The changed IP address of application server may be send to the SMF with the notification and to the UE via PCO. |
| Solution #31 | Application Relocation with UE assistance. The solution assumes that only the application layer state is transferred from old EAS to the new EAS. Uses NAS signalling to inform the network about the new EAS IP address. |
| Solution #37 | AF-based EAS End-Point-Address update via External Parameter Provisioning. Foresees impacts on the following Nodes or/and Functionality: AF, PCF, UE. |
| Solution #52 | AF indicates “keepExisting” indication. During ULCL change, the N9 forwarding tunnel is established based on the indication. The AF based on UP path change notification or based on its own decision, reselects new EAS. The new EAS is informed to UE via application signalling. |
| Solution #53 | In clause 6.53.2.2, the AF selects new EAS, and provide the new EAS to UE via application signalling. Forwarding tunnel may be established between new and target ULCL. |
| Solution #54 | For SSC mode 2/3, the SMF notifies the AF of the DNAI change, based on DNAI change, the target EAS can be selected, and context is migrated. The UE is informed of the target EAS. |

For all these possible solutions, there are 2 different ways to inform the UE of the new EAS IP address:

- Alt A: the EAS IP address is notified to UE via application layer signaling. It includes: Sol #25, #27, #53, and #54.

- Alt B: The EAS IP address is notified to 5GC, and the 5GC sends the new EAS IP address to UE via NAS signaling. It includes: Sol #28, #31 and #37.

In our view, Alt A is possible if the application layer supports to inform the target EAS to UE. Alt B also need be supported considering that the UE may not support such application layer redirection mechanism.

That EAS IP address carried in NAS messages sent to UE makes impact on UE. The SMF sends NAS message that carries EAS IP address to UE via ePCO and UE OS/kernel must deliver EAS IP address to upper layer such as Application Client. It depends on UE capability to support deliver EAS IP address to upper layer. Now UE doesn’t support this capability. UE OS are specific designed and UE has no guaranteed to support this application characteristic via 5GS. Also, for service providers such as operators and 3rd party, they can’t know in advance whether all of the edge computing consumer own this UE capability. The UE without this capability will no longer enjoy this edge service. It is harmful for ecology of edge computing service and widely deployment. Without UE capability, 5GC can’t provide EAS IP address to application layer via NAS messages. It is not recommended for normative work.

Besides above solution #31 support to send a EAS IP address via NAS signaling to UE. This is not recommended to be adopted for normative work.

Conclusion:

The AF may reselect a new EAS for UE due to UP path change notification or its internal trigger. When the new EAS is reselected, the UE is informed with the new EAS IP address via the application layer signaling. Providing EAS IP address to application layer via NAS messages is not recommended.

- Proposal

The following change is proposed to TR 23.747 v1.0.0

/********************** Start of Changes************************/

7.x  Evaluation of Solutions for Key Issue #2

7.x.y Evaluation for Key Issue #2: AF based EAS rediscovery

In solution #25, #27 (EAS Reselection part), #28, #31, #37, #52, #53 (clause 6.53.2.2), #54, the new EAS can be selected by AF. After that the new EAS address need be notified to UE.

For all these possible solutions, there are 2 different ways to inform the UE of the new EAS IP address:

- Alt A: the EAS IP address is notified to UE via application layer signaling. This option doesn’t have any impact on the 5GC and has already been supported by some applications. It includes: Sol #25, #27, #53, and #54.

- Alt B: The EAS IP address is notified to 5GC, and the 5GC sends the new EAS IP address to UE via NAS signaling. This option has impact on both UE and 5GC.It includes: Sol #28, #31 and #37.

Alt A is possible if the application layer supports to inform the target EAS to UE.

That EAS IP address carried in NAS messages sent to UE makes impact on UE. The SMF sends NAS message that carries EAS IP address to UE via ePCO and UE OS/kernel must deliver EAS IP address to upper layer such as Application Client. It depends on UE capability to support deliver EAS IP address to upper layer. Now UE doesn’t support this capability. UE OS are specific designed and UE has no guaranteed to support this application characteristic via 5GS. Also, for service providers such as operators and 3rd party, they can’t know in advance whether all of the edge computing consumer own this UE capability. The UE without this capability will no longer enjoy this edge service. It is harmful for ecology of edge computing service and widely deployment. Without UE capability, 5GC can’t provide EAS IP address to application layer via NAS messages. It is not recommended for normative work.

/********************** 2nd change ************************/

9	Conclusions

9.x  Conclusions for Key Issue #2

9.x.y Conclusions regarding solutions for Key Issue #2 for AF based EAS rediscovery

The AF may reselect a new EAS for UE due to UP path change notification or by its internal trigger, e.g. load balance. When the new EAS is reselected, the UE is informed with the new EAS address via the application layer signaling.

Without UE capability, 5GC can’t provide EAS IP address to application layer via NAS messages. Providing EAS IP address to application client has both impact on UE and network. It is not recommended for normative work.

/*********************** End of Changes **********************/