SA WG2 Meeting #141e	S2-2007517

Electronic Meeting, October 12 – 23, 2020

Source:	Nokia, Nokia Shanghai Bell

Title:	KI #3, Evaluation

Document for:	Approval

Agenda Item:	8.4

Work Item / Release:	FS_eNS_ph2 / Rel-17

Abstract of the contribution: This p-CR proposes a KI#3 way forward

# 1	Discussion

This paper provides an updated evaluation and conclusion for the Key Issue #3 solutions.

# 2	Proposal

This contribution proposes to implement the following updates to TR 23.700-40 v1.0.0.

* * * Start of Change 1 * * *

# 7	Evaluation

Editor's note:	This clause will provide a general evaluation of the solutions.

## 7.1	Evaluation on solutions of KI#1

## 7.2	Evaluation on solutions of KI#2

## 7.3	Evaluation on solutions of KI#3

Editor's note:	This clause will provide some interim evaluation based on solutions #13, #20, #21, #22 that will need further updates to address e.g. roaming aspects.

High level aspects of the solutions:

-	Solution 22 has RAN impact. It lets RAN to enforce the SMBR (Slice Maximum Bitrate).. Currently, RAN is able to be aware of the S-NSSAI of the PDU Session. And RAN is able to be enforce the UE AMBR per UE and GFBR/MFBR per QoS Flow.

Editor´s note:	Solution#22 needs to be validated with RAN WG2 and RAN WG3, due to RAN impacts.

-	Solution 13 uses UPF to enforce the DL slice level bitrate. This solution will require to select the same SMF/PCF and UPF for all the PDU Sessions within the slice. It is not necessary to introduce such limitation.

-	Distribution based solutions, i.e. Solution 20&21, let a centralized NF distribute the SMBR into pieces (i.e. Session AMBR and/or MFBR). They have no RAN impact. However, solutions do not explain how to resolve the fact that since the SMBR is distributed into Session-AMBRs, the aggregated SMBR enforced may be smaller than the SMBR, as such the SLA would not be fulfilled, as the UE will be throttled while SMBR is not fully consumed. The situation could be worse when a large amount of PDU Sessions exist as the SMBR is distributed over more Session AMBR. In general, the subscribed Session AMBR should be delivered when there is no competing traffic. In particular, no session AMBR means there is no limit to the rate the UE could enjoy (the UE should be able to enjoy up to the peak rate or the SMBR of the slice or the UE-AMBR for the UE. this solution leveraging the existing Session AMBR parameter would break the existing parameter definition and purpose.

# 8	Conclusions

Editor's note:	This clause captures interim conclusions from the study until the WG agrees these are stable and this editor's note is removed.

To limit the rate of a UE for a network slice that is identified to have maximum rate UL/DL for the UE, it is proposed that the RAN performs the rate limitation of the UE up to the Maximum Rate UL/DL for the UE. This includes not admitting more GBR resource type QoS flows if this would cause this Maximum rate limitation to be broken.