SA WG2 Meeting #S2-141E	S2-2007595

12 October - 23 October 2020, Electronic, Elbonia	(revision of S2-200xxxx)

Source:	ZTE

Title:	KI#9 proposal on the interim conclusion

Document for:	Agreement

Agenda Item:	8.9

Work Item / Release:	FS_5MBS / Rel-17

Abstract of the contribution: This paper give the interim conclusion for the key issue#9.

1. Proposal

*****************************************START of CHANGE **********************************************

7.X	Key Issue #9: Minimizing the interruption of public safety services upon transition between NR/5GC and E-UTRAN/EPC

For services that require interworking with LTE eMBMS the following considerations are taken into account:

-	LTE eMBMS requires the use of a BM-SC. This includes the use of a TMGI to identify the service and session on the LTE side.

-	For a UE to be able to identify the same service over LTE eMBMS and 5G MBS, then one approach is to identify the 5G MBS service with the same TMGI.

-	The only way to achieve this is by a joint BM-SC/MBSF which can activate the transport in both LTE and 5GS.

-	Therefore, a solution with joint BM-SC/MBSF is the only approach that can satisfy interworking between LTE eMBMS and 5G MBS.

*********************************************Next CHANGE **********************************************

8.X	Conclusions for Key Issue #9: Minimizing the interruption of public safety services upon transition between NR/5GC and E-UTRAN/EPC

To enable support mobility and service continuity between 5MBS (NR/5GC) and LTE MBMS, following are some interim principles to be considered for the way forward:

-	Both Full Service mode interworking solution and Transparent Service mode interworking solution are considered.

-	The unicast PDU session associated with MBS session shall be established before the mobility to EPS for the Transparent Service mode interworking solution.

*******************************************End of CHANGE **********************************************