SA WG2 Meeting #141e	S2-2007598r02

# Oct 12th – Oct 23rd, 2020 ; Elbonia

Source:	Nokia, Nokia Shanghai Bell

Title:	KI#1, Sol#6: Evaluation of the level on which PC5 DRX schedules should be mapped

Document for:	Approval

Agenda Item:	8.12

Work Item / Release:	FS_eV2XARC_Ph2 / Rel-17

Abstract of the contribution: This paper analyses the advantages and disadvantages of mapping default PC5 DRX schedules to PC5 RATs (proposed in Solution #6), compared to mapping them to V2X services or PQIs.

# 1 Introduction

Solution#6 proposes to define one PC5 DRX cycle per NR PC5 RAT and not per V2X service type, per PQI, or similar, arguing that per V2X service cycles would lead to longer ON durations. This paper adds an evaluation section which discusses this aspect, as well as further advantages and disadvantages of the different level on which "default/provisioned PC5 DRX cycles" could be defined.

# 2 Proposal

It is proposed to update TR 23.776-020 as follows:

* * * * First Change * * * *

### 6.6.5	Evaluation

Provisioned default PC5 DRX cycles (or schedules, as they are termed by other solutions) can be associated with the PC5 RAT type (i.e., one default cycle for all NR PC5 communications, as suggested by Solution#6) or they can be more fine-granular, e.g., associated with either V2X services (as suggested by Solution#2) or PQIs.

If V2X services apply their own, independent, and uncoordinated PC5 DRX cycles, this could indeed lead to increased ON durations of the UEs that run many services, as argued in clause 6.6.2

The main advantages and disadvantages of the different approaches are listed in the following:

-	PC5 DRX cycles/schedules per PC5 RAT

-	Advantages: Simpler solution leading to smaller V2X policies, absolute alignment of all (not customized or further updated/negotiated) UE PC5 DRX cycles.

-	Disadvantages: Too coarse PC5 DRX cycles that do not reduce the ON durations as much as it could be done for certain V2X services or V2X service combinations.

-	PC5 DRX cycles/schedules per V2X service(s)

-	Advantages: Alignment of the PC5 DRX cycles of the UEs that run the same V2X service types or V2X service type combinations, which are the UEs that are actually expected to communicate with each other. Shorter ON durations for most V2X services or V2X service combinations due to the high granularity of the set of applicable PC5 DRX cycles, thus helping to save energy.

-	Disadvantages: Needs custom logic for selecting a PC5 DRX cycle when multiple PC5 DRX cycles could cover/satisfy the V2X services running on the UE. In case V2X applications have different QoS requirements for the same V2X service type a longer PC5 DRX ON duration will be required to support the highest QoS reducing the power efficiency of the UE.

-	PC5 DRX cycles/schedules per PQI

-	Advantages: Can use the PQI priority to select a PC5 DRX cycle when different PC5 DRX cycles match different V2X services running on the UE. Shorter ON durations for most V2X services or V2X service combinations due to the high granularity of the set of applicable PC5 DRX cycles, thus helping to save energy.

### -	Disadvantages: Possible non-alignment of the PC5 DRX cycles of the UEs that run the same V2X service types or V2X service type combinations (because the same service types may eventually be executed with different PQIs on different UEs), which are the UEs that are actually expected to communicate with each other.

* * * * End of Changes * * * *