SA WG2 Meeting #141E (e-meeting)	S2-2007635

# 12 – 23 October, 2020, Elbonia

Source:	NEC, Apple

Title:	KI#2 Solutions evaluation and conclusion

Document for:	Approval

Agenda Item: 	8.4

Work Item / Release:	FS_eNS_Ph2 / Rel-17

Abstract of the contribution: This contribution proposes KI#2 solutions evaluation and conclusion.

# 1	Proposal

It is proposed to agree the following KI#2 evaluation and conclusion results in TR23.700-40, v1.0.0

/****************start of 1st change***************/

## 7.x	Key Issue #2 evaluation

Table 7.x-1: Key Issue #2 Solutions Evaluation

| Solution # | Title | Evaluation |
| --- | --- | --- |
| Sol#5 | NWDAF enhancements for supporting of network slice quota on the maximum number of PDU Sessions | Here are the solution principles: Quota holding entity – the quota is held in the NWDAF and may initially be provided there by the OAM at initiation of the network slice instance. However, when this solution was agreed, the NWDAF was assumed to be with PLMN granularity. NOTE: The eNA study is agreeing for location based NWDAF, i.e. multiple NWDAFs within the PLMN. This solution needs further clarification on how the quota is maintained with the new eNA NWDAF architecture – i.e. dependency on eNA NWDAF decision in Rel-17. Quota managing entity – the quota is managed by the NWDAF. At PDU Session establishment the SMF checks the quota availability with the NWDAF. NOTE: Need further clarification on how the quota is managed across the local NWDAFs. Quota enforcing entity – SMFs which interacts with the NWDAF. Roaming – supported. Enforced by the hSMF. |
| Sol#6 | PCF-based counting of PDU Sessions in a Network Slice | Here are the solution principles: Quota holding entity – the global quota is either configured in the master PCF or the global quota is a subscription information. Quota managing entity – the quota is managed by the master PCF. The master PCF checks for quota availability and updates the quota at PDU Session establishing or releasing. New services and signalling are introduced between the master PCF and the local PCFs. New service and signalling are introduced between the vPCF and hPCF. NOTE: Need further clarification if primary PCF fails. Quota enforcing entity – SMFs based on the input from the master PCF via the local PCFs. Roaming – supported. Quota enforced by the master hPCF via the vPCF or by the vPCF while roaming with LBO. |
| Sol#7 | Support of Network Slice SLA for Maximum Number of PDU sessions parameter | Here are the solution principles: This solution is very similar to sol#6. The main difference is that the quota is managed by local PCF which holds the local quota allocated and redistributed by the primary PCF. New service and signalling are introduced between primary PCF and local PCF. NOTE: Need further clarification if primary PCF fails. Quota holding entity – the global quota is held in the UDR Quota managing entity – the global quota is managed by a primary PCF and the local quota is managed by the local PCFs. The primary PCF retrieves the global quota from the UDM and then distributes it to local PCFs and takes care of its re-distribution between the local PCFs when needed. This brings complexity in the primary PCF interaction with the local PCFs and increased signalling. Quota enforcing entity – by SMFs based on the input from the local PCFs. Roaming – supported. Quota enforced by the primary PCF in the hPLMN via the PCF in the vPLMN or by the local vPCF while roaming with LBO. |
| Sol#8 | AMF and O&M based solution | Here are the solution principles: Quota holding entity – global quota is held in the OAM and local quota is held in multiple AMFs. NOTE: Need to consult with SA5 on the proposed interactions between the OAM (in the SA5 domain) and multiple AMFs (in SA2 domain) for the local quota update and re-adjustment. Quota managing entity – local granularity management by the AMFs and globally managed by the OAM. Requires local quotas updates between AMFs each time the UE moves to new registration area which incurs extra signalling. Quota enforcing entity – AMFs Roaming support – no information |
| Solu#9 | Monitoring multiple quotas of number of UEs/PDU Sessions per S-NSSAI at NWDAF | Here are the solution principles: This solution addresses several Key Issues. As per KI#2, it is very similar to sol#5. Quota holding entity – the quota is held in the NWDAF. NOTE: Similar to sol#5, this solution needs further clarification on how the quota is maintained with the new eNA NWDAF architecture - – i.e. dependency on eNA NWDAF decision in Rel-17. Quota managing entity – the quota is managed by the NWDAF. NOTE: This solution needs further clarification on how the quota is managed with the eNA NWDAF architecture in Rel-17. Quota enforcing entity – AMFs which interacts with the NWDAF. This also has an impact from the new eNA NWDAF architecture in Rel-17. Option for different charging for use when quota is exceeded. Roaming – supported. In roaming the quota is managed by the hNWDAF. This could also be impacted by the new eNA NWDAF architecture update in Rel-17. |
| Sol#10 | Max number of PDU Sessions per Network Slice control via NSQ function | Here are the solution principles: Quota holding entity –the quota is held in a new NSQ function (may or may not be implemented as a standalone NF) with PLMN granularity. The quota of PDU Sessions per network slice' is statically stored in the vNSQ, Quota managing entity – the quota is managed by the NSQ function with PLMN granularity. A single NSQ function responsible for the quota management. Quota enforcing entity – SMFs with instructions from the NSQ function. Roaming – supported by the vNSQ in the roaming vPLMN. A new NSQ function – same NSQ function can be used for all types of quota management (i.e. KI#1 to KI#5). If NSQ is a standalone NF, further clarifications are needed on the new interface and signalling. |
| Sol#11 | Handling maximum number of sessions using NF status | Here are the solution principles: Quota holding entity – the quota is held in NRF which is slice specific. However it is not defined how the global (SLA) quota is provisioned there. Quota managing entity – the quota is managed by the NRF per network slice. NRF collects PDU Session est/rel information from the SMFs and instructs the AMFs for enforcement. As the NRF is per network slice, additional signalling are expected between NRF and SMFs/AMFs . Same additional signalling is also expected to the roaming communication between vNRF and hNRF Quota enforcing entity – AMF based on instructions from the NRF. Roaming – enforced by the hNRF via the vNRF. A new NRF function per network slice. |
| Sol#18 | Proactive Slice Quota Management in AMF | Here are the solution principles: This solution is a combination between sol#10 (new function entity SQM in sol#18 for NSQ in sol#10) and sol#8 (local quota held and managed by the AMFs). Quota holding entity – the global quota managed by SQM and the local quota is managed by AMFs. . Unlike to the reactive approach in sol#10 of the quota management, this proactive approach does not require signalling transaction for every PDU session establishment, local quotas are redistributed on needed basis, and therefore it has least amount of additional signalling and less impacts on the existing system. |
| Sol#19 | Support of network slice quota control and enforcement | Here are the solution principles: Quota holding entity – the global quota is held in the new function QCF however it is not clear where the global quota comes from. The local quota is held in the new functions QEF which is allocated by QCF based on analytics from the NWDAF. Quota managing entity – the quota is managed by new local functions QEF. However, managing the local quotas between the QEFs (e.g. redistribution when needed) was not described. Quota enforcing entity – by QEF and QCF with a help from AMF. However, the quota enforcement is not clear as the solution mentions about master AMF and it does not explain what this master AMF is. Roaming – no information. |
| Sol#32 | Operator quota control policy on the number of PDU session | Here are the solution principles: Quota holding entity – It is not clear where the global quota is held. The solution says that SMF and NF may be configured with quota control for specific PDU Sessions but that is not the global quota. And it is not clear which entity is behind NF either. Quota managing entity – the quota looks to be managed by the NF. Not enough details again. Quota enforcing entity – maybe again the NF. But not much information about the NF so, not clear how this all works. Roaming – no information. |
| Sol#35 | Quota enforcement considering service type | This is not complete solution. This is about the quota managing entity NF which determines the quota status and the service type, i.e. prioritisation in quota management. |
| Sol#36 | Handover of a PDU session | This is not a standalone solution. It is about how to count the number of PDU sessions when switching an existing PDU session between 3GPP access and non-3GPP access. It claims it could be addition to other solutions. |
| Sol#38 | Network slice quota control and enforcement provided by CHF based solution | Quota holding entity – the quota is held in the CHF and provided by the OAM at network slice instance creation. Quota managing entity – the quota is managed by CHF Quota enforcing entity – by CHF via SMF Roaming – no information how it is supported. NOTE: Need to consult with SA5 regarding the impacts on OAM and CHF. |

/**************** end of 1st change****************/

/**************** start 2nd change****************/

/**************** end of 2nd change****************/