SA WG2 Meeting #S2-143E	S2-2100009

24 February - 09 March, 2021, Electronic meeting

SA WG2 Meeting #S2-142E	S2-2008502

16 - 20 November, 2020, Electronic meeting

3GPP TSG RAN WG2 Meeting #112-e	R2-2011164

Electronic, November 2 – 13, 2020

Title:	LS on UTRAN UE capabilities from CN to gNB

Response to:

Release:	Release 16

Work Item:	SRVCC_NR_to_UMTS-Core and RACS-RAN-Core

Source:	RAN2

To:	SA2

Cc:	CT3

Contact person:	Xun TANG

tangxun (at) huawei (dot) com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

UTRA capabilities include information that changes every time the UE connects to UTRA, some of which concern START values.

To avoid START values de-synchronization and the key replay, RAN2 agreed that

  - The gNB does not upload the UE UTRA-FDD capabilities to the AMF;

  - Prior to initiating mobility the gNB always retrieves the latest capabilities from the UE. I.e. would not use UTRA capabilities received from CN, if provided.

RAN2 would like to know if

  - For the manufacturer assigned case, the UE capability ID is intended to cover UTRA capabilities; also,

  - CN may provide UTRA capabilities to gNB.

# 2	Actions

To 3GPP SA2

ACTION: SA2 is kindly requested to take RAN2’s consideration into account.

# 3	Dates of next TSG RAN WG2 meetings

TSG RAN WG2 Meeting #113-e	25 January – 5 February 2021, Electronic