3GPP TSG-WG SA2 Meeting #143E e-meeting 	S2-2102925

12-16 April 2021, Elbonia

# Title:	[DRAFT] Reply LS on N3IWF FQDN for emergency service

# Reply to:	LS on N3IWF FQDN for emergency service (S2-2102109/C4-211521)

# Release:	Rel-17

# Work Item:	TEI17

Source:	SA2

To:	CT4, CT1

Cc:	-

Contact Person:

Name:	Sebastian Speicher

Tel. Number:

E-mail Address:	speicher@qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	S2-2102924 (TS 23.501 CR#2848)

# 1.	Overall Description:

SA2 would like to thank CT4 for the LS on N3IWF FQDN for emergency service. SA2 agreed the attached CR to clarify that separate FQDNs are expected to be used for ePDG and N3IWF selection. In addition SA2 would like to provide the following feedback to CT4's questions:

CT4 question 1): Which one does the highlighted "Operator Identifier Emergency FQDN" and "Visited Country Emergency FQDN" refer to?

a) emergency ePDG FQDN

b) either emergency ePDG FQDN or emergency N3IWF FQDN

SA2 reply: The Operator Identifier Emergency FQDN and Visited Country Emergency FQDN refer to separate FQDNs for ePDG and N3IWF selection, respectively. SA2 clarified this in the attached CR.

Question 2): If the answer to Q1) is "b", why does emergency N3IWF FQDN need to be defined?

SA2 reply: Re-using ePDG FQDNs for N3IWF selection has the disadvantage that the UE would not be able to determine based on DNS whether a given PLMN supports emergency services via ePDG only or N3IWF only or via both.

This implies that the UE would need to try to select and connect to the node identified by the FQDN in order to determine its node type (i.e. whether the node is an N3IWF or ePDG). This is particulary inefficient for UEs that only support connectivity with one of the two node types (i.e. N3IWF or ePDG only): UEs that only support connectivity e.g. to N3IWF may need to try to unnecessarily connect to many ePDGs before eventually selecting a PLMN that supports emergency services via N3IWF (and vice versa).

Therefore SA2 decided to clarify that the UE uses separate FQDNs for N3IWF selection for emergency services.

# 2.	Actions:

To CT4 and CT1

ACTION:

SA WG2 kindly requests CT4 and CT1 to take the above into account.

# 3.	Date of Next TSG SA WG2 Meetings:

3GPPSA2#145-e	17-28 May 2021	Electronic Meeting

3GPPSA2#146-e	TBD	Electronic Meeting