SA WG2 Meeting #144e	S2-2102162r01

# April 12 – 16, 2021; Elbonia             		 	(revision of S2-210)

Source: 	Nokia, Nokia Shanghai Bell

Title: 	 Issue that we cannot guarantee that the UE uses the EASDF's IP address for its subsequent DNS Query

Document for: 	Agreement (P-CR)

Agenda Item: 	8.3

Work Item / Release:	eEDGE_5GC / Rel-17

Abstract of the contribution: Issue that we cannot guarantee that the UE uses the EASDF's IP address for its subsequent DNS Query

# 1 Discussion

This PCR is part of multiple PCR(s) that address different aspects / items of 6.2.3.2.2 “EAS Discovery Procedure with EASDF”:

- General corrections

- EASDF registration to NRF

- The EN “How to guarantee that the UE uses the EASDF's IP address for the subsequent DNS Query in step 8 is FFS”

The intent is to allow discussion on orthogonal topics in different threads (avoiding one thread with 10 different topics) and to at the end merge the different PCR(s) if found useful.

This PCR addresses item 3.

# 2 Proposal

It is proposed to update TS 23.548 as follows

>>>>BEGINNING OF CHANGES<<<<

## 6.2	EAS Discovery and Re-discovery

### 6.2.1	General

Editor's note:	This clause describes general parts including e.g. privacy considerations, which DNS properties that are enabling DNS based Edge AS Discovery, recommendations/limitations for cases that OS/user overrides DNS setting.

Editor's note: it is FFS how to direct the UE DNS query for EAS (re-)discovery towards the MNO's indicated DNS server/resolver/EASDF when application/user interface/HLOS override the DNS settings provided by the MNO to the UE.

In order for a UE to use the enhancements for EAS discovery and re-discovery it is essential that the DNS Query originated by the UE is sent to the DNS server/resolver or EASDF indicated by the HPLMN.

The solutions in this specification (e.g., EAS (re-)discovery) are applicable if the UE or the UE application client or both are aware of the usage of Edge Computing.

Based on the type of deployment used by the operator, different solutions apply:

1.	Edge Computing service is provided over dedicated PDU sessions. This approach requires the use of Edge-specific DNN(s)/S-NSSAI(s). In this case the HPLMN indicates to the UE that the used PDU session is Edge-specific. All the DNS queries sent over such DNN/S-NSSAI are directed to the proper DNS server/resolver or EASDF.

2.	Edge Computing service is provided over generic PDU sessions shared with other services (Edge as an internet service). This approach does not require the use of Edge-specific DNN(s)/S-NSSAI(s). In this case it is assumed that applications agreed with the HPLMN that the DNS queries are sent to the HPLMN Indicated DNS server/resolver or EASDF and that such indication cannot be overwritten by the user (via, e.g., user interface of the UE).

When indicating to the UE the address of the DNS server/resolver or of the EASDF, the HPLMN also indicates that such address is associated to Edge supporting features. Once received the UE shall forward such indication to the application layer and HLOS.

Editor's Note: How to enforce that the UE application and HLOS use the HPLMN's indicated DNS server/resolver of EASDF in case of DNS over HTTPS (DoH) or DNS over TLS (DoT) is FFS.

In case of unencrypted DNS requests (Do53), the following can be used to enforce that the DNS Query is sent to the proper DNS server/resolver or EASDF:

- 	If supported by the UE and if instructed by the HPLMN, the UE uses the UE based DNS query redirection (see clause 6.2.1A),

-	alternatively, if the UE does not support the UE based DNS Query redirection or the HPLMN does not instruct to use it, the PLMN uses the UPF based DNS query redirection (see Option D of clause 6.2.3.2.3).

### 6.2.1A	UE based DNS query redirection

In order for an operator to support EAS (re-)discovery, the UE originated DNS queries for Edge applications need to be sent to the DNS server/resolver or EASDF indicated by the HPLMN. To do that, the HPLMN instructs the UE to intercept the DNS queries/replies to/from the UE application/HLOS and the application/HLOS DNS server/resolver and redirect them in the following way:

-	On the UL, the UE modifies the destination IP address of the IP frame with the address of the DNS server/resolver or EASDF indicated by the HPLMN in the ePCO (see TS 23.501 [x] cl. 5.6.10.1). In DL, the UE modifies the source IP address to be the application/HLOS DNS server address.

-	In order to select which DNS queries are to be re-directed, if the operator uses Edge-specific DNN(s)/S-NSSAI(s), then for each PDU session established via such specific DNN(s)/S-NSSAI(s) and based on an additional indication in the ePCO the HPLMN instructs the UE that all DNS Queries exchanged inside a given PDU session shall be redirected to the MNO indicate DNS server/resolver or EASDF. If the operator does not use Edge-specific DNN(s)/S-NSSAI(s), then the HPLMN instructs the UE, based on SLAs with different application providers, to filter each intercepted DNS query based on the application identifier and the configuration in URSP.

NOTE: the steps above are not applicable to DNS over TLS (DoT) or DNS over HTTPS (DoH).

>>>>NEXT CHANGE<<<<

##### 6.2.3.2.2	EAS Discovery Procedure with EASDF

>>>>SKIP UNCHANGED TEXT<<<<

5.	The SMF includes the IP address of the EASDF as DNS server in PDU Session Establishment Accept message as in step 11 of clause 4.3.2.2.1 of TS 23.502 [3]. The UE configures the EASDF as DNS server for that PDU Session.

The UE module requires the UE OS to set the OS DNS configuration in the UE as unchangeable. The OS DNS configuration remains unchanged or not visible to the user or application until 5GC sends another DNS configuration via ePCO to UE..

Editor's note: It is FFS how the 5GC/UE module indicate to the OS to make the OS DNS configuration unchangeable.

If the UE module detects that the DNS configuration in the UE OS is changed, the UE module may block the DNS query sent by the UE OS or notify the 5GC If notified, the 5GC may then resend or change the DNS configuration back to original DNS configuration in UE OS.

Editor's note: It is FFS if, upon notification from the UE, further actions are required by the 5GC (e.g., blocking the DNS query).

6.	The SMF may invoke Neasdf_DNSContext_Update Request (PDU Session Context ID, rules to handle DNS queries from the UE) to EASDF. The update may be triggered by UE mobility, e.g. when UE moves to a new location, or by a reporting by EASDF of a DNS Query with certain FQDN, or, the update may be triggered by insertion/removal of Local PSA, e.g. to update rules to handle DNS messages from the UE or by new PCC rule information.

Editor's note:	It is FFS whether and how the AF-triggered DNS server information change trigger DNS context update procedure.

7.	The EASDF responds with Neasdf_DNSContext_Update Response.

8.	The UE sends DNS Query message to the EASDF.

The SMF may, based on local policies, have instructed a UPF to detect (UL) DNS traffic from the UE that is not targeting the EASDF's IP address provided to the UE by the SMF in step 5;

This detection may correspond to a PDR referring to the ports used by DNS protocols (53, 853) and to the EASDF's IP address provided by the SMF to the UE over NAS.

The SMF may, based on local policies, further request the UPF to forward such DNS traffic to the SMF in which case the SMF takes care itself of forwarding further the DNS traffic once it has applied corrective actions ; Or alternatively the UPF may be instructed by the SMF to buffer such DNS traffic and to report to the SMF; in this alternative case, once corrective actions have been carried out by the SMF, the SMF may ask the UPF to forward the buffered DNS traffic.

The corrective actions carried out by the SMF may be to insert offload capabilities (UL CL and local PSA) based on UE’s location for the PDU Session regardless of the detection (via DNS requests) of the UE willingness to reach an EAS.

Editor's note: Details of corrective actions undertaken by the SMF are FFS.

The local policies mentioned above may be based on the (DNN, S-NSSAI).

NOTE: the steps above are not applicable to DNS over TLS (DoT) or DNS over HTTPS (DoH).

The rest of the procedure assumes the UE has indeed used the EASDF as its network DNS resolver.

>>>>END OF CHANGES<<<<