3GPP TSG-SA WG2 Meeting #144E e-meeting	S2-2102209r01

# Elbonia, 12-16 April, 2021									     			 (was S2-21XXXXX)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 2612 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | TS23.502 KI#1 Network Slice Admission Control Function (NSACF) – Result Update |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, NEC, Apple, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNS_Ph2 |  |  |  |  |  | Date: |  |  | 2021-01-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This CR enables the NSACF to optionally return to the AMF the EAC activation/De-activation flag in the Registration Accept/Reject message. This avoids the need for the SUBSCRIBE/NOTIFY framework for this puprose and provides equivalent functionality. A registration Accept/Reject message can include any list of S-NSSAIs subject to NSAC and as such supports a fully dynamic support matching the SUBSCRIBE/NOTIFY framework with less impact. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Enable the NSCAF to return to the AMF the EAC activation/De-activation flag in the Registration Accept/Reject message. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Support the SUBSCRIBE/NOTIFY framework to provide same functionality at much higher implemntation cost. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.11.2, 4.2.11.3, 5.2.21.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR… CR… |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * start of 1st change * * * *

4.2.11	Network Slice Admission Control Function (NSACF) procedures

4.2.11.1	General

The Network Slice Admission Control Function procedures are performed for an S-NSSAI which is subject to Network Slice Admission Control (NSAC) as described in TS 23.501 [2].

Editor's note:	It is FFS how the NSAC works in the roaming case.

4.2.11.2	Number of UEs per network slice availability check and update procedure

The number of UEs per network slice availability check and update procedure is to update (i.e. increase or decrease) the number of UEs registered with a S-NSSAI which is subject to NSAC. The AMF is configured with the information indicating which network slice is subject to NSAC.

Figure 4.2.11.2-1: Number of UEs per network slice availability check and update procedure

1.	The AMF triggers the Number of UEs per network slice availability check and update procedure to update the number of UEs registered with a network slice when a network slice subject to NSAC is included or removed from the Allowed NSSAI for a UE. The procedure is triggered in the following cases:

-	At UE Registration procedure, as per clause 4.2.2.2.2:

-	before the Registration Accept in step 21 if the EAC mode is active; or

-	after the Registration Accept message if the EAC mode is not active;

-	At UE Deregistration procedure, as per clause 4.2.2.3, after the Deregistration procedure is completed;

-	At UE Configuration Update procedure (which may result from NSSAA procedure):

-	before the UE Configuration Update message if the EAC mode is active; or

-	after the UE Configuration Update message if the EAC mode is not active;

NOTE:	Depending on the deployment, there may be different NSACFs for different S-NSSAIs that are subject to NSAC. In this case, the AMF during the registration, triggers the Number of UEs per network slice availability check and update procedure for each NSACF.

Editor's note:	If one S-NSSAI the number of the UE/PDU session is managed by several different NSACF, it is FFS how to assure the AMF reaches the same NSACF and how to coordinate the number of UE among the NSACF(s)

2.	If the S-NSSAI is verified that it can be included in the Allowed NSSAI, and the S-NSSAI is subject to NSAC, the AMF sends Nnsacf_NumberOfUEsPerSliceAvailabilityCheckAndUpdate_Request message to the NSACF. The AMF includes in the message the UE ID, the S-NSSAI(s) for which the number of UEs registered per network slice update is required and the update flag which indicates whether the number of UEs registered with the S-NSSAI(s) is to be increased when the UE has gained registration to network slice(s) subject to NSAC or the number of UEs registered with the S-NSSAI(s) is to be decreased when the UE has deregistered from S-NSSAI(s) or could not renew its registration to an S-NSSAI subject to NSAC.

3.	The NSACF updates the current number of UEs registered for the S-NSSAI, i.e. increases or decrease the number of UEs registered per S-NSSAI based on the information provided by the AMF in the update flag parameter.

If the update flag parameter from the AMF indicates increase and the NSACF finds the UE ID is already in the list of UEs registered with the network slice, the number of the UEs registered with the network slice is not increased as the UE has already been counted as registered with the network slice. If the UE ID is not in the list of UE IDs registered with the network slice and the maximum number of UEs registered with the network slice has not been reached yet, the NSACF adds the UE ID in the list of UEs registered with the network slice and increases the current number of the UEs registered with the network slice. If the UE ID is not in the list of UEs registered with that S-NSSAI and the maximum number of UEs per network slice for that S-NSSAI has already been reached, then the NSACF returns a result parameter indicating that the maximum number of UEs registered with the network slice has been reached.

If the update flag parameter from the AMF indicates decrease, the NSACF removes the UE ID from the list of UEs registered with the network slice for each of the S-NSSAI(s) indicated in the request from the AMF and also the NSACF decreases the number of UEs per network slice that is maintained by the NSACF for each of these network slices.

4.	The NSACF returns the Nnsacf_NumberOfUEsPerSliceAvailabilityCheckAndUpdate_Response in which the NSACF includes the S-NSSAI(s) for which the maximum number of UEs per network slice has already been reached along with a result parameter indicating that the maximum number of UEs registered with the network slice has been reached.

If for all the S-NSSAI(s) the NSACF returned the maximum number of UEs per network slice has been reached, the AMF rejects the UE request for registration. In the Registration Reject message the AMF includes the rejected S-NSSAI(s) in the rejected NSSAI parameter, a reject cause set to 'maximum number of UEs per network slice reached' for each S-NSSAI and optionally a back-off timer.

Editor's note:	It is FFS whether the AMF shall send the Registration Reject message when for all the S-NSSAI(s) that the UE requested to register for and were subject to NSAC the NSACF returned the maximum number of UEs per network slice has been reached.

Otherwise, the AMF returns Registration Accept message in which the AMF includes the rejected S-NSSAI(s) in the rejected NSSAI list for which the NSACF has indicated that the maximum number of UEs per network slice has been reached, a reject cause set to 'maximum number of UEs per network slice reached' for each rejected S-NSSAI and optionally a back-off timer.

The Nnsacf_NumberOfUEsPerSliceAvailabilityCheckAndUpdate_Response optionally includes a list of S-NSSAIs subject to NSAC and for each S-NSSAI, whether EAC flag is activated/de-activated. For EAC handling see clause 4.2.11.3.

NOTE: The list of S-NSSAI may include any S-NSSAI supported by the AMF.

Editor's note:	It is FFS whether and how to restrict the signalling sent from the AMFs to the NSACF in case the maximum number of UEs has been reached for prolonged time.

4.2.11.3	Configuration for Early Admission Control (EAC) update procedure

EAC enables the NSACF to indicate to the AMF whether early admission control can be deactivated for an S-NSSAI subject to the NSAC. This is achieved through a flag which can be set to activated or deactivated for the S-NSSAI subject to NSAC. The AMF uses the EAC flag to decide when to trigger the number of UEs per network slice availability check and update procedure so that delays to the registration procedure may be avoided when the number of UEs registered to an S-NSSAI is relatively low compared to the maximum limit. The EAC flag activation/deactivation is set per S-NSSAI and based on operator configured threshold for the S-NSSAI.

NSACF

AMF

2. Nnsacf_NumberOfUEsPerSliceEACNotify (EAC flag=activated/deactivated)

3. Based on the EAC flag the AMF decides when to trigger the update calls to the NSACF

1. The number of UEs registered with a network slice subject to NSAC reaches a certain threshold

NSACF

AMF

2. Nnsacf_NumberOfUEsPerSliceEACNotify (EAC flag=activated/deactivated)

3. Based on the EAC flag the AMF decides when to trigger the update calls to the NSACF

1. The number of UEs registered with a network slice subject to NSAC reaches a certain threshold

If the EAC flag indicates EAC mode activated, the AMF triggers the number of UEs per network slice availability check and update procedure before the Registration Accept step of the registration procedure or before the UE Configuration Update message. In EAC activated mode the AMF will take the response from NSACF into account when rejecting or accepting the UE registration.

If the EAC flag indicates EAC mode deactivated, the AMF triggers the number of UEs per network slice availability check and update procedure after Registration Accept step of the registration procedure or after the UE Configuration Update message. In EAC deactivated mode the AMF will reject or accept the UE registration without taking the NSACF response into account.

* * * * end of 1st change * * * *

* * * * start of 2nd change * * * *

5.2.21	Network Slice Admission Control Function (NSACF) services

5.2.21.1	General

The following table illustrates the NSACF services.

Table 5.2.21.1-1: List of NSACF services

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnsacf_NumberOfUEsPerSlice | AvailabilityCheckAndUpdate | Request/Response | AMF |

5.2.21.2	Nnsacf_NumberOfUEsPerSlice services

5.2.21.2.1	General

Service Description: The Nnsacf_NumberOfUEsPerSlice services control the number of UEs registered with a network slice for the network slices subject to NSAC. The AMF can request the NSACF to check whether the number of UEs registered with a network slice has reached the maximum number of UEs per network slice and the AMF can also request the NSACF to update the number of UEs registered with a network slice.

5.2.21.2.2	Nnsacf_NumberOfUEsPerSliceAvailabilityCheckAndUpdate service operation

Service Operation name: Nnsacf_NumberOfUEsPerSliceAvailabilityCheckAndUpdate

Description: Updates the number of UEs registered with a network slice (e.g. increase or decrease) when the UE registration status for a network slice subject to NSAC has changed. Also, if the number of the UEs registered with the network slice is to be increased and the Early Availability Check (EAC) mode in the NSACF is activated for that network slice (see Nnsacf_NumberOfUEsPerSliceEACNotify service operation), the NSACF first checks whether the number of UEs registered with the network slice has reached the maximum number of UEs per network slice threshold. If the maximum number of UEs registered with the network slice has already been reached, the UE registration for that network slice is rejected. If the EAC is not activated, the NSACF increases or decreases the number of UEs per network slice as per the input parameters below.

Inputs, Required: S-NSSAI(s), S-NSSAIs supported by the AMF, UE ID (SUPI), update flag.

Inputs, Optional: None.

The S-NSSAI(s) parameter is a list of one or more network slices for which the number of UEs registered with a network slice is to be updated and checked if the maximum number of UEs per network slice threshold has already been reached.

S-NSSAIs supported by the AMF is the list of the S-NSSAIs supported by the AMF.

The UE ID is used by the NSACF to maintain a list of UE IDs registered with the network slice.

The update flag input parameter indicates whether the number of UEs registered with a network slice is to be:

-	increased when the UE registers to a new network slice subject to NSAC. If the UE ID is already in the list of UEs registered with the network slice, the number of UEs registered with the network slice is not increased as the UE has already been counted as registered with the network slice. If the UE ID is not in the list of UE IDs registered with the network slice and the maximum number of UEs registered with the network slice has not been reached yet, the NSACF adds the UE ID in the list of UEs registered with the network slice and increases the number of the UEs registered with the network slice. If the UE_ID is not in the list of UEs registered with that S-NSSAI and the maximum number of UEs per network slice for that S-NSSAI has already been reached, then the NSACF returns maximum number of UEs per network slice reached result;

-	decreased when the UE deregisters for a network slice that is subject to NSAC. The NSACF decreases the number of the UEs registered with the network slice and removes the UE ID from the list of UEs registered with the network slice.

Editor's note:	It is FFS how to support in case multi NSACF is supported, e.g. discover the same NSACF, coordination of the local maximum number among NSACF.

Outputs, Required: maximum number of UEs per network slice reached, availability status.

Outputs, Optional : List of S-NSSAIs supported by the AMF and subject to NSAC including the EAC activation/deactivation status flag per S-NSSAI.

The availability status indicates the current status of the network slice availability (e.g. a percentage out of the max number of UEs registered with a network slice).

Optionally the NSACF may return a list of S-NSSAIs subject to NSAC and for each S-NSSAI, whether EAC flag is activated/de-activated.

Editor's note:	It is FFS how to handle at the same AMF different UE get the different activation flag.

* * * * end of 2nd change * * * *