3GPP TSG-WG SA2 Meeting #144E e-meeting 	S2-2102387

Elbonia, April 12 – 16, 2021	(revision of S2-210xxxx)

Source:	Huawei, HiSilicon, Nokia, Nokia Shanghai-Bell, CATT

Title:	TS 23.247: MBS Session Join and Establishment

Document for:	Approval

Agenda Item:	8.9

Work Item / Release:	5MBS / Rel-17

Abstract: This contribution updates the main procedure for MBS Session Join and Establishment for MBS.

1. Introduction/Discussion

In SA2 #143E meeting, the basic call flow of clause 7.2.1 was agreed. The leftover issue is mainly about how to establish the DL transmission path between MB-UPF and RAN node, specifically, the following issue need to be considered:

-	How RAN 5MBS capability is made know by 5GC, so as to determine the delivery method.

-	How to exchange signaling between RAN node and 5GC to establish the DL N3 tunnelling.

This document tries to resolve the open issues mentioned above and specify the details.

2. Text Proposal

It is proposed to capture the following changes vs. TS 23.247.

# * * * * First change * * * *

## 7.2	MBS procedures for multicast Session

### 7.2.1	MBS join and Session establishment procedure

#### 7.2.1.1	General

Session Join procedure is used by UEs to inform the 5GC of the UE interest in an MBS Session. The user plane management is described in clause 6.6.

#### 7.2.1.2	Establishment of a PDU Session that can be associated with multicast session(s)

A PDU Session associated with multicast session(s) is established using the procedures as specified in TS 23.502 [6] clause 4.3.2.2 with the following differences:

NOTE:	The DNN and S-NSSAI are used to establish the PDU session which can carry the Multicast operation, i.e. join/leave, and can be associated with multicast MBS session(s).

-	In step 2, based on the indication of establishing a PDU Session associated with multicast session(s), and optionally an indication from subscription data that the user is allowed to join multicast sessions in the UDM, the AMF selects an SMF capable of handling multicast sessions based on DNN and S-NSSAI, or locally configured data or a corresponding SMF capability stored in the NRF. For indirect discovery, the AMF requests the SCP to select an SMF capable of handling multicast sessions.

#### 7.2.1.3	MBS join and Session establishment procedure

Editor's note:	Whether AMF is informed of UE join, and whether AMF should be involved in the MBS Session handling are FFS.

The following steps are executed before the UE requests to join the MBS session:

- The MBS Session has been configured. Details see 7.1.1. Session configuration for multicast is optional.

- The UE registers in the PLMN and establishes a PDU session.

- The UE has known at least the MBS Session ID of a multicast group that the UE can join, e.g. via announcement.

UE

NG-RAN

AMF

SMF

UPF

NRF

MB-SMF

MB-UPF

AF

1a. UL NAS message (N1 SM container (PDU Session Modification Request))

1b. Nsmf_PDUSession_UpdateSMContext request

5. Nsmf_PDUSession_UpdateSMContext response

3. Nnrf_NFDiscovery request/response

4. Nsmf_MBSSession_Create request/response

6. N2 message request

13. Multicast data

14. Multicast data

15. Bear selection

16. Multicast data via PTP or PTM

Transmission via 5GC shared MBS traffic delivery

17. Multicast data

18. Multicast data via PDU Session

19. Multicast data via PDU Session

Transmission via 5GC Individual MBS traffic delivery

2. Authorization check, see clause 6.1.1

Establishment of resources for 5GC shared MBS traffic delivery

7a. N2 message

7b. Nsmf_MBSSession_Create request

7c. Configuration for Shared delivery

7d. Nsmf_MBSSession_Create response

7e. N2 message

9. N2 message response

10. Nsmf_PDUSession_UpdateSMContext request

Establishment of resources for 5GC Individual MBS traffic delivery

11a. Configuration for Individual delivery

11b. Nsmf_MBSSession_Update request

11c. Configuration for Individual delivery

11d. Nsmf_MBSSession_Update response

11e. Configuration for Individual delivery

8. RRC message (PDU Session Modification command)

12. Nsmf_PDUSession_UpdateSMContext response

Figure 7.2.1.3-1: PDU Session modification for UE joining multicast session

1.	To join the multicast group, the UE sends a PDU Session Modification Request which contains an MBS Session ID). The MBS Session ID indicates the multicast group that UE wants to join.

Editor's note:	Whether AMF is informed of UE join is FFS.

2.	Based on the received MBS Session ID, the SMF determines this is MBS Session join request. The SMF authorizes MBS Session join request, see clause 6.1.1. If the Multicast MBS session is configured but the configuration indicates that the data transmission has not started yet, the SMF may reject the MBS session join with appropriate cause value. If authorization check fails, the SMF indicates cause value in the PDU Session Modification Reject sent to the UE and goes to step 5.

Editor's note:	It is ffs if database interactions are required for the authorization check.

3.	If SMF has no information about the MBS Session context for the indicated MBS Session ID, SMF checks at the NRF whether a MBS Session context for the indicated MBS Session ID exists in the system, by using Nnrf_NFDiscovery_Request request (MBS Session ID). If a MBS Session context already exists in the system, the NRF responses with Nnrf_NFDiscovery_Requestresponse (MB-SMF ID).

Editor's note:	Whether it is the SMF or the AMF that should discover MB-SMF is FFS.

Editor's note:	Whether SMF acting as the MB-SMF is needed and how it works if needed is FFS.

Editor's note:	More consideration on how to prevent denial of service attack type situation when first UE joining the multicast group, triggers the MB-UPF to join the multicast tree towards the content provider is FFS.

4.	By using Nsmf_MBSSession_Create request (MBS Session ID), SMF interacts with MB SMF to retrieve multicast QoS flow information of the indicated MBS session.

Editor's note:	it is FFS whether MB-SMF may also send transport information related to the MBS Session to the SMF, e.g. common tunnel ID and/or low layer source specific multicast address.

Editor's note:	Whether AMF should be involved in the MBS Session handling is FFS.

5.	SMF responds to AMF through Nsmf_PDUSession_UpdateSMContext response(N2 SM information (PDU Session ID, MBS Session ID, MB-SMF ID, [multicast QoS flow information], updated PDU Session information, [mapping information between unicast QoS flow(s) and multicast QoS flow(s))], N1 SM container (PDU Session Modification Command).

The multicast QoS flow information and the mapping information between unicast QoS flow(s) and multicast QoS flow(s) are included in N2 SM information if the multicast session is active, in order to:

-	create an MBS session context for the indicated MBS session in the RAN, if it does not exist already; and

-	inform about the relation including the mapping information between the multicast context and the UE's PDU session context to RAN.

Based on operator policy, the SMF may prepare for 5GC individual MBS traffic delivery fall-back. The SMF maps the received QoS information of the multicast QoS Flow into PDU Session's QoS Flow information, and includes the information of the QoS Flows and the mapping information about the QoS Flows in the SM information sent to RAN.

Editor's note:	Details information included in N2 SM information will be aligned with RAN WG3.

Editor's note:	Whether it needs to inform about the relation including the mapping information between the multicast context and the UE's PDU session to UE is FFS.

6.	The N2 message, which includes the multicast session information and PDU session modification information is sent to the RAN.

Editor's note:	Whether it is the AMF or the SMF that triggers the MBS Session resource setup in NG-RAN is FFS.

If the MBS is not supported by NG-RAN, 5GC individual MBS traffic delivery may be used. Otherwise, 5GC shared MBS traffic delivery is adopted.

The NG-RAN uses the MBS Session ID to determine that the PDU Session Modification procedures corresponds to the indicated multicast session.

If the multicast QoS information is received and the NG-RAN supports MBS, the associated unicast QoS flow information is not used to allocate the radio resource.

NOTE x2:	It is NG-RAN that decides whether radio resource is allocated or not.

When the NG-RAN receives an MBS Session ID but MBS Session context does not exist for that MBS Session ID, the NG-RAN use the included MBS Session QoS information to allocate resources to serve this multicast session. Otherwise the indicated MBS Session has been established before. The NG-RAN can use those allocated resource for MBS Session data packet transferring to UE.

[Conditional] If unicast transport is applied for the MB-N3 tunnel, step 7 is used for 5GC shared MBS traffic delivery and shared tunnel has not been established for the indicated MBS session:

Editor's note:	The name of MB-N3 is FFS.

Editor's note: This part needs to align with the outcome of S2-2102941 if any.

7a.	NG-RAN signals a message towards AMF, and the information for establishing the tunnel for DL MBS transmission. MBS Session ID are included.

If the NG-RAN node uses a unicast transport for shared delivery, it allocates a downlink tunnel ID for the reception of MBS data and includes the downlink tunnel information in the request.

Editor's note:	How NG-RAN interacts with MB-SMF is FFS.

7b.	AMF invokes Nsmf_MBSSession_Create request (MBS Session ID, RAN id) towards the indicated MB-SMF.

Editor's note:	Whether AMF needs provides RAN id to MB-SMF, or it stores the information for the RAN nodes (e.g., RAN id) instead for the subsequent signaling is FFS.

Editor's note:	Whether AMF directly sends message to MB-SMF, or via SMF is FFS.

7c.	If downlink tunnel ID is included, MB-SMF configures MB-UPF to transmit the MBS data for multicast towards NG-RAN using the downlink tunnel ID.

7d.	MB-SMF responds to AMF to through Nsmf_MBSSession_Create response.

Editor’s notes: How LL IP Multicast Address and QoS Profile are made available in NG-RAN is FFS.

7e.	AMF forwards multicast distribution session response to NG-RAN node.

8. The NG-RAN issues AN specific signalling exchange with the UE that is related with the MBS session resource reconfiguration. As part of the AN specific signalling exchange, the N1 SM container is provided to the UE.

9.	The NG-RAN sends the PDU session modification response.

If the MBS is supported by NG-RAN and shared tunnel is established between the NG-RAN and MB-UPF, the accepted multicast QoS flow information is included in the N2 SM response container. Otherwise, the accepted unicast QoS flow is included in the N2 SM response container.

10.	The AMF invokes Nsmf_PDUSession_UpdateSMContext request to the SMF.

Per the accepted multicast QoS flow information, the SMF determines that the shared tunnel is used for multicast packet transferring.

NOTE x3:	If the shared tunnel is used, the interaction with UPF is not needed for the indicated MBS session

[Conditional] Step 11 is used for 5GC Individual MBS traffic delivery, e.g. the related NG-RAN does not support multicast.

If the shared tunnel between the UPF(PSA) and MB-UPF for individual delivery have not been established, step 11a to 11e are executed.

11a.	If unicast transport for the multicast data between UPF and MB-UPF is to be used, SMF allocates a downlink tunnel endpoint and configures UPF. Or, SMF requests UPF to allocate a downlink tunnel ID.

11b.	SMF invokes Nsmf_MBSSession_Update request (MBS Session ID, DL tunnel info) towards MB-SMF that includes MBS Session ID and downlink tunnel info of UPF, for establishing the multicast session distribution between MB-UPF and UPF.

11c.	MB-SMF configures MB-UPF to transmit the multicast distribution session towards UPF using the received downlink tunnel ID.

11d.	MB-SMF responds to SMF through Nsmf_MBSSession_Update response. For multicast transport between MB-UPF and UPF, it also indicates in the downlink tunnel information the transport multicast address for the multicast session.

11e.	For multicast transport between MB-UPF and UPF, SMF configures UPF to receive the multicast distribution session and forward the data within unicast transport.

12. The SMF invokes Nsmf_PDUSession_UpdateSMContext response to the AMF.

13. MB-UPF receives multicast PDUs, either directly from the content provider or via the MBSTF that can manipulate the data.

Step 14 to 16 are for 5GC shared MBS traffic delivery:

14. MB-UPF sends multicast PDUs in the N3/N9 tunnel associated to the multicast distribution session to the RAN. There is only one tunnel per multicast distribution session and NG-RAN node, i.e., all associated PDU sessions share this tunnel.

15. The NG-RAN selects PTM or PTP radio bearers to deliver the multicast PDUs to UEs that joined the multicast group.

16. The NG-RAN performs the transmission using the selected radio bearer.

Step 17 to 19 are for 5GC individual MBS traffic delivery:

17. MB-UPF sends multicast PDUs in the N3/N9 tunnel associated to the multicast distribution session to UPF. There is only one tunnel per multicast distribution session and destination UPF, i.e., all associated PDU sessions share this tunnel.

18. UPF forwards the multicast data via unicast.

19. The NG-RAN forwards the multicast data via unicast.

NOTE x4:	Details of the DL MBS data transmission could refer to clause 6.7.

#### 7.2.1.4	Multicast session join procedure involving MBSF

Editor's note:	Details are FFS.

# * * * * End of changes * * * *