3GPP TSG-WG SA2 Meeting #144E e-meeting 	S2-2102577r01

Elbonia, 12 - 16 April, 2021

Source:	Huawei, HiSilicon

Title:	Missed parameters within the Service Announcement

Document for:	Agreement

Agenda Item: 	8.9

Work Item / Release:	5MBS /Rel-17

Abstract of the contribution: This contribution proposes to add missed parameter which AF need inform UE in the service announcement.

- Discussion

For 5G MBS, the UE needs to know the DNN and S-NSSAI of the associated PDU Session. The UE join the 5MBS session in the related PDU Session. Also the related PLMN is expected to be included. It is proposed to send the PLMN ID, DNN and S-NSSAI of the associated PDU Session to UE in service announcement.

- Proposal

It is proposed to add the related information in TS 23.247.

**************************** 1st Change ***************************

6.x	Service Announcement

Service Announcement provides the UE with descriptions specifying the multicast or broadcast services to be delivered as part of MBS Session.

The Service Announcement includes the MBS Session ID(s), which is represented by TMGI or a Source Specific IP Multicast Address, for the service. When the MBS Session ID is Source Specific IP Multicast Address, the Service Announcement may include the PLMN ID of the PLMN in which the service is delivered.

The Service Announcement may include an MBS Session Type, which indicates whether the MBS Session for the service is multicast or broadcast.

For local MBS service, the Service Announcement may include the MBS service area information.

If the MBS Session is multicast, the Service Announcement includes the DNN and S-NSSAI of the PDU Session that the UE uses to join the MBS Session for the service via the PLMN.

The Service Announcement maybe provided to UE by AF or MBSF.

Editor’s Note: Other entities that can send Service Announcement to UE is FFS, and whether DNN and S-NSSAI of the PDU Session that the UE uses to join the MBS Session for the service via the PLMN can be included in the Service Announcement is FFS.

Editor’s Note: The details of Service Announcement will be defined with coordination with SA4/SA6, including which information is aware by UE.

********************************* End Change ****************************